3GPP TSG-SA WG6 Meeting #37e	S6-200728 rev1

E-meeting, 14 – 26 May 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	EAS selection

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Jicheol Lee <jicheol.lee at samsung.com>

1. Introduction

Based on the agreement on the cardinality of entities, it is decided as follows:

NOTE:	EAS(s) belonging to the same EAS ID can be provided by multiple ECSP(s) in an EDN.

In addition, since one EEC may communication with one or mote ECS(s), as well as one or more EES(s) concurrently, there should be a mechanim for an EEC to select an EAS where multiple EASs are provided.

We can consider that

- MNO provides ECSP precedence to the EEC and the EEC to select EAS belonging to the ECSP where each ECSP has one EAS per AC.

- MNO provides EAS section criteria to the EEC and the EEC to select EAS where multiple EASs are available

2. Reason for Change

Based on the agreement on the cardinality, multiple EAS for one AC can be possible. However, there is no mechanism for EEC or ECS to select one EAS when multiple EAS for one AC are available.

3. Conclusions

It is proposed to add description for an EAS selection mechanisms based on EAS selection criteria

4. Proposal

It is proposed to add EAS selection criteria in Provisioning Response.

It is proposed to agree the following changes:

* * * First Change * * * *

#### 8.3.3.2	Provisioning response

Table 8.3.3.2-1 describes the information elements for a Provisioning Response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.2.-1: Provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection info | M | DNN (or APN) |
| EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| EDN Service Area (NOTE) | O | Cell list, List of TA, PLMN IDs |
| ECSP info | O | Information for Edge Computing Service Provider |
| EAS selection criteria | O | Information for EEC to select EAS where multiple EASes are available. |
| NOTE: The EDN service area IE is included only if the EDN is a LADN. |  |  |

Editor’s Note: The detailed information of the EAS selection criteria is FFS