3GPP TSG-SA WG6 Meeting #37e	S6-200895

E-meeting, 14 – 26 May 2020	(revision of S6-200729)

Source:	Samsung

Title:	ECS discovery

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Jicheol Lee <jicheol.lee at samsung.com>

1. Introduction

There are multiple scenarios and multiple ways to configure the ECS address and configuration information. This paper discusses the potential scenarios with considerations and lists the potential ways to configure ECS configuration to the ECS.

Potential approaches:

- Using 5GC UE configuration

- Pre-configuration on the EEC

- Automatic derivation of ECS address from the Home PLMN or Serving PLMN

2. Reason for Change

There is no mechanism to configure the ECS for out-of-box scenario

3. Conclusions

It is proposed to describe multiple ECS configuration mechanisms

4. Proposal

Provide a new clause to describe “for ECS discovery” in ‘high level description’

* * * First Change * * * *

## 4.X	Discovery of Edge Configuration Server

The MNO may provide ECS configuration information to the UE via 5GC. If the ECS configuration information is provided by 5GC, the EEC shall use the information for the initial provisioning request. Otherwise, the EEC shall use pre-configured ECS address for the initial provisioning if ECS configuration information is preconfigured with the EEC. For non-roaming scenario, if no configured ECS address is reachable, the EEC shall derive the Home MNO ECS address (for example, http://ecs.mnc123.mcc45.3gppnetweork.org/provisioning for HPLMN with MCC=123, MNC=45) based on the HPLMN’s information and use it for the initial provisioning address.

For roaming scenario, if no configured ECS address reachable for the serving PLMN the EEC shall derive Visited MNO ECS address based on the registered PLMN’s information and use it for the initial provisioning address.

Editor’s Note: It is FFS whether and how 5GC provides ECS configuration information to the UE. It is upto SA2 decision.

Editor’s Note: It is SA3 decition whether the EEC connects the Home ECS before connecting Visited ECS to perform EEC authentication/authorization.

Editor’s Note: It is FFS whether the Home ECS provides Visited ECS information based on the subscription information of Home PLMN.

The ECSP ECS configuration information shall include an ECS address, ECSP provider identifier. If the MNO provides ECSP’s edge computing service via MNO’s network infrastructure, the MNO may include the EDN connection information  in ECSP EES configuration information.

Editor’s Note: It is FFS whether EDN connection information includes MNO’s network slice information based on the SA2 feedback.

* * * End of Change * * * *