3GPP TSG-SA WG6 Meeting #37-e	S6-200749

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics

Title:	Pseudo-CR on EEC-EES subscription requirements

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution expands the requirements of EEC subscription-notification to dynamic information of EASs along with the existing requirement for availability.

2. Reason for Change

Certain other information about EASs apart from its availability may change dynamically – such as its description, endpoint, features, schedule, service area, service KPIs, service continuity support and its status (enabled/disabled etc.). This information may impact the service being provided to the application client by the EEC, such as support for service continuity, or the features being provided by the AC to the user.

Changes in these EAS characteristics may even require actions such as new EAS discovery by the EEC; and therefore should be promptly notified to the EEC.

Notifications of changes in availability of an EAS such as availability of a new EAS or unavailability of a previously available EAS help EEC adjust its services to the AC accordingly, by promptly taking appropriate action. Such notifications also eliminate the need of EEC continuously probing the EES for EASs with certain characteristics such as EAS type or EAS service KPIs.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0

* * * First Change * * * *

### 5.2.7	Subscription service

#### 5.2.7.1	General

#### 5.2.7.2	Requirements

[AR-5.2.7.2-a]	The application layer architecture shall provide mechanisms for an Edge Enabler Client to subscribe to Edge Enabler Server for notifications about changes in dynamic information of Edge Application Servers.

[AR-5.2.7.2-b]	The application layer architecture shall provide mechanisms for an Edge Enabler Server to notify changes in dynamic information of Edge Application Servers subscribed by the Edge Enabler Client.

[AR-5.2.7.2-c]	The application layer architecture shall provide mechanisms for an Edge Enabler Client to subscribe to Edge Enabler Server for notifications about changes in availability of Edge Application Servers.

[AR-5.2.7.2-d]	The application layer architecture shall provide mechanisms to notify an Edge Enabler Client about changes in availability of Edge Application Servers.

[AR-5.2.7.2-e]     The application layer architecture shall provide mechanisms to notify an Edge Enabler Client about changes in Edge Enabler Server’s information and availability status (e.g. EES endpoint change or EES is about to become unavailable due to overload, maintenance window, etc.).

[AR-5.2.7.2-f]	The application architecture shall provide mechanisms to notify an Edge Application Server about relevant changes to elements in the edge application architecture.

Editor's Note: What relevant changes will be supported by subscription service for Edge Application Servers is TBD.