3GPP TSG-SA WG6 Meeting #37-e	S6-200758

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics, AT&T

Title:	Pseudo-CR on UE location reporting API – updates and API definition

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

The contribution updates the UE location reporting API adding the missing aspects, and provides the API definitions.

2. Reason for Change

Text needed updates to align with the TS, while the procedures for an EAS to update a subscription or unsubscribe were not specified. Also the API definitions were pending.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0

* * * First Change * * * *

### 8.6.2	UE location API

#### 8.6.2.1	General

The Edge Enabler Server exposes the UE locationAPI to the Edge Application Server in order to support tracking or checking the valid location of the UE. The UE location API exposed by the Edge Enabler Server relies on the SCEF/NEF northbound API for monitoring event of UE location or LCS (Location Service) as specified in 3GPP TS 23.502 [3], 3GPP TS 23.271 [7], 3GPP TS 36.305 [8], 3GPP TS 23.273 [9] and 3GPP TS 38.305 [10].

The Edge Application Server can request UE location API for one-time reporting to check current UE location and for continuous reporting to track UE's location.

The UE location API supports both request-response for one-time queries (in order to check UE’s current location) as well as subscribe-notify models for providing UE's location to EAS on a continuous basis and enabling the EAS to track UE’s location (as UE location changes).

Editor's note: Whether the Edge Enabler Server exposes the UE location API to an Edge Enabler Client is FFS.

#### 8.6.2.2	Procedure

8.6.2.2.1	Request-response model

Pre-conditions:

1.	The Edge Application Server is authorized to discover and to use UE location API provided by the Edge Enabler Server.

2.	The Edge Enabler Server is authorized to use Nnef Event Exposure API for UE location, based on service level agreement with MNO.

3.	UE Identifier between Edge Application Server and the Edge Enabler Server is authorized for the UE location API (e.g. appropriate access token is received by EAS based on user's consent).

Figure 8.6.2.2.1-1 illustrates the interactions between the Edge Enabler Server and the Edge Application Server for one-time location report.

Figure 8.6.2.2.1-1: UE location API request-response model

1.	The Edge Application Server sends UE location request to the Edge Enabler Server. The UE location request shall include the UE Identifier. It may also include location granularity to indicate requested format of the location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.), which can be understood by the Edge Application Server and location accuracy to indicate the requested accuracy of the location information.

NOTE 1:	The trigger condition of the UE location API is up to application service logic, which is out of scope of this specification.

2.	The Edge Enabler Server checks the UE location:

a.	If the UE locationrequest from the Edge Application Server includes the location granularity, the Edge Enabler Server considers the location granularity parameter for checking the location of the UE.

b.	If the Edge Enabler Server caches locally the location of the UE as the latest, the Edge Enabler Server may use this cached UE location to respond to the Edge Application Server.

c.	The Edge Enabler Server may modify the format of the UE location to fit to the location granularity requested from the Edge Application Server in the step 1, if supported. For example, if the Edge Enabler Server receives the UE location in a format of tracking area ID or cell ID, the Edge Enabler Server can modify the representation of the location information to the GPS Coordinates, or civic addresses (e.g. streets, districts, etc.) as requested by the Edge Application Server.

NOTE 2:	Format modification mechanism of the UE location to fit the location granularity is outside the scope of this specification. Step 2.c. can be performed if the EES is deployed by the PLMN operator.

3.	The Edge Enabler Server responds to the Edge Application Server with the location of the UE, and optionally the timestamp of the location.

##### 8.6.2.2.2	Subscribe-notify model

###### 8.6.2.2.2.1	Subscribe

Figure 8.6.2.2.2.1-1 illustrates the subscribe operation between the Edge Application Server and the Edge Enabler Server for continuous UE location reporting.

Pre-conditions:

1.	The Edge Application Server is authorized to discover and to use UE location API provided by the Edge Enabler Server.

2.	The Edge Enabler Server is authorized to use Nnef Event Exposure API for UE location, based on service level agreement with MNO.

3.	UE Identifier between Edge Application Server and the Edge Enabler Server is authorized for the UE location API (e.g. appropriate access token is received by EAS based on user's consent).

Figure 8.6.2.2.2.1-1: UE location API: Subscribe operation

1.	The Edge Application Server sends UE location subscribe request to the Edge Enabler Server for tracking a UE's location continuously. The UE location subscribe request shall include the UE Identifier. It may also include location granularity to indicate requested format of the location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.), which can be understood by the Edge Application Server and location accuracy to indicate the requested accuracy of the location information.

2.	The Edge Enabler Server checks if the request from the Edge Application Server is authorized. If authorized, the Edge Enabler Server sends success response to the Edge Application Server. If it is not authorized, the Edge Enabler Server sends failure response with rejection cause.

###### 8.6.2.2.2.2	Notify

Figure 8.6.2.2.2.2-2 illustrates the notify operation between the Edge Enabler Server and the Edge Application Server for continuous location.

Pre-conditions:

1.	The Edge Application Server has subscribed to UE location API provided by the Edge Enabler Server.

Figure 8.6.2.2.2.2-1: UE location API: Notify operation

1.	The Edge Enabler Server detects the location of the UE e.g., receiving location of the UE from the 3GPP system. The Edge Enabler Server may cache the detected location information locally with timestamp as the latest location information of the UE. The Edge Enabler Server determines to notify the location information of the UE to the Edge Application Server which has subscribed for UE's location.

2.	The Edge Enabler Server sends UE location notification to the Edge Application Server. The Edge Enabler Server includes the location of the UE and optionally the timestamp of the location.

The Edge Enabler Server may modify the format of location information to fit to the location granularity requested from the Edge Application Server during the subscribe operation, if supported. For example, if the Edge Enabler Server receives the UE location in a format of tracking area ID or cell ID, the Edge Enabler Server can modify the representation of the location information to the GPS Coordinates, or civic addresses (e.g. streets, districts, etc.) as requested from the Edge Application Server.

###### 8.6.2.2.2.3	Subscription update

Figure 8.6.2.2.2.3-1 illustrates the subscription update operation between the Edge Application Server and the Edge Enabler Server.

Pre-conditions:

1.	The Edge Application Server has subscribed to UE location API provided by the Edge Enabler Server.

Figure 8.6.2.2.2.3-1: UE location API: Subscription update operation

1.	The Edge Application Server sends UE location subscription update request to the Edge Enabler Server. The request includes the subscription identifier of the subscription to be updated and the UE identifier of the same same UE of which the location was subscribed. The UE location subscription update request may also include updated Location granularity and Location accuracy information.

2.	The Edge Enabler Server checks if the request from the Edge Application Server is authorized. If authorized, the Edge Enabler Server updates the subscription request and sends the UE location subscription update response to the Edge Application Server.

###### 8.6.2.2.2.4	Unsubscribe

Figure 8.6.2.2.2.4-1 illustrates the unsubscribe operation between the Edge Application Server and the Edge Enabler Server to stop the UE location notifications.

Pre-conditions:

1.	The Edge Application Server has subscribed to UE location API provided by the Edge Enabler Server.

Figure 8.6.2.2.2.4-1: UE location API: unsubscribe operation

1.	The Edge Application Server sends the UE location unsubscribe request to the Edge Enabler Server.

2.	The Edge Enabler Server checks if the Edge Application Server is authorized or not. If authorized, the Edge Enabler Server terminates the subscription of the Edge Application Server and sends success response to the Edge Application Server.

* * * Next Change * * * *

#### 8.6.2.3	Information flows

##### 8.6.2.3.1	UE location request

Table 8.6.2.3.1-1: UE location request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | Identifies the UE for which location will be reported (e.g. GPSI, or an access token which not only identifies the UE, but also provides the end user's consent for reporting UE's location to a given EAS) |
| Security credentials | M | Securtiy credentials of the EAS |
| Location Granularity | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the Edge Application Server |
| Location accuracy | O | Indicates accuracy of the location information |

##### 8.6.2.3.2	UE location response

Table 8.6.2.3.2-1: UE location response

| Information element | Status | Description |
| --- | --- | --- |
| UE location | M | Location of the UE |
| Timestamp | O | Indicates the age of the UE's location information |
| Location accuracy | O | Indicates accuracy of the location information |

##### 8.6.2.3.3	UE location subscribe request

Table 8.6.2.3.3-1: UE location subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | Identifies the UE for which location will be reported (e.g. GPSI, or an access token which not only identifies the UE, but also provides the end user's consent for reporting UE's location to a given EAS) |
| Security credentials | M | Securtiy credentials of the EAS |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Location Granularity | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the Edge Application Server |
| Location accuracy | O | Indicates accuracy of the location information |

##### 8.6.2.3.4	UE location subscribe response

Table 8.6.2.3.4-1: UE location subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the subscription is successful or failed |
| Cause (NOTE 1) | O | Indicates the cause of subscription request failure |
| Subscription ID (see NOTE 2) | O | Subscription identifier corresponding to the subscription |
| NOTE 1: The Cause IE is included when the Result indicates failure. NOTE 2: The Subscription ID IE are included only when the Result indicates success. |  |  |

##### 8.6.2.3.5	UE location notification

Table 8.6.2.3.5-1: UE location notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| UE location | M | Location of the UE |
| Timestamp | O | Indicates the age of the location information |
| Location accuracy | O | Indicates accuracy of the location information |

##### 8.6.2.3.6	UE location subscription update request

Table 8.6.2.3.6-1: UE location subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| Security credentials | M | Securtiy credentials of the EAS |
| UE ID | M | UE ID of the same UE of which the location was subscribed (e.g. GPSI, or an access token which not only identifies the UE, but also provides the end user's consent for reporting UE's location to a given EAS). |
| Notification Target Address (NOTE) | O | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Location Granularity (NOTE) | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the Edge Application Server |
| Location accuracy (NOTE) | O | Indicates accuracy of the location information |
| NOTE: At least one of these IEs is present. |  |  |

##### 8.6.2.3.7	UE location subscription update response

Table 8.6.2.3.7-1: UE location subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the subscription update is successful or failed. |
| Subscription ID | M | Subscription identifier corresponding to the subscription |
| Cause (NOTE) | O | Indicates the cause of subscription update failure |
| NOTE: The Cause IE is included when the Result indicates failure. |  |  |

##### 8.6.2.3.8	UE location unsubscribe request

Table 8.6.2.3.8-1: UE location unsubscribe

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| Security credentials | M | Securtiy credentials of the EAS |

##### 8.6.2.3.9	UE location unsubscribe response

Table 8.6.2.3.9-1: UE location unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the request to unsubscribe is successful or failure |
| Cause | O | Indicates that the cause information for subscription failure |
| NOTE: The Cause IE is included when the Result indicates failure. |  |  |

#### 8.6.2.4	APIs

##### 8.6.2.4.1	General

Table 8.6.2.4.1-1 illustrates the API for UE location.

Table 8.6.2.4.1-1: UE location API

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| EDGE3_UE_location API | Request | Request/Response | Edge Application Server |
|  | Subscribe | Subscribe/Notify | Edge Application Server |
|  | Notify | Subscribe/Notify | Edge Application Server |
|  | Subscription_Update | Subscribe/Notify | Edge Application Server |
|  | Unsubscribe | Subscribe/Notify | Edge Application Server |

##### 8.6.2.4.2	EDGE3_ UE_location_request operation

Service operation name: EDGE3_ UE_location_request

Description: The consumer requests one time report of a UE's location.

Inputs: See clause 8.6.2.3.1.

Outputs: See clause 8.6.2.3.2.

See clause 8.6.2.2.1 for details of usage of this operation.

##### 8.6.2.4.3	EDGE3_ UE_location_subscribe operation

Service operation name: EDGE3_ UE_location_subscribe

Description: The consumer subscribes for continuous reporting of a UE's location.

Inputs: See clause 8.6.2.3.3.

Outputs: See clause 8.6.2.3.4.

See clause 8.6.2.2.2.1 for details of usage of this operation.

##### 8.6.2.4.4	EDGE3_ UE_location_notify operation

Service operation name: EDGE3_ UE_location_notify

Description: The consumer is notified with report of the UE's location.

Inputs: See clause 8.6.2.3.5.

Outputs: None.

See clause 8.6.2.2.2.2 for details of usage of this operation.

##### 8.6.2.4.5	EDGE3_ UE_location_subscription_update operation

Service operation name: EDGE3_ UE_location_subscription_update

Description: The consumer updates an existing subscription for continuous reporting of a UE's location.

Inputs: See clause 8.6.2.3.6.

Outputs: See clause 8.6.2.3.7.

See clause 8.6.2.2.2.3 for details of usage of this operation.

##### 8.6.2.4.6	EDGE3_ UE_location_unsubscribe operation

Service operation name: EDGE3_ UE_location_unsubscribe

Description: The consumer cancels an existing subscription for continuous reporting of a UE's location.

Inputs: See clause 8.6.2.3.8.

Outputs: See clause 8.6.2.3.9.

See clause 8.6.2.2.2.4 for details of usage of this operation.