3GPP TSG-SA WG6 Meeting #37-e	S6-200760

e-meeting, 14th – 26th May 2020	(revision of S6-200491)

Source:	Samsung

Title:	Pseudo-CR on UE identifier API

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution proposes to include the UE identifier API in the TS. This capability of EES, exposed towards EAS, was discussed during the study phase – solution #5.

2. Reason for Change

In order to invoke the capability exposure APIs such as UE location reporting API, the EAS needs to identify the UE. This procedure provides an API for the Edge Application Servers to obtain the UE Identifier from the Edge Enabler Servers, using the user information (e.g., IP address) available with the EAS.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0.

* * * First Change * * * *

### 8.x.y	UE Identifier API

#### 8.x.y.1	General

EES exposes UE Identifier API to the EAS in order to provide an identifier uniquely identifying a UE. This API is used by an EAS to obtain the identifier of the UE if the EAS does not have it. This identifier, called Edge UE ID, is used by the EAS to invoke capability APIs specific to UEs over EDGE-3.

#### 8.x.y.2	Procedure

Figure 8.x.y.2-1 illustrates the interactions between the Edge Enabler Server and the Edge Application Server.

Pre-conditions:

1.	The Edge Application Server is authorized to discover and to use UE Identifier API provided by the Edge Enabler Server.

Figure 8.x.y.2-1: UE Identifier API

1.	The Edge Application Server invokes UE Identifier API exposed by the Edge Enabler Server

2.	The Edge Enabler Server uses the received user information in the step 1 (e.g. IP address) and obtains the UE identifier.

Editor's note:	A mechanism for EES to translate the EAS provided information to Edge UE ID is needed (e.g. based on input from the EEC).

Editor's note:	A SA2 defined mechanism to obtain the UE identifier from the 3GPP network using the user information (e.g., IP address) is needed.

3.	The Edge Enabler server provides the obtained UE identifier as Edge UE ID to the Edge Application Server. The Edge UE ID is specific to the given Edge Application Server and may be assigned by the EES or the 3GPP Network.

Editor's note:	Whether and how user's consent is obtained to share the UE identifier with a particular EAS is SA3's responsibility.

4.	The Edge Application Server uses the Edge UE ID received in step 3 to invoke capability exposure API(s) provided by the Edge Enabler Server over EDGE-3.

The Edge Enabler Server can provide an updated Edge UE ID to the Edge Application Server if the Edge UE ID has changed due to privacy reason (e.g., change of GPSI).

The Edge Enabler Server can also invalidate an Edge UE ID, previously provided to an Edge Application Server, if there is no need to support the Edge UE ID for capability exposure API(s) anymore.

#### 8.x.y.3	Information flows

##### 8.x.y.3.1	UE Identifier API request

Table 8.x.y.3.1-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| User information | M | Information about the User or UE available with the EAS for e.g. ACR (Anonymous Customer Reference as specified in OMA-TS-REST_NetAPI_ACR [xx]) or the IP address etc. |
| Security Credentials | M | Security credentials of the EAS. |

##### 8.x.y.3.2	UE Identifier API response

Table 8.x.y.3.1-2: UE Identifier API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure |
| Edge UE ID (NOTE) | O | Identifier uniquely identifying the UE. |
| NOTE : Edge UE ID is included only when the Result IE indicates success. |  |  |

#### 8.x.y.4	APIs

Editor's note:	APIs to be provided.