3GPP TSG-SA WG6 Meeting #37-e	S6-200771

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	CATT, China Unicom

Title:	Pseudo-CR on traffic related functions update

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Ling Zhang, zhangling@catt.cn

1. Introduction

This contribution specifies traffic related functions in EES function and EDGE-2 descriptions.

2. Reason for Change

The current specification defines the access of EES to 5GC via NEF or SCEF. When regarded as a trusted AF to 5GC, the EES can perform appropriate traffic steering for the UEs by direct interactions with PCF. Hence the Application Function influence on traffice routing as defined in TS 23.501 as well as TS 23.503 should be supported.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedure for the 5G System; Stage 2".

[4]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[5]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[8]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[9]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[10]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[11]	3GPP TS 23.003: "Numbering, addressing and identification".

[xx]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

* * * Second Change * *  *

### 6.3.2	Edge Enabler Server

Edge Enabler Server (EES) provides supporting functions needed for Edge Application Servers and Edge Enabler Client.

Functionalities of Edge Enabler Server are:

a)	provisioning of configuration information to enable the exchange of Application Data Traffic with the Edge Application Server;

b)	providing information related to the Edge Application Servers to the Edge Enabler Client; and

c)	supporting the functionalities of API invoker and API exposing function as specified in 3GPP TS 23.222 [6]; and

d)	interacting with 3GPP Core Network for accessing the capabilities of network functions either directly (e.g. via PCF) or indirectly (e.g. via SCEF/NEF/SCEF+NEF).

* * * Third Change * * * *

### 6.4.3	EDGE-2

EDGE-2 reference point enables interactions between the Edge Enabler Server and the 3GPP Core Network. It supports:

a)	access to 3GPP Core Network functions and APIs for retrieval of network capability information, e.g. via SCEF and NEF APIs as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3], 3GPP TS 29.522 [4], 3GPP TS 29.122 [5], and with the EES acting as a trusted AF in 5GC (see 3GPP TS 23.501 [2] clause 5.13, 3GPP TS 23.503 [xx]).

NOTE:	EDGE-2 reference point reuses 3GPP reference points or interfaces of EPS or 5GS considering different deployment models.