3GPP TSG-SA WG6 Meeting #37e	S6-200780

E-meeting, 14th – 26th May 2020	(revision of S6-200xxx)

Source:	Huawei, Hisilicon

Title:	Basic application context relocation procedure

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

This pCR is proposed to a application context relocation procedure.

2. Reason for Change

A general procedure is proposed which focuses on the interaction from source EES to target EAS and leaves all the possible options to start this whole procedure as the triggers at source EES.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0.

* * * First Change * * * *

## 8.8	Capabilities to support service continuity

### 8.8.2	Procedures

#### 8.8.2.aa	Basic application context relocation procedure

In this procedure, the application context relocation is performed from a source EAS to a target EAS. Further, the on-going application traffic related to an Application Client at the UE is routed or switched from a source EAS to a target EAS. The Application Client behaviour at the UE is unaffected by this procedure.

The high level signalling flows for application context relocation is illustrated in the figure 8.x.2.aa-1.

Pre-conditions:

1.	The UE already has on-going application traffic with the source Edge Application Server.

2.	The Edge Enabler Client at the UE and the source Edge Application Server has subscribed to the source Edge Enabler Server for application context relocation event notifications.

3.	The source Edge Enabler Service has subscribed with 3GPP Core Network on user plane management related events.

Figure 8.8.2.aa-1: Basic Application context relocation procedure

1.	The source Edge Enabler Server determines the application context relocation related to a UE application traffic based on triggers/events (e.g. Application context relocation request from source Edge Application Server, UE is moving to another EDN, source EAS overload) at the source Edge Enabler Server.

2.	The source Edge Enabler Server communicates with Edge Configuration Server, which provides application context relocation assistance based on the request from the source Edge Enabler Server and the information about other Edge Enabler Server(s), to retrieve information of the target Edge Enabler Server which has the registered Edge Application Server that can serve the Application Client at the UE. This procedure is specified in clause 8.8.2.bb.

3.	The source Edge Enabler Server sends the application context relocation request (source Edge Application Server information, UE information, CN information) to the target Edge Enabler Server retrieved in step 2. The source Edge Application Server information includes the EAS ID and the IP address of the source Edge Application Server. The CN information (such as the subscription to NEF, PCF) may be included to assist the target Edge Enabler Server to interact with the 5GC.

4.	The target Edge Enabler Server determines the target Edge Application Server as per the EAS ID received in step 3.

5.	The target Edge Enabler Server sends the application context relocation request (source Edge Application Server information, UE information) to the target Edge Application Server for application context relocation.

6.	When the target Edge Application Server accepts the request and is well prepared for the application context relocation, the target Edge Application Server sends the application context relocation response to the target Edge Enabler Server. The IP address of target Edge Application Server is included. Also other information such as the IP address and port number and protocol used for receiving the application context are also included, if required.

NOTE 1:	If target Edge Enabler Server has enough information about the target EAS regarding application context transfer, then steps 5 and 6 can be skipped.

7.	The target Edge Enabler Server sends the application context relocation response to the source Edge Enabler Server.

8.	The source Edge Enabler Server sends the application context relocation notification with the IP address of the target Edge Application Server to the source Edge Application Server (8a). The source Edge Enabler server sends application context relocation notification to suspend the application traffic to the Edge Enabler Client (8b).

9.	When the source Edge Application Server determines the point that the application context can be transferred, the source Edge Application Server delivers the application context to the target Edge Application Server via the source EES and target EES.

NOTE 2:	This step can use methods to transfer the application context directly from the source Edge Application Server to the target Edge Application Server. Such methods are implementation specific and out of scope of this specification.

10.	When the application context transfer is completed, the application context transfer complete message is sent from target Edge Application Server to the source Edge Application Server via the target EES and the source EES (10a). The source Edge Enabler Server sends the application context transfer complete message to resume application traffic (10b).

11.	Upon receiving the user plane management notification from 3GPP core network, the source Edge Enabler Server provides a notification response as per the AF traffic influence procedure as described in 3GPP TS 23.502 [3]. The application traffic from the UE is now routed to the target EAS.

NOTE 3:	The notification from the 3GPP core network may be received anytime until step 10.

NOTE 4:	Step 11 is applicable for 5GC.

Editor's note:	Whether and how to handle the case where there is no EAS deployed in the target EDN is FFS.