3GPP TSG-SA WG6 Meeting #37e	S6-200784

E-meeting, 14th – 26th May 2020	(revision of S6-200xxx)

Source:	Huawei, Hisilicon

Title:	Utilizing network capabilities from 3GPP core network

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

This pCR provides a proposal for utilizing network capabilities from 3GPP core network.

2. Reason for Change

The functional entities of the edge enabler layer can utilize network capabilities from 3GPP core network. The details of the utilization of 3GPP CN capabilities are proposed to be captured separately.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0.

* * * First Change * * * *

## 8.N	Utilizing 3GPP core network capabilities

### 8.N.1	General

The functional entities of the Edge Enabler Layer can utilize the 3GPP core network capabilities (i.e. 5GC, EPC) to fulfill the needs of the edge service operations. This clause specifies the details of the 3GPP network capabilities consumed by each functional entity.

### 8.N.2	Capabilities utilized by Edge Configuration Server

When required for the edge service operations (e.g. service provisioning) at the Edge Configuration Server, the Edge Configuration Server may utilize:

-	user plane path management events by subscribing with the 3GPP core network for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

-	the location information from the API exposed by 3GPP core network, e.g. SCEF/NEF/SCEF+NEF or LCS (Location Service) as specified in 3GPP TS 23.502 [3], 3GPP TS 23.271 [7], 3GPP TS 36.305 [8], 3GPP TS 23.273 [9] and 3GPP TS 38.305 [10] to obtain the UE's location from the 3GPP Core Network.

### 8.N.3	Capabilities utilized by Edge Enabler Server

When required for the edge service operations (e.g. EAS discovery) at the Edge Enabler Server, the Edge Enabler Server may utilize:

-	user plane path management events by subscribing with the 3GPP core network for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

-	the location information from the API exposed by 3GPP core network, e.g. SCEF/NEF/SCEF+NEF or LCS (Location Service) as specified in 3GPP TS 23.502 [3], 3GPP TS 23.271 [7], 3GPP TS 36.305 [8], 3GPP TS 23.273 [9] and 3GPP TS 38.305 [10] to obtain the UE's location from the 3GPP Core Network.