3GPP TSG-SA WG6 Meeting #37-e	S6-200789

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Update to EAS discovery procedure

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed to update the EAS discovery procedure.

2. Reason for Change

When UE requests for EAS discovery, currently, the registered EAS based on the application client information of the UE is returned by EES.

However, there are some cases where the UE is always associated with one or more application client(s) and the UE does not have to explicitly communicate the application client(s) information to the EES as the relationship of the UE and the application client(s) does not change. For such application dedicated UE(s), there is a need that the UE identifier can be used to resolve the EAS discovery without any application client(s) information sent from the UE to the EES.

For example, consider future of the factory scenario, where some IoT devices that are deployed in fixed locations and such IoT devices have dedicated applications (e.g. Specialized sensor device, Camera). The deployed EES for such scenario provide support for these application dedicated IoT devices. There is no requirement to re-select the EES for such application dedicated IoT devices. Usually for such application dedicated IoT devices, the UE identifier or Device identifier is indicative of the application itself.

Thus, it is required that the EAS discovery procedure shall support both cases

-	the EAS being discovered is determined based on requesting application specified in the EAS discovery request

OR

-	the UE identifier or Device identifier.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0.

### 8.5.2	Edge Application Server Discovery

#### 8.5.2.1	General

Edge Application Server discovery enables Edge Enabler Clients to obtain information about available Edge Application Servers of interest.

EAS discovery is supported for a wide range of UEs. Such UEs may be highly capable of supporting any application or dedicated to a limited set of applications.

The identification of the Edge Application Servers being discovered is based on:

- matching query filters or Application Client Profiles provided in the Edge Application Server discover request; or

- the relationship of the requesting UE identifier and one or more supported Application Client profiles. This relationship is part of the UE subscription data of the ASP/ECSP/PLMN operator.

Editor's note:	How the UE subscription data is made available at the EES is FFS?

#### 8.5.2.2	Procedure

Pre-conditions:

1.	Information related to the Edge Enabler Server is available with the Edge Enabler Client.

Figure 8.5.2.2-1: Edge Application Server Discovery procedure

1.	The Edge Enabler Client sends an Edge Application Server discovery request to the Edge Enabler Server. The discovery request may contain query filters to retrieve information about a particular Edge Application Server or a category of Clarification EAS endpoint returned to EEC, e.g. gaming applications. Query filters may also indicate a desired level of service permissions (e.g. trial, gold-class, etc.), desired features (e.g. multi-player or single player), or desired location availability.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Enabler Server determines the UE identifier and checks if the Edge Enabler Client is authorized to discover the requested Edge Application Server(s). The authorization check may apply to an individual Edge Application Server, a category of Edge Application Servers or to the Edge Data Network, i.e. to all the Edge Application Servers. If the discovery request contained query filters that indicated a desired level of service permissions, features, and/or location availability, then the Edge Enabler Server may also check if the discovered Edge Application Servers are able to provide the desired level of service permissions, features, and/or location availability.

The Edge Enabler Server determines a set of registered Edge Application Servers that match the query filters as defined in table 8.5.2.3.1-2 , if provided in the request. If the request does not include query filter(s) or if the request does not include EAS ID(s) orApplication Client Profile(s) in the query filter(s), then the EES determines a set of registered Edge Application Servers based on the relationship of the UE identifier and corresponding supported application client profile(s), as per UE subscription data.

3.	The Edge Enabler Server sends an Edge Application Server discovery response including information about the discovered Edge Application Servers. For registered discovered Edge Application Servers, this includes endpoint information. Depending on the query filters or Application Client Profiles in the discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, Application Client locations(s) that the Edge Application Servers can support, KPIs, etc.

Upon receiving the Edge Application Server discovery response, the Edge Enabler Client uses the endpoint information for routing of the outgoing application data traffic to Edge Application Server(s), as needed, and may provide necessary notifications to the Application Client(s).

#### 8.5.2.3	Information flows

##### 8.5.2.3.1	Edge Application Server discovery request

Table 8.5.2.3.1-1 describes information elements for the Edge Application Server Discovery Request from the Edge Enabler Client to the Edge Enabler Server. Table 8.5.2.3.1-2 provides further detail about the Query Filter information element.

Table 8.5.2.3.1-1: Edge Application Server discovery request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the Edge Enabler Client. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| List of Query Filter(s) | O | List of characteristics per Application for discovery purposes, as detailed in Table 8.5.2.3.1-2. |

Table 8.5.2.3.1-2: Application Query Filter

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID list | O | List of EAS IDs to be discovered. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| Application provider(s) | O | Provider of the Application Client |
| Service permission level(s) | O | Desired level of service permissions e.g. trial, gold-class |
| Desired feature(s) | O | Desired service features e.g. single vs. multi-player gaming service |
| Location availability | O | The desired location(s) (e.g. geographical area, route) where the service should be available. |
| Application Client Profile(s) | O | Profiles of Application Clients with parameters used to determine matching EA. Application Client Profiles are further described in Table 8.2.2-1. |

8.5.2.3.2	Edge Application Server discovery response

Table 8.5.2.3.2-1 describes information elements for the Edge Application Server Discovery Response from the Edge Enabler Server to the Edge Enabler Client.

Table 8.5.2.3.2-1: Edge Application Server discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure to satisfy the EAS discovery request |
| Reason (NOTE 1) | O | The reason for failure. |
| Discovered EAS list (NOTE 2) | O | List of discovered EAS. Each element corresponds to the information related to one EAS and includes the information described below. |
| >EAS profile | M | The EAS profile as specified in clause 8.2.4 corresponding to the discovered EAS |
| NOTE 1: The Reason IE is included when the Result IE indicates failure. NOTE 2: The Discovered EAS list IE is included when the Result IE indicates success. |  |  |

Editor's Note:	The use of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

* * * End of Change * * * *