3GPP TSG-SA WG6 Meeting #37-e	S6-200790

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Support SEAL and VAE capabilities

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed to support the SEAL server capabilities and VAE server capabilities in EDGE architecture.

2. Reason for Change

The SEAL server capabilities and VAE server capabilities can be exposure to EAS via the EES architecture. The CAPIF based mechanism ,SEAL based meachnism and VAE based mechnism can be used to expose the SEAL service APIs and VAE APIs to the Edge Application Servers.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedure for the 5G System; Stage 2".

[4]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[5]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[8]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[9]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[10]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[11]	3GPP TS 23.003: "Numbering, addressing and identification".

[aa]	3GPP TS 23.434: "Service enabler architecture layer for verticals; Functional architecture and information flows; Stage 2".

[bb]	3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows ".

* * * Next Change * * * *

### 6.3.2	Edge Enabler Server

Edge Enabler Server (EES) provides supporting functions needed for Edge Application Servers and Edge Enabler Client.

Functionalities of Edge Enabler Server are:

a)	provisioning of configuration information to enable the exchange of Application Data Traffic with the Edge Application Server;

b)	providing information related to the Edge Application Servers to the Edge Enabler Client;

c)	support the functionalities of API invoker and API exposing function as specified in 3GPP TS 23.222 [6];

d)	support the exposure of functionalities of SEAL server functions as specified in 3GPP TS 23.434 [aa]; and

e)	support the exposure of functionalities of VAE server function as specified in 3GPP TS 23.286 [bb].

* * * Next Change * * * *

### 6.4.4	EDGE-3

EDGE-3 reference point enables interactions between the Edge Enabler Server and the Edge Application Servers. It supports:

a)	registration of Edge Application Servers with availability information (e.g. time constraints, location constraints);

b)	de-registration of Edge Application Servers from the Edge Enabler Server;

c)	providing access to network capability information (e.g. location information);

d)	SEAL server APIs over SEAL-S reference point as specified in 3GPP TS 23.434 [aa]; and

e)	VAE server APIs over Vs reference point as specified in 3GPP TS 23.286 [bb];

NOTE:	Optimized distribution of events across the EDGE-3 interface is the responsibility of CT groups.

* * * Next Change * * * *

### 8.6.X	Support SEAL service APIs

#### 8.6.X.1	General

There are several options to support SEAL service APIs to be exposed to the Edge Application server.

The Edge Enabler server may act as the CAPIF core function, and the SEAL servers acting the AEF and publish the SEAL service API to the Edge Enabler server. Further, the SEAL service APIs is discovered by the Edge Application Servers acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [6].

The Edge Enabler Server may integrate the SEAL server functions and act as SEAL server instance(s), and the Edge Application server acts as VAL server. The Edge Enabler server exposes SEAL service APIs to Edge Application server via EDGE-3 which utilizes the SEAL-S reference point as specified in 3GPP TS 23.434 [aa].

### 8.6.Y	Support Vertical Application Enabler server APIs

#### 8.6.Y.1	General

There are several options to support vertical application enabler server (e.g., V2X application enabler server) APIs to be exposed to the Edge Application server.

The Edge Enabler server may act as the CAPIF core function, and the vertical application enabler server acting the AEF and publish the vertical application enabler server APIs to the Edge Enabler server. Further, the vertical application enabler server APIs is discovered by the Edge Application Servers acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [6].

The Edge Enabler Server may integrate the vertical application enabler server function and act as vertical application enabler server instance, and the Edge Application server acts as VAL server. The Edge Enabler server exposes vertical application enabler server APIs to Edge Application server via EDGE-3. When the vertical application enabler server is the VAE server the EDGE-3 supports the functionality of Vs reference point as specified in 3GPP TS 23.286 [bb].

* * * End of Change * * * *