3GPP TSG-SA WG6 Meeting #37-e	S6-200790

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Support SEAL and VAE capabilities

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed to support the SEAL server capabilities and VAE server capabilities in EDGE architecture.

2. Reason for Change

The SEAL server capabilities and VAE server capabilities can be exposure to EAS via the EES architecture. The CAPIF based mechanism ,SEAL based meachnism and VAE based mechnism can be used to expose the SEAL service APIs and VAE APIs to the Edge Application Servers.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0.

* * * First Change * * * *

-

* * * Next Change * * * *

### A.X	Deployment of EES in relation with SEAL services and Application Enabler Services

### A.X.1	Deployment of SEAL services

The Edge Enabler server can act as the CAPIF core function. The SEAL servers can act as the AEF and publish the SEAL service API to the CAPIF core functionand the SEAL service APIs is discovered by the Edge Application Servers acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [6].

A.X.2	Deployment of Application Enabler services

The Edge Enabler server can act as the CAPIF core function. The vertical application enabler server can act  as the AEF and publish the vertical application enabler server APIs to the CAPIF core functionand the vertical application enabler server APIs is discovered by the Edge Application Servers acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [6].

* * * End of Change * * * *