3GPP TSG-SA WG6 Meeting #37-e	S6-200817_Rev1

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Title:	[draft] LS on Application Registration in relation to mgmt. aspects

Release:	Rel-17

Work Item:	EDGEAPP

Source:	SA6

To:	SA5

Contact Person:

#### Name:	Omar Elloumi

E-mail Address:	omar.elloumi@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA6 is working on the Architecture for enabling Edge Applications in Rel-17. SA6 TS 23.558 has developed Edge Application Server (EAS) registration to Edge Enabler Server (EES). The registration procedure includes the following information elements:

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Service Area | O | The geographical service area that the EAS serves |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heart beat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

A brief definition of most important functions is provided below:

- “Edge Application Server (EAS) is the application server resident in the Edge Data Network, performing the server functions. The Application Client connects to the Edge Application Server in order to avail the services of the application with the benefits of Edge Computing.”

- “Edge Enabler Server (EES) provides supporting functions needed for the Edge Enabler Client to support Application Clients to connect with Edge Application Servers.”

- “Edge Enabler Client (EEC) provides supporting functions needed for Application Client(s).”

More details can be consulted in section 6.3 of TS 23.558 V0.2.0.

Since some of the EAS profile information elements may require inputs from the management plane, SA6 believes such elements may require provisioning into EES directly from the management plane.

2. Actions:

ACTION: 	SA6 asks SA5 to kindly look into EAS Profile and provide feedback on which information elements can be provisioned into EES from management plane directly.

3. Date of Next SA6 Meetings:

SA6#38 20th – 30th July 2020	e-meeting