3GPP TSG-SA WG6 Meeting #37-e	S6-200xxx

e-meeting, 14th – 26th May 2020	(revision of S6-200723)

Source:	Ericsson

Title:	Pseudo-CR on Network capability exposure clarification to EAS

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution introduces some clarifications for the network capability exposure to EAS

2. Reason for Change

In previous meeting, it was agreed:

“The Edge Application Server interacts with the CAPIF core function and the Edge Enabler Server. The Edge Enabler Server interacts (directly or via SCEF/NEF) with the underlying 3GPP core network entities to satisfy the API requests from the Edge Application Servers.”

This statement not 100% correct since the CAPIF only supports the northbound APIs (e.g. NEF API) to publish AEF services via APF, the 3GPP core network internal services (e.g. PCF services) are not managed by the CAPIF. Therefore the statement should be revised to:

“The Edge Application Server interacts with the CAPIF core function and the Edge Enabler Server. The Edge Enabler Server acts as the API topology hiding entry and interacts with 3GPP northbound service API provider (e.g. SCEF/NEF) to satisfy the API requests from the Edge Application Servers.”

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

### 8.7.3	Network capability exposure via Edge Enabler Server

The Edge Enabler Server may re-expose the network capabilities of the 3GPP core network to the Edge Application Server(s) as per the CAPIF architecture specified in 3GPP TS 23.222 [6].

Depending on the deployment models (centralized or distributed) employed,

-	the Edge Enabler Server assumes the role of the API exposing function (may also acts as the API topology hiding entry) as described in 3GPP TS 23.222 [6]; and

-	the Edge Application Server assumes the role of an API invoker.

If CAPIF is supported, the Edge Application Server interacts with the CAPIF core function and the Edge Enabler Server. The Edge Enabler Server may further interact with 3GPP northbound service API provider (e.g. SCEF/NEF/SCEF+NEF) or directly with 3GPP core network entities (e.g. PCF) to satisfy the service API invocation requests from the Edge Application Servers.

Editor's note: Whether and how Edge Enabler Server can re-expose 3GPP network capabilities to Edge Application Server without CAPIF is FFS.

* * * End of Changes * * * *