3GPP TSG-SA WG6 Meeting #37-e	S6-200xxx

e-meeting, 14th – 26th May 2020	(revision of S6-200724)

Source:	Ericsson

Title:	Pseudo-CR on clarification for EDGE3_User_plane_management_event_subscribe

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution solves editor note in clause 8.6.3.3.1.

2. Reason for Change

There is an editor note in table 8.6.3.3.1-1:

Editor's note:	The detailed applicable information in "Event Report" is FFS.

The event report information in TS 23.502 has:

| Event Reporting Information Parameter | Description | Presence requirement |
| --- | --- | --- |
| Event reporting mode | Mode of reporting - e.g reporting up to a maximum number of reports, periodic reporting along with periodicity, reporting up to a maximum duration | mandatory |
| Maximum number of reports | Maximum number of reports after which the event subscription ceases to exist | (see NOTE 2) |
| Maximum duration of reporting | Maximum duration after which the event subscription ceases to exist | (see NOTE 2) |
| Immediate reporting flag | The Event provider NF notifies the current status of the subscribed event, if available, immediately to the consumer NF. |  |
| Sampling ratio | Percentage of sampling (1%..100%) among impacted UEs. | optional (see NOTE 3) |
| Group Reporting Guard Time | Parameter for group-based monitoring configuration to indicate the time for which the Monitoring Event Reporting(s) related with the UEs in a group can be aggregated before sending them to the consumer NF. | optional |
| NOTE 2: The requester shall include 2) Maximum number of reports or 3) Maximum duration of reporting, or both, depending on 1) Event reporting mode. NOTE 3: Parameter only applicable to certain event IDs reporting metrics (e.g. Number of UEs present in a geographical area) used and used e.g. by the NWDAF for data collection. |  |  |

The EES southbound interaction with 3GPP 5GC uses the UP path change event in the AF traffic influence function as described in TS 23.501, clause 5.6.7, and the UP path change has different reporting information supported. More specifically, in table 5.6.7-1 of TS 23.501, it only supports event reporting mode of “on event detection”. The immediate reporting is not supported, periodic reporting is not supported either.

Of course, event reporting mode of “one time reporting” can be supported by the EES by unsubscribing the PCF/NEF API once the 1st report is received, EES can also control the max. number of report and reporting period so to provide an enhanced version of UP path change with more event reporting control. But the following reporting information should not be applicable for EES:

- Periodic reporting (either nothing to report or the report is delayed which is not good for edge computing)

- Sampling ratio & Group reporting Guard time (not applicable for the UP path change event)

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

##### 8.6.3.3.1	User plane path management event API subscribe request

Table 8.6.3.3.1-1 describes the information elements for a User Plane Path Management Event API Subscribe Request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.1-1: User plane path management event API subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the UE (i.e. GPSI or identity token) |
| Event ID(s) | M | Event ID(s) as specified in 3GPP TS 23.501 [2] |
| Event Report | M | Event Reporting Information as specified in table 4.15.1-1 of 3GPP TS 23.502 [3]. (NOTE) |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Event Filter | O | Event filter as specified in table 5.6.7-1 of 3GPP TS 23.501 [2] and only the information relevant for the user plane path change event subscription is applicable. |
| NOTE: The Sampling ratio and Group Reporting Guard Time are not applicable, and the periodic reporting is not applicable either. |  |  |

Editor's note:	Whether the immediate reporting can be supported is FFS.

* * * End of Changes * * * *