3GPP TSG-SA WG6 Meeting #37-e	S6-200772

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	CATT

Title:	Pseudo-CR on new key issue on Local MBMS support

Spec:	3GPP TR 23.764 v0.5.0

Agenda item:	8.3

Document for:	Approval

Contact:	Ling Zhang, zhangling@catt.cn

1. Introduction

This contribution addresses a new key issue on Local MBMS support.

2. Reason for Change

The Local MBMS (L.MBMS) was introduced in TS 23.285 since Release 15 for the purpose of MBMS latency improvements for V2X communication. To support faster session setup in Local MBMS the pre-configuration (e.g. M1 interface infromation and transport network IP Multicast Address) is used instead of allocation of IP multicast distribution on the go. This brings more required work at the V2X Application Server to perform the preconfiguration and session handling.

The TS 23.286 leverages the multicast resource management capability by SEAL layer and doesn’t specify the use of  Local MBMS features.

It is beneficial to study the V2X application layer support for Local MBMS as the proposal below.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.764 v0.5.0.

* * * First Change * * * *

## 5.X	Key issue X – Support for Local MBMS/MBS

The Local MBMS was introduced in 3GPP TS 23.285 [6] for the purpose of MBMS latency improvements. To support Local MBMS the V2X Application Server can provide the transport network level informaiton to the BM-SC (see subclause 5.4.2.2 of 3GPP TS 23.285[6]) via either the MB2 or xMB interface. The V2X application layer should consider how to support the use of Local MBMS functions.

As to 5G aspects, the 3GPP TR 23.757 assumes that the V2X service may restrict multicast or broadcast communication services only available in a local service area and for a specific time. Such assumptions can be considered in both 4G and 5G cases. It is also worth investigating whether and how the V2X application layer support for a local service area of MBMS or MBS.

The following aspects need to be studied:

-	In Local MBMS deployment scenarios which entity of the V2X application layer provides the configurations? Whether the V2X application layer and Local MBMS functions need to coordinate on aligned configuration and usage?

-	If the Local MBMS/MBS has a service area within which the V2X multicast or broadcast services are available, whether and how the V2X application layer support this local service area?