3GPP TSG-SA WG6 Meeting #37-e	S6-200701

E-meeting, 11st May – 21th May 2020

Source:	ZTE Corporation

Title:	FAE client registration

Spec:	3GPP TR 23.745 v0.7.0

Agenda item:	8.4

Document for:	Approval

Contact:			Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution is FAE client registration.

2. Reason for Change

This contribution is for Key Issue 7 - Device Onboarding

The FF Application registration procedures of FF Device Onboarding automatic process (solution #4) include:

-	The FAE client registers to the FAE server via FAE-1 reference point. (In this contribution)

-	The FF application specific client registers to the FAE client via FAE-C reference point. (Specific application related, out of scope of the present document)

-	The FF application specific server registers to the FAE server via FAE-S reference point. (Specific application related, out of scope of the present document)

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.745 v0.7.0.

* * * First Change * * * *

## 7.4	Solution #4: Device Onboarding support in FF

#### 7.4.1.x	FAE client registration

Pre-conditions:

- The FF UE has already connected & registered to 5G network.

- The FF UE is configured with FF application default information (i.e. the physical address of FAE server).

The procedure for FAE client registration is illustrated in figure 7.4.1.x-1.

Figure 7.4.1.x-1: FAE client registration

- The FAE client sends a registration request to the FAE server with FF device information (i.e. software, hardware, capabilities, optional management objects if available).

2.  The FAE server sends a registration response to the FAE client indicating success or failure result.

#### 7.4.1.y	FAE client deregistration

Pre-condition:

- The FAE client has already registered with the FAE server.

- The FAE client can proactive deregistration or on demand of the FAE sever.

The procedure for FAE client deregistration is illustrated in figure 7.4.1.y-1.

Figure 7.4.1.y-1: FAE client deregistration

1.	The FAE client sends a deregistration request to the FAE server.

2.	The FAE server sends a deregistration response to the FAE client.

* * * End Change * * * *