3GPP TSG-SA WG6 Meeting #37-e	S6-200707

E-meeting, 11st May – 21th May 2020

Source:	ZTE Corporation

Title:	QoS monitoring

Spec:	3GPP TR 23.745 v0.7.0

Agenda item:	8.4

Document for:	Approval

Contact:			Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution is QoS monitoring.

2. Reason for Change

This contribution is for Key issue 5 - QoS monitoring

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.745 v0.7.0.

* * * First Change * * * *

## 7.X	Solution #x: QoS monitoring

### 7.X.1	Solution description

This solution addresses Key issue 5 - QoS monitoring.

QoS monitoring is a critical requirement for many FF applications.

The issue to address here via the NEF, according to the procedures defined in 3GPP TS 23.502 [y] clause 4.15. NEF supports APIs of exposure of network events, analytics for QoS monitoring.

The detail procedures over NEF Northbound Interface are defined in 3GPP TS 23.522 [x] clause 4.4:

- Procedures for setting up an AF session with required QoS, which defines QoS monitoring event (QOS_MONITORING) within "QoSMonitoring_5G" feature

- Procedures for analytics information exposure, which defines analytics event (QOS_SUSTAINABILITY) within "QoS_Sustainability" feature

#### 7.x.1.1	QoS monitoring procedure

Pre-conditions:

1.	The FF UE has already registered, configured, provisioned.

2.	The FF UE has an established connection to 5G network.

The procedure for QoS monitoring is illustrated in figure 7.x.1.1-1.

Figure 7.x.1.1-1: QoS monitoring procedures

1.	The FF application specific server sends a QoS monitoring subscribe request to the FAE server.

2.	The FAE server interacts with NEF to subscribe to QoS monitoring events, including:

-	QOS_MONITORING, supported by AsSessionWithQoS API of 3GPP TS 23.522 [x]

-	QOS_SUSTAINABILITY, supported by AnalyticsExposure API API of 3GPP TS 23.522 [x]

3.	The FAE server sends a QoS monitoring subscribe response to the FF application specific server.

4.	When QoS monitoring event is triggered, NEF sends a QoS monitoring notification to the FAE server.

5.	The FAE server notifies the FF application specific server about the QoS monitoring event.

NOTE: Steps 4 and 5 are repeated when QoS monitoring events occur.

6.	When the FF application specific server decides to unsubscribe for the QoS monitoring, the FF application specific server sends a QoS monitoring unsubscribe request to the FAE server.

7.	The FAE server interacts with NEF to unsubscribe from QoS monitoring events.

8.	The FAE server sends a QoS monitoring unsubscribe response to the FF application specific server.

* * * Second Change * * * *

# 2	References

[x]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[y]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

* * * End Change * * * *