3GPP TSG-SA WG6 Meeting #37-e	S6-200708

E-meeting, 11st May – 21th May 2020

Source:	ZTE Corporation

Title:	5GLAN group management

Spec:	3GPP TR 23.745 v0.7.0

Agenda item:	8.4

Document for:	Approval

Contact:			Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution is 5GLAN group management.

2. Reason for Change

Key issue 6 - 5GLAN group management describes integration of 5G LAN type service for group management.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.745 v0.7.0.

* * * First Change * * * *

## 7.X	Solution #x: 5GLAN group management

### 7.x.1	Solution description

This solution function addresses the Key issue #6.

#### 7.x.1.1	General

In factory network, the FF UE is using 5G LAN-type of services, when a FF UE is in a group, which includes 5GLAN related network information.

The FAE capabilities (FAE client and FAE server) utilize the group management service procedures (e.g. creation, join, leave) of SEAL based on the 5GLAN group configuration information (e.g. group join policy, group leader) provided by the FF application specific layer. The decisions and corresponding triggers (e.g. group creation, join, leave) for group management are responsibility of the FF application specific layer and the details of the group management are abstracted by the FAE capabilities. The group management service of SEAL provides support for platooning groups and pre-arranged groups for FF communications.

#### 7.x.1.2	Information flows

The following information flows of group management service of SEAL as specified in 3GPP TS 23.434 [8] are applicable for the V2X applications:

-	Group creation request specified in subclause 10.3.2.1;

-	Group creation response specified in subclause 10.3.2.2;

-	Group creation notify specified in subclause 10.3.2.3;

-	Group information query request specified in subclause 10.3.2.4;

-	Group information query response specified in subclause 10.3.2.5;

-	Group membership update request specified in subclause 10.3.2.6;

-	Group membership update response specified in subclause 10.3.2.7;

-	Group membership notification specified in subclause 10.3.2.8;

-	Group deletion request specified in subclause 10.3.2.9;

-	Group deletion response specified in subclause 10.3.2.10;

-	Group deletion notification specified in subclause 10.3.2.11;

-	Group information request specified in subclause 10.3.2.12;

-	Group information response specified in subclause 10.3.2.13;

-	Group information subscribe request specified in subclause 10.3.2.14;

-	Group information subscribe response specified in subclause 10.3.2.15;

-	Group information notify request specified in subclause 10.3.2.16;

-	Group information notify response specified in subclause 10.3.2.17;

-	Store group configuration request specified in subclause 10.3.2.18;

-	Store group configuration response specified in subclause 10.3.2.19;

-	Get group configuration request specified in subclause 10.3.2.20;

-	Get group configuration response specified in subclause 10.3.2.21;

-	Subscribe group configuration request specified in subclause 10.3.2.22;

-	Subscribe group configuration response specified in subclause 10.3.2.23;

-	Notify group configuration request specified in subclause 10.3.2.24;

-	Notify group configuration response specified in subclause 10.3.2.25;

-	Configure VAL group request specified in subclause 10.3.2.26;

-	Configure VAL group response specified in subclause 10.3.2.27;

-	Group announcement specified in subclause 10.3.2.28;

-	Group registration request specified in subclause 10.3.2.29;

-	Group registration response specified in subclause 10.3.2.30;

The usage of the above information flows are clarified as below:

-	The identity list is the list of FF UE IDs.

--	During group creation the identity list contains the list of FF UE IDs that are part of the group to be created. If the group member list is empty, an empty group is created; and

-	The VAL service ID list is the list of FF application specific server ID whose service communications are to be enabled on the group.

-	The the VAL service specific information corresponding the FF application specific information, i.e. 5G LAN type service information, TSN integration information, group name, group join policy, group leader ID.

-	The VAL group ID is the FF 5GLAN group ID.

-	The identity is the FF UE ID;

-	The VAL server is the FAE server;

-	The VAL group configuration information is the FF 5GLAN group configuration information;

-	The VAL group configuration data is the FF 5GLAN group configuration information;

-	The VAL group information reference is the FF 5GLAN group information reference; and

-	The VAL services requested is the FF application specific server communication service.

#### 7.x.1.3	Procedures

The following procedures of group management service of SEAL as specified in 3GPP TS 23.434 [8] are applicable for the FF applications:

-	Group creation specified in subclause 10.3.3;

--	Group creation support is provided for the FF applications to provide a 5GLAN group for the FF UEs to communicate with each other. The group creation supports group without any group members to enable group member joining.

-	Group information query specified in subclause 10.3.4;

-	Group membership specified in subclause 10.3.5;

-	Group configuration management specified in subclause 10.3.6;

-	Group announcement and join specified in subclause 10.3.8;

### 7.x.2	Solution evaluation

* * * End Change * * * *