3GPP TSG-SA WG6 Meeting #37-e meeting	S6-200742

14th – 26th May 2020	(revision of S6-XXXXXX)

Source:	China Mobile

Title:	Pseudo-CR on FS 5GMARCH Key Issue end-to-end latency

Spec:	3GPP TR 23.700-24 v0.5.0

Agenda item:	8.6

Document for:	Approval

Contact:	Yue Liu (liuyueyjy@chinamobile.com)

- 1. Introduction

This doc proposes the key issue that MSGin5G Service shall fulfill the requirements of end-to-end latency in all Message scenarios.

- 2. Reason for Change

Contribution with new text.

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-24 v0.5.0.

* * * First Change * * * *

# 5	Key Issues

## 5.X	Key Issue X: End-to-end latency

### 5.X.1	Description

In clause 5.1 of 3GPP TS 22.262 [2], the requirement of end-to-end latency is specified as below:

[R-5.1.2-001] The MSGin5G Service shall support UE sending and receiving a text or data message with end-to-end latency less than [500] ms.

NOTE 1:	Initial connection activation latencies may be longer depending on receiving UE power saving states, paging, etc.

The requirement [R-5.1.2-001] is specified in the general clause of service requirements so it will apply to all of the four message scenarios. A similar requirement [R-5.5.2-001] listed below is also listed in clause 5.5 of 3GPP TS 22.262 [2].

[R-5.5.2-001] The MSGin5G Service shall support broadcasting a text or data message with end-to-end latency less than [500] ms.

A further requirement for the Application-to-point message scenario listed below is also specified in 3GPP TS 22.262 [2]:

[R-5.3.2-003] The MSGin5G Service shall support Application Originated Mobile Terminated messaging service with max latency of 10 seconds while maintaining battery life of at least 3 months for small data traffic once every hour and typical sized IOT battery [200-500mAh].

This requirement applies to the AOMT messaging service, so [R-5.1.2-001] still applies to the MOAT messaging service.

Issues include:

- The main factors affecting the end-to-end latency in each message scenario are needed to be studied.

- How to ensure that the end-to-end latency in point-to-point message scenario is less than [500] ms.

- How to ensure that the end-to-end latency in AOMT messaging service is less than [10] seconds.

- How to ensure that the end-to-end latency in MOAT messaging service is less than [500] ms.

- How to ensure that the end-to-end latency in Group message scenario is less than [500] ms.

- How to ensure that the end-to-end latency in Broadcast message scenario is less than [500] ms

### 5.X.2	Identified Gaps

The following gaps have been identified to fulfil the identified requirements:

- Whether some procedures in 5G system can be used to limit the end-to-end messaging latency is needed to be studied. The procedures may be different in each message scenario.

- If new procedures are used, the potential impact on 5G system is needed to be studied.

- Specify some MSGin5G specific procedures to reduce the end-to-end messaging latency on top of 5GS if needed