3GPP TSG-SA WG6 Meeting #37-e meeting	S6-200744 DRAFT1

14th – 26th May 2020	(revision of S6-XXXXXX)

Source:	China Mobile, Convida Wireless LLC

Title:	Pseudo-CR on FS 5GMARCH Key Issue Message store and forward

Spec:	3GPP TR 23.700-24 v0.5.0

Agenda item:	8.6

Document for:	Approval

Contact:	Yue Liu (liuyueyjy@chinamobile.com)

- 1. Introduction

In 3GPP TS 22.262, the following requirement has been identified:

[R-5.1.2-005] The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available

This contribution proposes the key issue that MSGin5G Service shall fulfill this requirement in all Message scenarios. The MSGin5G Service requires the capability to store messages when a targeted UE is unavailable to receive the messages and then forward the messages to the UE when it becomes available.  However, the current MSGin5G Service is not able to support this feature.

- 2. Reason for Change

This Tdoc contribution with new text. It identifies a new technical issue that should be addressed, which is related to how the MSGin5G Service can be enabled to support store-and-forwarding of MSGin5G messages.

- 3. Conclusions

The delivery of messages by the MSGin5G Service may be optimized if the new technical issue is addressed.

- 4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-24 v0.5.0.

DRAFT 1 S6-200762 is merged to S6-200744 and Convida Wireless LLC is added as co-signer.

* * * First Change * * * *

# 5	Key Issues

## 5.X	Key Issue X: Message store and forward

### 5.X.1	Description

In many MSGin5G Service use cases, a targeted UE may not be immediately available to receive a message (e.g. UE may be disconnected or powered-off).  In such cases, the MSGin5G Service may not be able to deliver the message successfully to a targeted UE.

To satisfy these types of use cases, a requirement for  supporting Message store and forward capabilities is specified in [R-5.1.2-005] of 3GPP TS 22.262 [2] below:

[R-5.1.2-005]	The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available.

The requirement [R-5.1.2-005] is specified in the general clause so it will apply to all of the four message scenarios.

Store-and-forwarding of messages by the MSGin5G Service enables the storing (i.e. buffering) of messages when targeted UEs are unavailable and the forwarding of stored messages to targeted UEs once they become available. By supporting store-and-forwarding of messages, the MSGin5G Service can significantly improve the overall performance and efficiency of the system.

Issues include:

- How the MSGin5G Service can determine that theUE (including 5GMSGS UE, legacy-3GPP UE and non-3GPP UE) is unavailable e.g. based on the registration status, connection status or other conditions.

- How to determine that store and forward is needed to be used.

- What is the relationship between store and forward mechanism and other delivery mechanism, e.g. network triggering specified in [R-5.1.2-006] of 3GPP TS 22.262 [2].

- How to determine that the UE becomes available.

- If a message is stored by the MSGin5G Service, how long can the message be stored waiting for a UE to become available.

- What information is needed by the MSGin5G Service for managing the forwarding of messages.

5.X.2	Identified Gaps

The following gaps have been identified to fulfil the identified requirements:

- The capability for a MSGin5G Service to identify if a message is a candidate for store-and-forwarding based on the availability of the targeted UE and the information contained within the MSGin5G message (e.g. expiration time).

- The procedure to identify if the UE (including 5GMSGS UE, legacy-3GPP UE and non-3GPP UE) is available or unavailable to receive MSGin5G message needs to be studied.

- Study how to identify that store and forward mechanism needs to be used.

- The capability for an originator of a MSGin5G message to provide information to determine how long a message can be stored.

- The capability for an originator of a MSGin5G message to provide information to determine how the forwarding of messages should be managed.

- The capability for a MSGin5G Service to store and forward stored messages based on information provided by the originator.