3GPP TSG-SA WG6 Meeting #37-e	S6-200764_DRAFT2

E-meeting, 14th May – 26th May 2020

Source:	Convida Wireless LLC, Samsung, China Mobile

Title:	New Key Issue of Message Segmentation and Reassembly

Spec:	3GPP TR 23.700 v0.5.0

Agenda item:	8.6

Document for:	Approval

Contact:	Catalina Mladin, Convida, Mladin.Catalina@convidawireless.com

1. Introduction

In 3GPP TS 22.262, the following requirement has been identified:

[R-5.1.2-002] The MSGin5G Service shall support variable size of payload of text and data with a maximum of 2048 bytes and support segmented transmission if the content is larger than the maximum payload length of a message.

At the same time 3GPP TS 23.060 Annex C provides considerations on limiting packet sizes in order to “avoid packet fragmentation in the backbone network between the MS and the GGSN/PGW (and/or across the (S)Gi reference point) when some of the backbone links does not support packets larger than 1500 octets. Fragmentation within the backbone network creates a significant overhead. Therefore, operators might desire to avoid it”.

The mechanisms available in the 3GPP network for dealing with variable size payloads rely upon provisioning at the endpoints, during PDN connection establishment, the maximum packet size required to avoid segmentation in the backbone. Providing values smaller than necessary decrease the efficiency of the network.

For non-IP packet data network connections, the maximum packet size is provided as a part of the session management configuration by encoding it within the PCO. It is specified that maximum packet size of at least 128 octets is required “to provide a consistent environment for application developers”. This means that for using packet sizes larger than 128 octets, developers need to rely upon an underlying service or to implement segmentation in the application.

In 3GPP TS 23.502 shows that the AF is provided with the maximum packet size during NIDD configuration. Afterwards, for an MT NEF anchored data transport (clause 4.25.5), if NEF determines that delivery request does not conform with the NIDD configuration it skips the procedure and returns an error code in the response. This means that the expectation is that the adaptation to the maximum packet size network configuration is expected to be provided by the AF

Based on the studied usecases, the MSGin5G Service is required to support MIoT applications with the capability to segment messages (at the message originator side) and reassemble messages (at the message receiver side), in order to offload this requirement/functionality from the end applications.

2. Reason for Change

Support for segmentation and reassembly of messages is not provided in the access network given significant overhead for the network. Maximum packet sizes are configured in the access network per PDN connection. 3GPP TS 23.060 Annex C provides informative considerations for maximum packet size calculations based on packet size supported by the backbone, connection type (i.e. IP vs. non-IP) etc. SMS payload size is 140 octets. Therefore, applications using a variety of non-IP transports (e.g. NIDD, SMS, etc.) must perform over-the-top segmentation and reassembly if the size of their message payloads exceed the packet size limits.  By supporting segmentation and reassembly functionality within the MSGin5G Service, the burden and overhead of performing over-the-top segmentation and reassembly can be offloaded from applications.

3. Conclusions

The MSGin5G Service can be enhanced to provide additional value-add functionality to applications if segmentation and reassembly of 5GMSG messages is supported.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700 v0.5.0.

DRAFT 1 merges S6-200764 with S6-200743 and adds China Mobile as co-signers

DRAFT 2:

- The description section has been enhanced with an additional requirement.

- In the identified gaps, items for study are identified first, and the issue of support of fixes size segmentation has been added. The other items for study are slightly re-worded for clarity

- Reference for 23.060 and indication of 3GPP TS 22.262 as [2] have been added. A new change for clause 2 was introduced for this purpose.

* * * First Change * * * *

## 5.X	Key Issue X: Message Segmentation and Reassembly

### 5.X.1	Description

Support for segmentation and reassembly of messages is not provided in the access network given significant overhead for the network. Maximum packet sizes are configured in the access network per PDN connection. 3GPP TS 23.060 [15] Annex C provides informative considerations for maximum packet size calculations based on packet size supported by the backbone, connection type (i.e. IP vs. non-IP) etc. SMS payload size is 140 octets. Therefore, applications using a variety of non-IP transports (e.g. NIDD, SMS, etc.) must perform over-the-top segmentation and reassembly if the size of their message payload exceeds the packet size limits.  By supporting segmentation and reassembly functionality within the MSGin5G Service, the burden and overhead of performing over-the-top segmentation and reassembly can be offloaded from applications.

3GPP TS 22.262[2] defines the following requirement in the general clause, therefore being applicable to all of the four message scenarios:

[R-5.1.2-002] The MSGin5G Service shall support variable size of payload of text and data with a maximum of 2048 bytes and support segmented transmission if the content is larger than the maximum payload length of a message.

3GPP TS 22.262[2] also defines the following requirement in the interworking clause, therefore being applicable to the SMS scenario:

### [R-10.2-002] The MSGin5G Service shall be able to translate one message into multiple SMS messages when the length of a message of MSGin5G Service is large than the maximum length of a SMS message.

### 5.X.2	Identified Gaps

The following aspects have been identified for study:

- Weather performing segmentation using a fixed-length MTU size (e.g. 2048 octets) fulfils the segmentation service requirements.

- Whether any feature or procedure from the underlying network are needed and any potential impact to on 5G system of such features or procedures.

- Whether there are differences in handling between the four message scenarios.

- The relationship between message segmentation and other service requirements, e.g. store and forward (specified in [R-5.1.2-005] of 3GPP TS 22.262 [2]) and delivery status (specified in requirement [R-5.1.2-004] of 3GPP TS 22.262 [2])

For potential support of segmentation based on variable MTU size dependent on the access network, the following capability gaps have been identified:

- The capability for the MSGin5G Service to detect when the size of a MSGin5G message exceeds the maximum packet size of the access network transport.

- The capability for the MSGin5G Service to segment and deliver a MSGin5G message that exceeds the maximum packet size of the access network transport into a set of segmented messages that individually do not exceed the maximum packet size of the access network transport.

- The capability for the MSGin5G Service to reassemble a set of segmented messages received via individual access network packets into a single MSGin5G message.

- The capability for the MSGin5G Service to indicate successful reception of all the segmented messages.

- The capability for the MSGin5G Service to retransmit the segmented messages that are lost.

* * * Next Change * * * *

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.262:	"Message Service within the 5G System "

[3]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications"

[4]	GSMA PRD RCC.07: "RCC.07 Rich Communication Suite 9.0 Advanced Communications Services and Client Specification v10.0"

[5]	OMA OMA-ERELD-LightweightM2M-V1_1-20180612-C: "Enabler Release Definition for LightweightM2M"

[6]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)"

[7]	3GPP TS 23.038: "Alphabets and language-specific information"

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); protocol specification"

[9]	3GPP TS 23.434:	"Service Enabler Architecture Layer for Verticals"

[10]	3GPP TS 23.502:	"Procedures for the 5G System"

[11]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)"

[12]	ATIS-0700008.v002: "Cell Broadcast Entity (CBE)-to-Cell Broadcast Center (CBC) Interface Specification"

[13]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2"

[14]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2"

[15]	3GPP TS 23.060: " General Packet Radio Service (GPRS); Service description; Stage 2"