3GPP TSG-SA WG6 Meeting #37-e	S6-200765 DRAFT1

E-meeting, 14th May – 26th May 2020

Source:	Convida Wireless LLC, Samsung

Title:	Solution to the Key Issue of Message Segmentation and Reassembly

Spec:	3GPP TR 23.700 v0.5.0

Agenda item:	8.6

Document for:	Approval

Contact:	Catalina Mladin, Convida, Mladin.Catalina@convidawireless.com

1. Introduction

In 3GPP TS 22.262, the following requirement has been identified:

[R-5.1.2-002] The MSGin5G Service shall support variable size of payload of text and data with a maximum of 2048 bytes and support segmented transmission if the content is larger than the maximum payload length of a message.

Accordingly, a new KI identifying the need to enhance the MSGin5G Service to support MSGin5G message segmentation and reassembly has been proposed.

2. Reason for Change

Support for segmentation and reassembly of messages is not provided in the access network given significant overhead for the network. Maximum packet sizes are configured in the access network per PDN connection. 3GPP TS 23.060 Annex C provides informative considerations for maximum packet size calculations based on packet size supported by the backbone, connection type (i.e. IP vs. non-IP) etc. SMS payload size is 140 octets. .  Therefore, applications using a variety of transports (e.g. NIDD, SMS, etc.) must perform over-the-top segmentation and reassembly if the size of their message payloads exceed the packet size limits.  By supporting segmentation and reassembly functionality within the MSGin5G Service, the burden and overhead of performing over-the-top segmentation and reassembly can be offloaded from applications.

This solution supports MSGin5G message segmentation and reassembly for both MT and MO MSGin5G use cases.

3. Conclusions

The MSGin5G Service can be enhanced to provide additional value-add functionality to applications if segmentation and reassembly of 5GMSG messages is supported.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700 v0.5.0. providing a solution for 5GMSG segmentation and reassembly support.

DRAFT 1 contains changes to address comments received and adds Samsung as cosigner

* * * First Change * * * *

## 6.X	Solution X: MSGin5G Message Segmentation and Reassembly

### 6.X.1	Description

#### 6.X.1.1	General

This solution addresses the identified gaps under Key Issue #X by introducing MSGin5G message segmentation and reassembly functionality to the MSGin5G Service.

This solution supports MSGin5G message segmentation and reassembly for both MT and MO MSGin5G use cases. Depending on the use case, MSGin5G message segmentation and reassembly operations are performed either by the MSGin5G Server or the MSGin5G Client. For MT use case, the MSGin5G Server performs MSGin5G message segmentation while the MSGin5G Client performs MSGin5G message reassembly. For MO use case, the MSGin5G Client performs MSGin5G message segmentation while the MSGin5G Server performs MSGin5G message reassembly.

#### 6.X.1.2	Application-to-point MSGin5G Message Segmentation and Reassembly

Figure 6.X.1.2-1 shows the MSGin5G message segmentation and reassembly procedure for MT MSGin5G message use cases (e.g. AOMT).

Pre-conditions:

- A UE hosts a MSGin5G Client and an Application Client.

- The MSGin5G Client registers with the MSGin5G Server.

- An Application Server needs to deliver application data to the Application Client on the UE and the size of the application data exceeds the maximum packet size supported by the UE’s access network transport.

- Figure 6.X.1.2-1: Application-to-point MSGin5G Message Segmentation and Reassembly

- An Application Server sends a message to a MSGin5G Server that targets an Application Client on a UE and that has a size that exceeds the maximum allowed packet size of the access network transport supported by the targeted UE.

- The MSGin5G Server compares the size of the received message to the maximum allowed packet size of the access network transport of the targeted UE and detects that the size exceeds the limit. As a result, the MSGin5G Sever segments the received message into a set of segmented messages such that each segmented message can fit within an individual access network transport packet.  Within each segmented message, the information elements defined in Table 6.X.1.2-1 are included to enable the MSGin5G Client on the targeted UE to reassemble the segmented messages.

- Table 6.X.1.2-1: Segmentation and Reassembly Information Elements

| Information element | Status | Description |
| --- | --- | --- |
| Segmentation Set Identifier | M | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. |
| Segment Sequence Number | M | An incrementing counter that indicates the sequence number of each segmented message within a set of segmented messages. |
| Last Segment Flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. (See NOTE 1) |
| Total Segments | O | An integer indicating total number of segmented messages associated the Segmentation Set Identifier. (See NOTE 2) |
| NOTE 1: The Last Segment Flag needs to be include only in the last segment of the message. NOTE 2: The Total Segments needs to be included only in the first segment of the message. |  |  |

- The MSGin5G Server sends each segmented message to the targeted UE within an individual access network transport packet. The target UE checks for the Last Segment Flag in each of the received segment before proceeding to Step 4. If the next segment is not received within the expected time (based on configuration) then proceed to step 4.

- If MSGin5G Client has received segment with Last Segment Flag indication, the MSGin5G Client reassembles all the segmented messages into a single MSGin5G message based on the information elements defined in Table 6.X.1.2-1.

Editor's Note:	Recovery mechanism for missing segments in application-to-point message and delivery confirmation is FFS.

- The MSGin5G Client delivers the contents of the MSGin5G message to the targeted Application Client.

#### 6.X.1.3	Point-to-application MSGin5G Message Segmentation and Reassembly

Figure 6.X.1.3-1 shows the MSGin5G message segmentation and reassembly procedure for MO MSGin5G message use cases (e.g. MOAT).

Pre-conditions:

- A UE hosts a MSGin5G Client and an Application Client.

- The MSGin5G Client registers with the MSGin5G Server.

- An Application Client on the UE needs to deliver application data to an Application Server and the size of the application data exceeds the maximum packet size supported by the UE’s access network transport.

- Figure 6.X.1.3-1: Point-to-application MSGin5G Message Segmentation and Reassembly

- An Application Client on a UE sends a message to a MSGin5G Client that targets an Application Server and that has a size that exceeds the maximum allowed packet size of the access network transport supported by the UE.

- The MSGin5G Client compares the size of the received message to the maximum allowed packet size of the access network transport supported by the UE and detects that the size exceeds the limit. As a result, the MSGin5G Client segments the received message into a set of segmented messages such that each segmented message can fit within an individual access network transport packet.  Within each segmented message, the information elements defined in Table 6.X.1.2-1 are included to enable the MSGin5G Server to reassemble the segmented messages.

- The MSGin5G Client sends each segmented message to the MSGin5G Server within an individual access network transport packet. The target UE checks for the Last Segment Flag in each of the received segment before proceeding to Step 4. If the next segment is not received within the expected time (based on configuration) then proceed to step 4.

- If MSGin5G Client has received segment with Last Segment Flag indication, the MSGin5G Server reassembles all the segmented messages into a single MSGin5G message based on the information elements defined in Table 6.X.1.2-1.

Editor's Note:	Recovery mechanism for missing segments in point-to-application message and delivery confirmation is FFS.

- The MSGin5G Server delivers the contents of the MSGin5G message to the targeted Application Server.

Editor's Note:	MSGin5G Message Segmentation and Reassembly for point-to-point, group and broadcast messages is FFS.

### 6.X.2	Impacts on existing nodes and functionality

This solution enables the MSGin5G Service to support message segmentation and reassembly capability. This functionality can be layered over top of existing access network transports that do not support segmentation and reassembly. Therefore, the impact on existing nodes and functionality is minimal.

### 6.X.3	Solution evaluation

This solution enables the MSGin5G Service to support segmentation and reassembly of MSGin5G messages when the size of a MSGin5G exceeds the maximum packet size supported by an access network transport. By supporting segmentation and reassembly, the MSGin5G Service can improve the overall performance and efficiency of the system.

The solution is applicable for use cases scenarios that support a MSGin5G Client and MSGin5G Server in the communication path.  For MSGin5G use case scenarios that do not include a MSGin5G Client (e.g. legacy UE), this solution is not applicable.