3GPP TSG-SA WG6 Meeting #37-e	S6-200757

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics, Intel

Title:	Pseudo-CR on Service Areas

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This pCR introduces the concept of service areas under the 'commonly used values' clause.

2. Reason for Change

ECSPs and ASPs may allow access to Edge Computing service from specific areas i.e. allowing only the UEs within that area to access functional entities resident in the Edge Data Network. This area is called service area and allows for better management of the services provided by the ECSPs and ASPs.

The contribution proposes inclusion of following service areas under 'commonly used values' clause:

- EDN service area:

  - enables ECSPs to deploy EDNs with restricted access. This restriction may be for practical purposes, such as access being limited to UEs in a stadium or a premises such as office or university.

  - To deploy EDNs with restricted access, LADN is used. In a deployment using local DNs other than LADNs, the EDN is globally accessible.

  - When EDN is deployed using LADN, the EDN service area is same as the LADN service area.

  - All Edge Enabler Servers and Edge Application Servers resident in the EDN shall have service area same as or a subarea of the EDN service area.

  - If the UE knows the EDN service area, the UE shall not attempt to establish a PDU session for the EDN while the UE is outside the EDN service area.

- EES service area:

  - enables ECSPs to define areas within EDN service area, where Edge Computing services are available.

  - It is an optional characteristic of the Edge Enabler Server, and if not set by the ECSP, the EES service area is same as the service area of the EDN it resides in.

  - The service area of an Edge Enabler Server is limited by the service area of the EDN it resides in i.e. EES service area is either same or a subarea of the EDN service area.

  - Any Edge Application Server resident in the EDN shall register on the Edge Enabler Server if there is an EAS service area overlap with the EES service area.

  - The EES service area allows the ECS to select the appropriate EES(s) at the time of EEC provisioning, using UE's location or DNAI.

  - The EES service area allows for better management of EAS information on the EES. For e.g., for a globally accessible EDN, country (or possibly a more granular region like a stadium or a shopping mall) specific Edge Enabler Servers can be deployed by the ECSP. This allows for better management of country (or region) specific Edge Application Servers' information and local policies.

- EAS service area

  - enables ASPs to define area within EDN service area, where Edge Application Server's service is available.

  - Edge Application Servers resident in an EDN shall have service area same as or a subarea of the EDN service area.

  - Edge Application Server shall register on Edge Enabler Server(s) of the EDN with which there is a service area overlap.

  - The EAS service area allows for service planning as per ASP needs and better management of EAS load and KPIs within an area.

Why EES service area is required:

- It is reasonable to expect that the ECSPs will want to restrict access to the data centres based on where they are deployed and their capacity.

- for e.g. a data centre deployed in USA will not ideally qualify as edge for a UE in India; or

- a small processing unit deployed on a base station will be specific to the base station’s serving area; or

- a fairly small data centre within a shopping mall will be meant for the usage within the shopping mall.

EES service area can be used by the ECSP/MNO to design this deployment based on the capacity and location of the individual data centres.

- One EDN may provide access to multiple data centres each having different capacities, and possibly from multiple ECSPs (Consider deployment model 1, EDN is shared with internet DN).

- But, having the EES service area equivalent to EDN service area would mean that all the data centres have the same service area irrespective of their capacity, deployments and providers – this is not practical.

- LADNs can be used for data centres with requirements for restricted service area.

- But, it will either mean that all the EDNs with restricted area requirements have their own LADN EDN configurations (a management nightmare); or

- All the data centres accessible via the LADN EDN have service area same as the LADN EDN.

Both of these options are not practical.

- An ECSP may want to deploy a mix of restricted and unrestricted data centres. Having the access same as EDN service area will not be feasible; unless multiple EDN configurations are used; which is a management nightmare when multiple data centres and MNOs are involved.

- Capacity can be a dynamic property of the data centre, as capacity can be increased or decreased by the ECSP in a planned manner or servers can crash with the data centres. If the EDN service area is used to indicate the area the data centre server, it’d mean updating the configuration information with all the MNOs every time there is a change in capacity.

- As there is no restriction on number of EES within a data centre or within an EDN, EES can be employed with flexibility. Like for a globally accessible EDN, one EES can be employed for each country; with EES service area used for EES selection. This will also help in management of information based on the locality. Something similar to country level top level domains.

Please note that EES service area is in no way restricting the access to the EDN (or data centres within the EDNs) unless planned by the ECSP.

If an ECSP wants a globally accessible data centre, it can either use one EES with no service area restrictions, or multiple EES each serving one country – purely an ECSP choice for managing its offerings.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.2.0

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

AF	Application Function

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

EHE	Edge Hosting Environment

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

LADN	Local Area Data Network

NEF	Network Exposure Function

SCEF	Service Capability Exposure Function

SSID	Service Set Identifier

TAI	Tracking Area Identity

* * * Next Change * * * *

### 7.3.x	Service areas

#### 7.3.x.1		General

ECSPs and ASPs may allow access to Edge Computing service from specific areas i.e. allowing only the UEs within that area to access functional entities resident in the Edge Data Network. This area is called service area.

#### 7.3.x.2	Edge Data Network service area

Edge Data Network service area enables the ECSPs to deploy EDNs with access restricted to specific areas.

To deploy EDNs with restricted access, LADN is used as described in Annex A.2.3 of this document. In a deployment using local DNs other than LADNs, the EDN is globally accessible. When EDN is deployed using LADN, the EDN service area is same as the LADN service area and rules specified for LADN apply to the UE, as specified in 3GPP TS 23.501 [2].

In a deployment using local DNs other than LADNs, the EDN is globally accessible.

7.3.x.3	Edge Enabler Server service area

Edge Enabler Server service area enables the ECSPs to define areas within the EDN service area, where Edge Computing services are available.

It is an optional characteristic of the Edge Enabler Server, and if not set the EES service area is same as the service area of the EDN it resides in.

The service area of an Edge Enabler Server is limited by the service area of the EDN it resides in i.e. EES service area is either same as the EDN service area or a subarea of the EDN service area it resides in.

#### 7.3.x.4	Edge Application Server service area

Edge Application Server service area enables the ASPs to define an area where Edge Application Server's service is available.

The service area of an Edge Application Server is limited by the service area of the EDN it resides in i.e. EAS service area is either same as the EDN service area or a subarea of the EDN service area it resides in.

* * * Next Change * * * *

### 8.2.6	Edge Enabler Server Profile

The Edge Enabler Server profile includes information about the EES and the services it provides.

Table 8.2.6-1: Edge Enabler Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| Edge Application Server Profiles | M | List of Edge Application Servers registered with the EES. |
| EES Service Area | O | Area that the EES serves. EEC of the UEs within this service area can connect with this Edge Enabler Server. |
| EES Provider Identifier | O | The identifier of the EES Provider (such as ECSP) |

Editor's Note: The subset of EAS information provided by the EES is FFS.

* * * Next Change * * * *

## A.2.3	Option 3. Use of LADN

Edge computing services can be provided via Edge-dedicated Data Networks deployed as LADNs. The PLMN supports edge computing services in the EDN service areas which can be identified by the service area corresponding to the respective LADN DNNs. The LADN service area is the service area that the Edge Computing is supported. Each individual EAS in the LADN may support the same or smaller service area than the LADN.

Figure A.2.3-1: Option 3: Use of LADN(s)