3GPP TSG-SA WG6 Meeting #38-e	S6-201054

e-meeting, 20th – 31st July 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	Target EAS discovery with target DNAI

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

<Introduction part >

2. Reason for Change

This pCR introduces target DNAI provided from the 3GPP network within target EAS discovery request of the source EAS, so that the source EAS can discover the target EAS corresponding to the UP path change identified by the target DNAI. The proposed change resolves the following EN :

Editor's Note:	Whether the information in the discovery is sufficient to discover a proper EAS is FFS.

The target DNAI information can be provided to the source EES by the source EAS or the 3GPP network. The source EES can use the target DNAI to obtain information for the target EES and the target EAS, which is aligned with the UP path to be configured/activated by the 3GPP network.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558.

* * * First Change * * * *

### 8.8.2	Procedures

#### 8.8.2.1	Discover target Edge Application Server

Figure 8.8.2.1-1 illustrates the interactions between the source Edge Application Server and the source Edge Enabler Server and optionally between the source Edge Enabler Server and the Edge Configuration Server and between the source Edge Enabler Server and target Edge Enabler Server for fetching target Edge Application Server information. This procedure may be utilized by a source Edge Application Server which undertakes the transfer of application context information to a target Edge Application Server directly.

Pre-conditions:

1.	Information related to the Edge Enabler Server is available with the source Edge Application Server.

Figure 8.8.2.1-1: Discover target Edge Application Server

1.	The Edge Application Server invokes the Edge Application Server Discovery request on the source Edge Enabler Server. If target DNAI is available at the source Edge Application Server via UP path management event notification, the source Edge Application Server provides the source Edge Enabler Server with the target DNAI.

NOTE:	The trigger condition to invoke the Discover target Edge Application Server API is up to application service logic, which is out of scope of this specification.

2.	Upon receiving the request, the source Edge Enabler Server checks if the requesting Edge Application Server is authorized to perform the discovery operation. If it is authorized, the source Edge Enabler Server checks if there exists a target Edge Application Server information (registered or cached) that can satisfy the requesting Edge Application Server information and additional query filters. The target EAS is different from the S-EAS is the Discovery Reason indicates discovering a new EAS. If the source Edge Enable Server discovers the target Edge Application Server(s), the flow continues with step 5, else the source Edge Enabler Server retrieves the target Edge Enabler Server address from the Edge Configuration Server as specified in clause 8.x.2.bb.

Editor's Note: Procedure referenced in the above step 2, as clause 8.x.2.bb, is FFS.

3.	The source Edge Enabler Server invokes the Edge Application Server Discovery request on the target Edge Enabler Server.

4.	The target Edge Enabler Server discovers the target Edge Application Server(s) and responds with the discovered target Edge Application Server information to the source Edge Enabler Server. The source Edge Enabler Server may cache the target Edge Application Server information.

5.	The source Edge Enabler Server responds to the Edge Application Server with the target Edge Application Server Information.

### 8.8.3	Information flows

#### 8.8.3.1	Edge Application Server discovery request

Table 8.8.3.1-1 describes information elements for the Edge Application Server Discovery Request from the EAS to the Edge Enabler Server. Table 8.8.3.1-2 provides further detail about the Query Filter information element.

Table 8.8.3.1-1: Edge Application Server discovery request

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | EAS ID of the source EAS to discover the target EAS information that matches with the source EAS. |
| Application Service Provider ID | O | The ASP identifier. |
| UE identifier | O | The identifier of the UE (i.e. GPSI or identity token). |
| UE location | O | Location of the UE (e.g. geographical area, civic address). |
| Discovery Reason | O | Discovery reason (e.g. maintenance, UP path change). |
| Target DNAI | O | Target DNAI information. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Query Filter | O | List of characteristics for discovery purposes, as detailed in Table 8.8.3.1-2. |

Editor's Note:	The relationship of DNAI associated with EAS and the DNAI(s) associated with EDN is FFS.

Table 8.8.3.1-2: Query Filter

| Information element | Status | Description |
| --- | --- | --- |
| EAS service category | O | The EAS service category e.g. gaming. |
| Service permission level(s) | O | Desired level of service permissions e.g. trial, gold-class |
| Desired service feature(s) | O | Desired service features e.g. single vs. multi-player gaming service |
| Location availability | O | The desired location(s) (e.g. geographical area, route) where the service should be available. |
| EAS Profile | O | Profile of the consumer EAS, as described in Table 8.2.4-1. |

Editor's Note:	The logic on how the query filters are applied in conjunction with the EAS Profiles is to be clarified.

Editor's Note:	If and how complex Information Elements may be broken down into multiple elements is FFS