3GPP TSG-SA WG6 Meeting #37e	S6-201069r01

E-meeting, 14 – 26 May 2020	(revision of S6-xxxxxx)

Source:	Samsung

Title:	EAS selection

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Jicheol Lee <jicheol.lee at samsung.com>

1. Introduction

Based on the agreement on the cardinality of entities, it is decided as follows:

NOTE:	EAS(s) belonging to the same EAS ID can be provided by multiple ECSP(s) in an EDN.

In addition, since one EEC may communication with one or mote ECS(s), as well as one or more EES(s) concurrently, there should be a mechanim for an EEC to select an EAS where multiple EASs are provided.

During SA6#37e meeting, the following editor’s note has been captured for further considerations:

Editor’s Note: It is FFS how to select an EAS when the EEC discovered multiple EAS(es) from multiple ECSPs.

This paper discusses the two cases based on which entity to select an appropriate EAS as follows:

- AC decision

For the case scenario where Application Service Provider deployed multiple EASes, it makes sense that the AC chooses an appropriate EAS. It is only valid for the case of edge-aware AC.

- EEC decision

EEC can take a decision to select an appropriate EAS either by its local configuration or by MNO’s guideline. This can be applied for both edge-aware or edge un-aware application client.

For EEC decision case, the information for EEC to select EAS needs to be specified. Example information is as follows:

- Precedence per EES as described (new information of Provioning Response of this proposal)

- Discovered EAS Service KPI (that includes as described in the existing Table 8.2.5-1)

In addition, if the MNO ECS wants to differentiate the guideline how the EEC selects an appropriate EAS, it is required to provide a standard mechanism for ECS to provide the EAS selection criteria to the EEC. It is proposed to add EAS selection criteria in Provisioning Response in this paper.

2. Reason for Change

Based on the agreement on the cardinality, multiple EAS for one AC can be possible. However, there is no mechanism for EEC or ECS to select one EAS when multiple EAS for one AC are available.

3. Conclusions

It is proposed to add description for an EAS selection mechanisms based on EAS selection criteria

4. Proposal

It is proposed to agree the following changes:

* * * First Change (8.5.2: Edge Application Server Discovery) * * * *

### 8.5.2	Edge Application Server Discovery and Selection

#### 8.5.2.1	General

Edge Application Server discovery enables Edge Enabler Clients to obtain information about available Edge Application Servers of interest. The identification of the Edge Application Servers is based on matching query filters or Application Client Profiles provided in the request.

When more than one EAS is discovered for serving a specific AC, the EEC may select one EAS according to a set of criteria based on relevant information returned in the EAS Discovery process, if available.

NOTE: It is out of scope of the present specification how the EEC provides the EAS information to the AC and how AC selects one EAS when multiple EASs are provided.

When more than one EASs are discovered, the EEC may use the following information to select an appropriate EAS:

- Precedence per EES as described (based on the order of the EASes in the Edge Application Server discovery response (as described in clause 8.5.2.3.2)

- Discovered EAS Profile (as described in clause 8.2.4)

* * * End of Change * * * *