3GPP TSG-SA WG6 Meeting #37e	S6-201069r03

E-meeting, 14 – 26 May 2020	(revision of S6-xxxxxx)

Source:	Samsung

Title:	EAS selection

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Jicheol Lee <jicheol.lee at samsung.com>

1. Introduction

Based on the agreement on the cardinality of entities, it is decided as follows:

NOTE:	EAS(s) belonging to the same EAS ID can be provided by multiple ECSP(s) in an EDN.

In addition, since one EEC may communication with one or mote ECS(s), as well as one or more EES(s) concurrently, there should be a mechanim for an EEC to select an EAS where multiple EASs are provided.

During SA6#37e meeting, the following editor’s note has been captured for further considerations:

Editor’s Note: It is FFS how to select an EAS when the EEC discovered multiple EAS(es) from multiple ECSPs.

This paper proposed incoperated text based on the e-mail discussion.

2. Reason for Change

Based on the agreement on the cardinality, multiple EAS for one AC can be possible. However, there is no mechanism for EEC or ECS to select one EAS when multiple EAS for one AC are available.

3. Conclusions

It is proposed to add description for an EAS selection mechanisms based on EAS selection criteria

4. Proposal

It is proposed to agree the following changes:

* * * First Change (8.5.2: Edge Application Server Discovery) * * * *

### 8.5.2	Edge Application Server Discovery and Selection

#### 8.5.2.1	General

Edge Application Server discovery enables Edge Enabler Clients to obtain information about available Edge Application Servers of interest. The identification of the Edge Application Servers is based on matching query filters or Application Client Profiles provided in the request.

When more than one EAS are discovered for a specific AC, the EEC may select one EAS according to a set of criteria based on the EAS selection criteria and/or relevant information returned in the EAS Discovery process, if available.

NOTE: It is out of scope of the present specification how the EEC provides the EAS information to the AC

When more than one EASs for a specific AC are discovered, the EEC may use the following information to select an appropriate EAS:

- Discovered EAS Profile (as described in clause 8.2.4)

Editor’s Note: It is FFS whether the precedence of EAS, EES and ECSP is used for EAS selection. If needed, it is also FFS how it is presented (e.g. ordered list or adding precedence field)

Editor’s Note: It is FFS whether EAS selection criteria can be provided by MNO or ECSP to guide the EEC or is UE implementation.

Editor’s Note: The details of EAS selection criteria are FFS. Those can be prefered EES first, low latency first, or high computing first, desire to prefer EASs from the same EDN to reduce the number of connections to multiple EESs, EDN’s service area, etc.

* * * End of Change * * * *