3GPP TSG-SA WG6 Meeting #38-e	S6-201115

e-meeting, 20th – 31st July  2020	(revision of S6-20xxxx)

Source:	Samsung Electronics, Intel, AT&T, Convida, Qualcomm, ETRI, UANGEL

Title:	Pseudo-CR on Service provisioning

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

The contribution is to clean up service provisioning procedure and introduce subscribe notify model.

2. Reason for Change

The procedures in 23.558 v030 are limited to a request response model. However, as illustrated by clause 8.3.1 in the service provisioning flow, there is a need for a subscription based provisioning. This contribution introduces the subscription based procedures along with a clean-up of request response procedures and the information flows.

Changes in the request/response procedures include:

- references to clause 8.9.2 for usage of 3GPP core network capabilities such as UE's location by the ECS.

- alignment of the example information that can be included in the service provisioning request and service provisioning responses with the information flows specified in clause 8.3.3.

Changes in the information flows include:

- Service provisioning request:

> ’connectivity filter' IE is renamed to 'connectivity information' as it represents information about the UE connectivity with the network.

> 'ECSP information' IE is included for the EEC to request filtered EES information specific to an ECSP.

> 'ECSP information' IE and 'Application Client Profile(s)' IE are organized under a new IE called 'Provisioning filters'.

- Service provisioning response:

> Name of 'EES configuration information' is corrected to reflect 'EDN configuration information', and it has been made a list, as multiple EDNs may be available for the UE, concurrently.

> 'List of EDN configuration information' IE is restructured in 2 categories:

>> 'EDN connection information', including the 'DNN/APN' IE, 'S-NSSAI' IE and 'EDN service area IE' (only for LADNs).

>> 'List of Edge Enabler Servers' IE, including the 'EES Endpoint' IE, 'ECSP info' IE and a 'List of EAS IDs' IE.

Following new procedures and related information flows are included:

- Service provisioning subscription/notification

- Service provisioning subscription update

- Service provisioning unsubscribe

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.3.0.

* * * First Change * * * *

## 8.3	Service provisioning

### 8.3.1	General

Service provisioning allows configuring the Edge Enabler Client with information about available Edge Computing services, based on the hosting UE's location, service requirements, service preferences and connectivity. This configuration includes necessary address information for the Edge Enabler Client to establish connection with the Edge Enabler Server(s).

If the ECS is deployed by MNO contracted with one or more ECSP(s) with its(their) own EES(s), the ECS provides one or more ECSP EES configuration information via MNO ECS as describes in clause 8.3.3.2.

If the ECS is deployed by non-MNO ECSP, the ECS endpoint address is pre-configured with the EEC. The EEC that is configured with multiple ECSP's ECS endpoint address(es) may perform the service provisioning procedure per the ECS of each ECSP multiple times.

Figure 8.3.1-1 illustrates an overview of service provisioning. Service provisioning procedures support the following models:

-	Request/Response model; and

-	Subscribe/Notify model.

Figure 8.3.1-1: Overview of Service provisioning

The UE is initially provisioned with the configurations required to connect to the Edge Data Network. Once provisioned, the Edge Enabler Client of the UE registers with the selected Edge Enabler Server(s) from the list of provisioned Edge Enabler Server(s) it received from the ECS(s). The UE further consumes the edge computing services and performs various operations such as Edge Application Server discovery, Edge application communications, Application context relocation, etc. While the UE is consuming the edge computing services, there may be several triggers which may cause service provisioning to be triggered by the UE or by the Edge Configuration Server.

The triggers for service provisioning are classified as:

a.	Triggers at UE - Some examples are:

-	Application Client related updates available at the Edge Enabler Client due to AC installation/re-installation, Application Client requesting application server access (e.g. via internet browser)

-	Edge Enabler Client supporting one or more Application Clients may be updated due to Edge Enabler Client re-installation.

b.	Triggers at Edge Configuration Server – An example is:

-	Edge Enabler Server updates due to Edge Application Server installation/re-installation/re-location, EDN/DNAI changes of the UE.

NOTE:	The DNAI changes can only be detected at the Edge Configuration Server if the Core Network type is 5GC.

### 8.3.2	Procedures

#### 8.3.2.1	General

Following procedures are supported for service provisioning:

-	Request-response procedure;

-	Subscribe-notify procedures, including:

-	Subscription update procedure; and

-	Unsubscribe procedure.

#### 8.3.2.2	Request-response

Figure 8.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The Edge Enabler Client has been pre-configured or has discovered the address (e.g. URI) of the Edge Configuration Server.

2.	The Edge Enabler Client has been authorized to communicate with the Edge Configuration Server. The UE Identifier is either preconfigured or resulted from a successful authorization.

Editor's Note:	The authorization procedure required to enable communication with the Edge Configuration Server is FFS.

Figure 8.3.2.2-1: Service provisioning – Request/Response

1.	The Edge Enabler Client sends a service provisioning request to the Edge Configuration Server. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information and provisioning filters.

2.	Upon receiving the request, the Edge Configuration Server performs an authorization check to verify whether the Edge Enabler Client has authorization to perform the operation. If required, the ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.2. The Edge Configuration Server determines the service provisioning information such as EDN connection information and list of Edge Enabler Servers based on UE's location and the provisioning filters (as per the request in step 1).

3.	The Edge Configuration Server responds to the Edge Enabler Client's request with a service provisioning response which includes a list of Edge Data Network configuration information, e.g. identification of the Edge Data Network, service area information (for LADN), and the required information (e.g. URI, IP address) for establishing a connection to the Edge Enabler Server.

If the Edge Data Network configuration information includes an LADN DNN as an identifier for the Edge Data Network, the Edge Enabler Client considers the LADN as the Edge Data Network. Therefore, the service area of Edge Data Network is the LADN Service Area, which can be discovered using the UE Registration Procedure. The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the Edge Enabler Client may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

#### 8.3.2.3	Subscribe-notify

##### 8.3.2.3.1	General

Clause 8.3.2.3.2 & 8.3.2.3.3 together illustrate the service provisioning procedure based on Subscribe/Notify model.

Clause 8.3.2.3.4 illustrates the service provisioning update procedure.

Clause 8.3.2.3.5 illustrates the service provisioning unsubscribe procedure.

##### 8.3.2.3.2	Subscribe

Figure 8.3.2.3.2-1 illustrates the service provisioning request procedure between the Edge Enabler Client and the Edge Configuration Server.

Pre-conditions:

1.	The Edge Enabler Client has been pre-configured or has discovered the address (e.g. URI) of the Edge Configuration Server.

2.	The Edge Enabler Client has been authorized to communicate with the Edge Configuration Server. The UE Identifier is either preconfigured or resulted from a successful authorization.

Figure 8.3.2.3.2-1: Service provisioning subscription

1.	The Edge Enabler Client sends a service provisioning subscription request to the Edge Configuration Server. The service provisioning subscription request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, proposed expiration time and provisioning filters.

2.	Upon receiving the request, the Edge Configuration Server performs an authorization check to verify whether the Edge Enabler Client has authorization to perform the operation. If required, the ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.2. If the request is authorized, the Edge Configuration Server creates the subscription for provisioning.

3.	The Edge Configuration Server responds with a service provisioning subscription response, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the Edge Enabler Client shall send a Service provisioning subscription update request prior to the expiration time. If a Service provisioning subscription update request is not received prior to the expiration time, the Edge Configuration Server shall treat the Edge Enabler Client as implicitly unsubscribed.

##### 8.3.2.3.3	Notify

Figure 8.3.2.3.3-1 illustrates the service provisioning notification procedure between the Edge Enabler Client and the Edge Configuration Server.

Pre-conditions:

1.	The Edge Enabler Client has subscribed with the Edge Configuration Server for the provisioning information as specified in clause 8.3.2.3.2.

Figure 8.3.2.3.3-1: Service provisioning notification

1.	An event occurs at the Edge Configuration Server that satisfies trigger conditions for updating service provisioning of a subscribed Edge Enabler Client. The Edge Configuration Server determines the provisioning information it needs to send in the Service provisioning notification. The Service provisioning notification may include EDN connection information and list of Edge Enabler Servers based on UE's location and the provisioning filters received in service provisioning subscription request of the EEC.

2.	The Edge Configuration Server sends a provisioning notification to the Edge Enabler Client with the list of EDN configuration information determined in step 1.

If the EDN configuration information in the service provisioning notification includes an LADN DNN as an identifier for the EDN, the Edge Enabler Client considers the LADN as the Edge Data Network. Therefore, the service area of Edge Data Network is the LADN Service Area, which can be discovered using the UE Registration Procedure.

##### 8.3.2.3.4	Subscription update

Figure 8.3.2.3.4-1 illustrates the service provisioning subscription update procedure between the Edge Enabler Client and the Edge Configuration Server.

Pre-conditions:

1.	The Edge Enabler Client has subscribed with the Edge Configuration Server for the provisioning information as specified in clause 8.3.2.3.2.

Figure 8.3.2.3.4-1: Service provisioning subscription update

1.	The Edge Enabler Client sends a service provisioning subscription update request to the Edge Configuration Server. The service provisioning subscription update request includes the security credentials of the EEC received during EEC authorization procedure along with the subscription identifier and may include the UE identifier such as GPSI, connectivity information, proposed expiration time for the updated subscription and provisioning filters.

2.	Upon receiving the request, the Edge Configuration Server performs an authorization check to verify whether the Edge Enabler Client has authorization to perform the operation. If required, the ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.2. If authorized, the Edge Configuration Server updates the stored subscription for provisioning as requested in step 1.

3.	The Edge Configuration Server responds with a service provisioning subscription update response.

##### 8.3.2.3.5	Unsubscribe

Figure 8.3.2.3.5-1 illustrates the service provisioning unsubscribe procedure between the Edge Enabler Client and the Edge Configuration Server.

Pre-conditions:

1.	The Edge Enabler Client has subscribed with the Edge Configuration Server for the provisioning information as specified in clause 8.3.2.3.2.

Figure 8.3.2.3.5-1: Service provisioning unsubscribe

1.	The Edge Enabler Client sends a service provisioning unsubscribe request to the Edge Configuration Server. The service provisioning unsubscribe request includes the security credentials of the EEC received during EEC authorization procedure along with the subscription identifier.

2.	Upon receiving the request, the Edge Configuration Server performs an authorization check to verify whether the Edge Enabler Client has authorization to perform the operation. If authorized, the Edge Configuration Server cancels the subscription for provisioning as requested in step 1.

3.	The Edge Configuration Server responds with a service provisioning unsubscribe response.

### 8.3.3	Information flows

#### 8.3.3.1	Service provisioning request

Table 8.3.3.1-1 describes the information elements for service provisioning request from the Edge Enabler Client to the Edge Configuration Server.

Table 8.3.3.1-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| Provisioning filters | O | Set of information to filter the provisioning information, as described in Table 8.3.3.1-2. |

Editor's Note: It is FFS whether the EEC ID is mandatory or whether the UE ID should be included.

Table 8.3.3.1-2: Provisioning filters

| Information element | Status | Description |
| --- | --- | --- |
| ECSP information | O | Information about the preferred ECSP such as ECSP's name |
| List of Application Client characteristics (NOTE 1) | O | Describes the Application Clients for which a matching Edge Application Server is needed. |
| > Application Client profile | M | Application Client profile containing parameters used to determine matching EAS. Application Client profiles are further described in Table 8.2.2-1. |
| List of Edge Application Server characteristics (NOTE 1, NOTE 2) | O | Describes the characteristic of required Edge Application servers. |
| > EAS ID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS service area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > EAS status | O | Required status of the EAS (e.g. enabled, disabled, etc.) |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Only one of the information elements shall be present. NOTE 2: The ‘List of Edge Application Server characteristics’ IE must include at least one optional IE, if used as a provisioning filter. |  |  |

#### 8.3.3.2	Service provisioning response

Table 8.3.3.2-1 describes the information elements for service provisioning response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.2.-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| List of EDN configuration information | O | List of Edge Data Networks. |
| > EDN connection information | M | Information required by the UE to establish connection with the Edge Data Network. |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| >> S-NSSAI | O | Network Slice information if MNO provides |
| >> EDN Service Area (NOTE) | O | EDN service area information. The service area information is a list of Cell list or List of TA, PLMN IDs |
| >List of Edge Enabler Servers | M | List of Edge Enabler Servers of the Edge Data Network. |
| >> EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| >> ECSP info | O | Information for Edge Computing Service Provider |
| >> List of EAS IDs | O | A list of EAS ID(s) available via the EES. |
| > Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE: The EDN service area IE is included only if the EDN is a LADN. |  |  |

#### 8.3.3.3	Service provisioning subscription request

Table 8.3.3.3-1 describes the information elements for service provisioning subscription request from the Edge Enabler Client to the Edge Configuration Server.

Table 8.3.3.3-1: Service provisioning subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address | M | Notification Target Address of the EEC where the notification is to be sent by the ECS |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| Provisioning filters | O | Set of information to filter the provisioning information, as described in Table 8.3.3.1-2. |

#### 8.3.3.4	Service provisioning subscription response

Table 8.3.3.4-1 describes the information elements for service provisioning subscription response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.4.-1: Service provisioning subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

#### 8.3.3.5	Service provisioning notification

Table 8.3.3.5-1 describes the information elements for service provisioning notification from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.2.-1: Service provisioning notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the ECS for the request |
| List of EDN configuration information | O | List of Edge Data Networks. |
| > EDN connection information | M | Information required by the UE to establish connection with the Edge Data Network. |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| >> S-NSSAI | O | Network Slice information if MNO provides |
| >> EDN Service Area (NOTE) | O | EDN service area information. The service area information is a list of Cell list or List of TA, PLMN IDs |
| >List of Edge Enabler Servers | M | List of Edge Enabler Servers of the Edge Data Network. |
| >> EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| >> ECSP info | O | Information for Edge Computing Service Provider |
| >> List of EAS IDs | O | A list of EAS ID(s) available via the EES. |
| NOTE: The EDN service area IE is included only if the EDN is a LADN. |  |  |

#### 8.3.3.6	Service provisioning subscription update request

Table 8.3.3.6-1 describes the information elements for service provisioning subscription update request from the Edge Enabler Client to the Edge Configuration Server.

Table 8.3.3.6-1: Service provisioning subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| EEC ID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address | M | Notification Target Address of the EEC where the notification is to be sent by the ECS |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | Details about the connectivity of the UE with the PLMN, such as type of the 3GPP network. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| Provisioning filters | O | Set of information to filter the provisioning information, as described in Table 8.3.3.1-2. |

#### 8.3.3.7	Service provisioning subscription update response

Table 8.3.3.7-1 describes the information elements for service provisioning subscription update response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.7.-1: Service provisioning subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

#### 8.3.3.8	Service provisioning unsubscribe request

Table 8.3.3.8-1 describes the information elements for service provisioning unsubscribe request from the Edge Enabler Client to the Edge Configuration Server.

Table 8.3.3.8-1: Service provisioning unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the ECS |
| Security credentials | M | Security credentials of the EEC |

#### 8.3.3.9	Service provisioning unsubscribe response

Table 8.3.3.9-1 describes the information elements for service provisioning unsubscribe response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.9.-1: Service provisioning unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure |