3GPP TSG-SA WG6 Meeting #38-e	S6-201118

e-meeting, 20th – 31st July  2020	(revision of S6-20xxxx)

Source:	Samsung Electronics

Title:	Pseudo-CR on EDN and EAS service area

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

'EDN service area' and 'EAS service area' are two constructs that are being used in the TS without clear description of the constructs.

2. Reason for Change

The contribution illustrates the two constructs as commonly used values and resolves the following EN in clause 8.2.4:

Editor's Note: The current text in the TS has the definition of a 'service area' IE associated with certain entities (such as the EAS Service Area in the table above). The following are FFS: (1) Which entities should have a 'service area' and (2) What is the relationship between the service areas of the different entities if any.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.3.0.

* * * First Change * * * *

### 7.3.x	Service areas

#### 7.3.x.1		General

ECSPs and ASPs may allow access to Edge Computing service from specific areas i.e. allowing only the UEs within that area to access functional entities resident in the Edge Data Network. This area is called service area.

#### 7.3.x.2	Edge Data Network service area

ECSPs can use LADNs, as described in Annex A.2.3 of this document, to deploy EDNs with access restricted from specific areas. When an EDN is deployed using LADN, the EDN service area is same as the LADN service area and rules specified for LADN apply to the UE, as specified in 3GPP TS 23.501 [2].

Editor's Note:	For deployments using local DNs other than LADNs, the EDN service area is for further study.

#### 7.3.x.3	Edge Application Server service area

Edge Application Server service area enables the ASPs to define an area where Edge Application Server's service is available.

The service area of an Edge Application Server is limited by the service area of the EDN it resides in i.e. EAS service area is either same as the EDN service area or a subarea of the EDN service area it resides in.

* * * Next Change * * * *

## A.2.3	Option 3. Use of LADN

Edge computing services can be provided via Edge-dedicated Data Networks deployed as LADNs. The PLMN supports edge computing services in the EDN service areas which can be identified by the service area corresponding to the respective LADN DNNs. The LADN service area is the service area that the Edge Computing is supported. Each individual EAS in the LADN may support the same or smaller service area than the LADN.

Figure A.2.3-1: Option 3: Use of LADN(s)

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

AF	Application Function

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

EHE	Edge Hosting Environment

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

LADN	Local Area Data Network

NEF	Network Exposure Function

SCEF	Service Capability Exposure Function

SSID	Service Set Identifier

TAI	Tracking Area Identity

* * * Next Change * * * *

### 8.2.4	Edge Application Server Profile

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Service Area | O | The geographical service area that the EAS serves |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heart beat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Editor's Note: The inclusion of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

Editor's Note: It is FFS whether the Application Client ID(s) should be included and how Application Client ID can be mapped with EAS(s) at an EES.