3GPP TSG-SA WG6 Meeting #38-e	S6-201038

e-meeting, 20th – 31st July  2020

Source:	Tencent

Title:	Pseudo-CR on Annex X: UAV Aeronautical data communication with USS/UTM

Spec:	3GPP R 23.755

Agenda item:	8.5

Document for:	Approval

Contact:	shuaiizhao@tencent.com

1. Introduction

In the context of the FS_UASAPP study, SA6 received a reply LS from SA1 (S6-200620) stating that “Indicated parameters show by which means the UAV and UAV controller shall be identified and inform that list of identification data sent towards the UTM could be quite long”. It is then essential for SA6 as an application enabler to recognize how UAV data communication might be happening and how data would be formed.

This contribution lists potential communicated data format and sources in terms of UAV aeronautical parameter setting, data inquery and reply methods.

2. Reason for Change

The airspace-related UAV location stage 1 requirements are specified in TS 22.125 which are required to be supported by the 5GS. The 3GPP system shall provide the airspace location of a UE (or UAV itself) and enable data between UAV and USS/UTM. The data contains current position, take-off location, mission type, route data, operating status.

Various types of aeronautical charts are available in different countries and for different purposes (low/high altitude, visual/instrument flight rules, planning, etc.).  For the operation of small UAVs, of particular relevance are the US sectional charts, as well as the “Low Altitude Authorization and Notification Capability” (“LAANC”) information, which is displayed in the form of a chart on a web browser.  Charts are updated on a comparatively long-term cycle (for example; VFR sectional charts are updated every six months).  For manned aircraft, use of charts that are currently “in force” is required.

ASTM F3411-19 has developed the standard specification for Remote ID which defines two acceptable remote ID mechanism: Broadcast and network ID. The data format carried by by Remote ID message is also being defined.

However, it does not matter which ID might be in use among UAS’s tracking or authentication, the aeronautical information provided by ASTM is very important since it give the UE (UAS) the gists about what data might be transmitted between UAS and USS/UTM. It might have an affect on the UAS architecture design.

Meanwhile, requirement at Table 7.2-1 in stage 1 TS 22.125 also consider the message size for certain communications.

This Annex will also help with:

- Proposed new KI proposed in S6-210039

- Providing relvent supplement data information communicated between UAV and USS/UTM, which is specifided in the TS 22.125.

Therefore, it is imporant for SA6 to study the message format and size to fullfil stage 1 requirement.

3. Conclusions

4. Proposal

It is proposed to discuss and agree on this addition of a dedicated annex in the TR 23.755 providing potential communicated data format and sources in terms of UAV aeronautical parameter setting, data inquery and reply methods.

* * * First Change * * * *

Annex X (informative):
UAV Aeronautical data communication with USS/UTM

# X.1	Introduction

The description in this annex is derived from material external to 3GPP, such as the FAA, LAANC and ASTM International.

NOTE:	Given that the content of the external references will change over time, the content of this clause will not be maintained to match the current status of the references.

Various types of aeronautical charts are available in different countries and for different purposes (low/high altitude, visual/instrument flight rules, planning, etc.).  For the operation of small UAVs, of particular relevance are the US sectional charts, as well as the “Low Altitude Authorization and Notification Capability” (“LAANC”) information, which is displayed in the form of a chart on a web browser.  Charts are updated on a comparatively long-term cycle (for example; VFR (Virtual Flight rules) sectional charts are updated every six months).  For manned aircrafts, use of charts that are currently “in force” is required.

More recently, airspace authorities including the FAA have implemented modern query interfaces that allow automated download of information pertinent to a specific location with a granularity much finer than a state. These interfaces can be based on RESTful operations. REST, also known as Representational State Transfer, is a technique in which a client can query a server identified by a base Unified Resource Indicator (URI) through standard HTTP methods including, for example, GET, POST, PUT, PATCH, or DELETE) and in a defined format. One such defined standardized format is known as Java Object Notation or JSON.

This annex provides data information related to drone operations which were specified in TS 22.125[3] requriements.

## X.2	Essential Airport information for UAV operation

UAV locatoin information is an important compoment between UAV and UTM. The 3GPP system shall be able to enable trust-based location communication such as UAV take-off location, route information, fly path, location update either live or additional information based on networking condition. Therefore, having some knowledge about what UAV location contains could be very valueable for application enabler in terms of application interface design and transport protocol implications.

As pointed out by ASTM F3411 standardization [16], aeronautical information such as airport coverage, floor/ceiling (MSL), runway number and radio contact frequency (if it is a towered airport) are important to keep UE (in this case is a UAV) away from the active airport. The FAA’s LAANC program supports REST API based operation for a UAV to retrieve data from USS/UTM. Since there is no visual boundary in the sky, there is no easy way for a UE (UAV) to know its current airspace location. The following table x.2-1 lists some important aeronautical properties mentioned in ASTM International and the FAA:

Table X.2-1: Essential Airport information for UAV operation

| Aeronautical Property | Brief Description |
| --- | --- |
| Location | Airport center’s GPS location in terms of Latitude and Longitude Ex, KLVK: (37°41.60'N, 121°49.22'W) |
| Radius | The radius of the airport It contains multiple radius value due to various 3D airspace division |
| Ceiling | The ceiling of an airport in MSL/AGL Ex: KLVK 2900-meter AGL |
| Floor | The floor of an airport in MSL (mean sea Level) Ex: KVLK 400-meter MSL |
| Airport Class | The airport category Ex: A, B, C, D, E, G |
| Runway | Airport runways’ number/direction in magnetic direction Ex: runway 10 (100 degree) |

The following is a summary for each Aeronautical Property listed in table X.2-1

- Location : The airport GPS location is mandatory, which is the key parameter for UAV to identify an airport.

- Radius : The radius of the airport

- Ceiling/floor : The ceiling/floor of an airport in MSL/AGL

- Airport Class: The classification of various airport types includes controlled and uncontrolled airspaces.

- Runway: Runway direction of the airport. It could be multiple if there are multiple runways.

## X.3	Essential UAV flight restriction parameters

Potential fly hazedous can be communicated between UAV and UTM through 3GPP network. As pointed out in Clause 4.2 of TS 22.125 [3],  such alert may be communicated UTM-navigated C2 communication.  Fly hazedous could contain the current UAV’s location, fly path which can include the current altitude, ceiling, floor of the current path and time of airborne. If a UAV was pre-approved to an active restricted area such as an active airport, military operation area and etc.

Airspace restrictions are published in charts that are updated every few months. However, so-called Temporary Flight Restrictions (TFRs) or airport closures through Notice to Airmen (NOTAMs) can  could issue at any time and often become effective immediately. Even a professional operator of a manned aircraft can find himself violating such TFRs on occasion—a situation that can often be sorted out by direct communication of the operator with ATC. However, a typical drone operator is not in direct communication in ATC and could not know that his/her operation violates airspace restrictions including TFRs.

The drone could have a datalink using, for example, a 5G network link, to a server maintained by an aviation regulatory authority, a private entity entrusted by regulatory authorities to publish charts or TFRs, private operators without such mandate but offering it commercially or as a free service, and so forth.  Over the datalink, the drone could request information whether it is legal to fly in the airspace. The definition of that airspace could be an area in 3D space which the drone can access in a given time interval considering its performance (speed, climb rate, endurance, and so on).  Advantageously, to minimize the amount of information the server could need to transmit, the drone could include its current position, which it could have obtained by means such as its onboard GPS, 2D triangulation from 5G radio towers, and so on.  The drone could query such information at certain times, such as power-on, pre-flight check, take-off, in given time-intervals, at given distances from its controller, and so forth.

Table X.3-1 lists the referenced NOTAM/TFR data from FAA’s LAANC, TFR program and ASTM F3411-19.

Table X.3-1: Essential UAV flight restriction parameters

| NOTAM/TFR Parameters | Description |
| --- | --- |
| Location | NOTAM /TFR’s GPS location in terms of Latitude and Longitude Ex, KLVK: (37°41.60'N, 121°49.22'W) |
| Radius | The radius of the NOTAM/TFRs |
| Ceiling | The ceiling of a NOTAM/TFRs’ Ex: From the surface up to and including 3000 feet AGL |
| datetime | The effective date of a NOTAM/TFRs’ |
| Floor | The floor of a NOTAM/TFR in MSL (mean sea Level) Ex: 400 feet MSL |
| exempt | The exemption of operation in a NOTAM/TFR |

The following is a summary for each Aeronautical Property listed in table X.3-1:

- Location: NOTAM /TFR’s GPS location in terms of Latitude and Longitude

- Ceiling/floor: The ceiling/floor of a TFR in MSL/AGL

- Datatime: UAV Fly restriction timestamp.

- Exempt: an indicator of a special fly privilege such as fire rescue situation.

## X.4	UAS Facility Map

Currently in the Northa American, the mostly used UAS fly map is provided by the FAA. It contains UAV fly route data information such as fly vectors in the form of a JSON format to represent a geometry map coverage. The 3GPP system shall provide a mechasim to provide, deliver route information. This subsection provides a general UAS flight map catagories provided from FAA.

The published FAA Aeronautical information includes but not limited to VFR (virtual flight rules) sectional, Terminal Area chart, IFR Enroute, area and planning charts.  If a UAS/drone operates under specific national airspace such as in controlled airspace Class A, B, C, D, E, and uncontrolled airspace class G, it is important to obtain ATC authorization and airport NOTAM and be aware of current TFR in the vicinity.

FAA also provides a designated database for UAS operations under national security consideration. There is currently four UAS flight restriction data source: national security UAS flight restriction, part-time national security UAS flight restrictions, Recreational Flyer fixed sites, FAA UAS facility Map. Table X.4-1 shows a brief description of each data source.

Table X.4-1 FAA UAS facility map property summary

| Source | Description |
| --- | --- |
| National security UAS flight restriction | The FAA, pursuant to Title 14 of the Code of Federal Regulations (CFR) § 99.7, Special security instructions (SSI), has prohibited all UAS flight operations within the airspace defined under NOTAM FDC 7/7282. |
| part-time national securiry UAS flight restrictions | Partime UAS operation zone |
| Recreational Flyer fixed sites | This dataset represents active recreational flyer fixed sites (commonly referred to as flying fields) that are established by an agreement with the FAA. The fixed sites depicted here are located in controlled airspace two or more miles from an airport. At these sites, recreational UAS operations are authorized up to the unmanned aircraft system (UAS) facility map (UASFM) altitudes |
| FAA UAS facility Map | The UAS Facility Maps are designed to identify permissible altitudes (above ground level) at which UAS, operating under the Small UAS Rule (14 CFR 107), can be authorized to fly within the surface areas of controlled airspace. |

A UAS can retrieve UAS flight map from nearby USS/UTM. Table X.4-2 summarize some important flight map properties from FAA UAS facility map data source.

Table X.4-2: FAA UAS facility map property summary

| Properties | Description |
| --- | --- |
| Ceiling and floor Geometry rings | The Geometry data of a flight map in terms of ceiling and floor |
| Ceiling | The ceiling of a fly map Ex: From the surface up to and including 400 feet AGL |
| Floor | The floor of an fly map Ex: 0 feet AGL |