3GPP TSG-SA WG6 Meeting #38-e	S6-201037

e-meeting, 20th – 31stth July 2020	(revision of S6-20xxxx)

Source:	InterDigital

Title:	Pseudo-CR on enhancing EAS Profile with Network Service Area

Spec:	3GPP TS 23.558 v 0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Atle Monrad, Atle.Monrad@InterDigital.com

1. Introduction

For certain edge services, it may be necessary/advantageous to have fine grained configurability of the Edge Application Server (EAS) service area.

As such, this pCR proposes to extend the EAS Profile

2. Reason for Change

Provide finer grained EAS service area definition.

3. Proposal

It is proposed to agree to the following change to clause 8.2.4 of 3GPP TS 23.558 v 0.3.0.

* * * First Change * * * *

### 8.2.4	Edge Application Server Profile

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Service Area | O | The geographical service area that the EAS serves |
| EAS Network Service Area | O | The network service area that the EAS serves (e.g. cell IDs, tracking area IDs) |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heart beat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Editor's Note: The inclusion of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

Editor's Note: It is FFS whether the Application Client ID(s) should be included and how Application Client ID can be mapped with EAS(s) at an EES.

Editor's Note: The current text in the TS has the definition of a 'service area' IE associated with certain entities (such as the EAS Service Area in the table above). The following are FFS: (1) Which entities should have a 'service area' and (2) What is the relationship between the service areas of the different entities if any.

* * * End Change * * * *