3GPP TSG-SA WG6 Meeting #38-e	S6-20xxxx

e-meeting, 20th – 31stth July 2020	(revision of S6-201268, S6-200980)

Source:	Apple, Ericsson

Title:	Pseudo-CR on Specification of the Session with QoS API

Spec:	3GPP TS 23.558, v 0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Robert Zaus / <rzaus> <at> <apple> <dot> <com>

1. Introduction

For certain Edge services the Edge Application Server (EAS) needs the capability to request the 5G core network to provide a data session (QoS flow) with a specific QoS.

According to TR 23.758, Solution #11 "QoS Management for 5G Edge Applications", information related with QoS can be exposed to the EAS directly from the NEF or, as a 2nd option, the EAS can request QoS and receive QoS notifications via the Edge Enabler Server (EES), i.e. via the EDGE-3 and N5 reference points.

2. Reason for Change

The first solution mentioned above is based on the use of service based operations between AF (= EAS) and NEF (Nnef_AFsessionWithQoS service, see 3GPP TS 23.502) and has already been specified, but for the 2nd option the necessary procedures for the EDGE-3 reference point are missing.

3GPP TS 23.502 / Figure 4.15.6.6-1: Setting up an AF session with required QoS procedure

At SA6#37-e it was suggested that a separate API for the EDGE-3 reference point should be specified for this purpose.

The present pCR proposes a Session with QoS API by which the EAS can request the EES to trigger the establishment of a data session between AC and EAS with a specific QoS. (The proposal is drawing on the example of the Nnef_AFsessionWithQoS service shown in the figure above.)

It is more desirable for the EAS to have a separate interaction with edge enabling layer to influence the QoS for the service data flow, sponsor the traffic and/or monitor the QoS with the following add-on values:

-	If the EAS does not have access directly with 3GPP network, it can use EES provided Policy and Charging capability exposure service via EDGE-3 reference point.

-	For the EES provided Policy and Charging capability exposure service, since it may be associated with different AF events (e.g. usage reporting), another advantage to use such EES provided service is to have the advanced control for event reporting. For instance, the event reporting mode of “one time reporting” can be supported in the EES by unsubscribing the corresponding PCF/NEF event once the 1st report is received, EES can also control the max. number of report and reporting period so to provide an enhanced version of Policy and Charging exposure with more event reporting control.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558, v 0.3.0.

* * * First Change * * * *

### 6.4.4	EDGE-3

EDGE-3 reference point enables interactions between the Edge Enabler Server and the Edge Application Servers. It supports:

a)	registration of Edge Application Servers with availability information (e.g. time constraints, location constraints);

b)	de-registration of Edge Application Servers from the Edge Enabler Server;

c)	providing access to network capability information (e.g. location information, QoS related information); and

d)	requesting the setup of a data session between Application Client and Edge Application Server with a specific QoS.

NOTE:	Optimized distribution of events across the EDGE-3 interface is out of scope of this specification.

* * * Next Change * * * *

### 8.6.X	Session with QoS API

#### 8.6.X.1	General

The Edge Enabler Server exposes the Session with QoS API to the Edge Application Server in order to support the setup of a data session between Application Client and Edge Application Server with a specific QoS and the modification of the QoS of this data session.

The Session with QoS API exposed by the Edge Enabler Server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [12], if the Edge Enabler Server is connected to the PCF via the N5 reference point, or on the northbound AF Session with QoS Service API exposed by the NEF as specified in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [12], if the Edge Enabler Server is connected to the PCF via an NEF.

The level of support of the Session with QoS API may change due to UE mobility between 5GC and EPC. E.g. QoS monitoring is only applicable in 5GC. If an Edge Enabler Server exposes the Session with QoS API to Edge Application Server(s) for a UE, the Edge Enabler Server monitors such changes and provides the available information to the Edge Application Server.

Alternatively to using the API described in the following clauses, the Edge Application Server may also act as an AF and directly access the 3GPP Core Network capabilities to request an AF session with QoS (see clause 8.7.2). For any data session between Application Client and Edge Application Server, the Edge Application Server shall use only one of these alternatives.

#### 8.6.X.2	Procedures

Figure 8.6.X.2-1 illustrates the session with QoS create operation between the Edge Application Server and the Edge Enabler Server. It is used to request reservation of resources for a data session between Application Client and Edge Application Server with a specific QoS and to subscribe to certain session with QoS event notifications.

Figure 8.6.X.2-1: Session with QoS API: create operation

1.	The Edge Application Server requests establishment of a data session between the Application Client and the Edge Application Server with a specific QoS (either QoS reference or bandwidth). The Edge Application Server shall include the UE's IP address and may include the Edge UE ID, the DNN and S-NSSAI used for the data session between Application Client and Edge Application Server. With the same request the Edge Application Server subscribes to receive certain session with QoS event notifications (e.g. notifications that the QoS targets can no longer (or can again) be fulfilled).

Editor's note: The support of a list of alternative QoS references via the EDGE-3 reference point is FFS.

Editor's note: It is FFS which events are supported via the EDGE-3 reference point.

Editor's note: For event reporting control, the Event reporting information IE and the Frequency of reporting IE associated with the Event list are FFS.

2.	The Edge Enabler Server checks if the Edge Application Server is authorized for this operation for the UE. If authorized, then the Edge Enabler Server invokes the Policy Authorization Create service or the AF Session with QoS service with the 3GPP Core Network (PCF or NEF, respectively) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], providing the specific QoS (QoS reference or bandwidth) to the PCF as described in 3GPP TS 23.503 [12], clause 6.1.3.22. Additionally the Edge Enabler Server may subscribe to notifications of resource allocation outcome and to other events described in clause 6.1.3.18 of TS 23.503 [12], e.g. notifications of when the QoS targets can no longer (or can again) be fulfilled.

3.	If the operation is successful, the Edge Enabler Server responds with a Context ID and a Result. The Context ID is to be used by the Edge Application Server for further requests (e.g. session with QoS update requests) pertaining to the same UE. If the Edge Application Server is not authorized or any other failure happens during the operation, the Edge Enabler Server provides a rejection response with cause information.

NOTE 1:	The Edge Enabler Server will report the resource allocation outcome, e.g. the successful allocation of the Service Data Flow(s) related to the data session, with a separate session with QoS notify operation (see TS 23.503 [12], clause 6.1.3.18).

Figure 8.6.X.2-2 illustrates the session with QoS update request operation between the Edge Application Server and the Edge Enabler Server. It is used to request a modification of the QoS of the data session between Application Client and Edge Application Server.

Figure 8.6.X.2-2: Session with QoS API: update operation

1.	The Edge Application Server requests a modification of the QoS of the data session between the Application Client and the Edge Application Server, e.g. by including a different QoS reference or different bandwidth value. The Edge Application Server shall include the Context ID.

The Edge Application Server may also request a modification of the event monitoring by subscribing to new events and updating or removing subscriptions to existing events.

Editor's note: The support of a list of alternative QoS references via the EDGE-3 reference point is FFS.

2.	The Edge Enabler Server checks if there is a context for the Context ID available and whether the Edge Application Server is authorized for this operation. If yes, the Edge Enabler Server invokes the Policy Authorization Update service or AF Session with QoS Update service with the 3GPP Core Network (PCF or NEF, respectively) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], providing the updated information to the PCF or NEF, respectively. The Edge Enabler Server may subscribe to additional events or unsubscribe to certain events, if necessary.

3.	If the operation is successful, the Edge Enabler Server responds with a Context ID and a Result. If the Edge Application Server is not authorized or any other failure happens during the operation, the Edge Enabler Server provides a rejection response with cause information.

NOTE 2:	The Edge Enabler Server will report the resource allocation outcome, e.g. the successful modification of the Service Data Flow(s) related to the data session, with a separate session with QoS notify operation (see TS 23.503 [12], clause 6.1.3.18).

Figure 8.6.X.2-3 illustrates the session with QoS revoke operation between the Edge Application Server and the Edge Enabler Server. It is used to revoke the data session between Application Client and Edge Application Server with a specific QoS and to unsubscribe to the related session with QoS event notifications.

Figure 8.6.X.2-3: Session with QoS API: revoke operation

1.	The Edge Application Server requests a revoke of the data session with QoS between the Application Client and the Edge Application Server. The Edge Application Server shall include the Context ID.

2.	The Edge Enabler Server checks if there is a context for the Context ID available and whether the Edge Application Server is authorized for this operation. If yes, then the Edge Enabler Server invokes the Policy Authorization Delete service or AF Session with QoS Revoke service with the 3GPP Core Network (PCF or NEF, respectively) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]. This deletes the application session context at the PCF or NEF and removes the subscription to any session with QoS event notifications.

3.	If the operation is successful, the Edge Enabler Server responds with a Context ID and a Result. If the Edge Application Server is not authorized or any other failure happens during the operation, the Edge Enabler Server provides a rejection response with cause information.

Figure 8.6.X.2-4 illustrates the notify operation between the Edge Enabler Server and the Edge Application Server for session with QoS event notifications.

Figure 8.6.X.2-4: Session with QoS API: notify operation

1.	The Edge Enabler Server detects a user plane event associated with the established session (i.e. it receives a Policy Authorization Notify operation from the PCC or an AF Session with QoS Notify operation from the NEF as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]). The Edge Enabler Server determines to notify the session with QoS event notification information (e.g., resource allocation outcome or information that the QoS targets can no longer (or can again) be fulfilled) to the Edge Application Servers which have subscribed to the session with QoS event notification.

2.	The Edge Enabler Server sends a session with QoS event notify operation to the Edge Application Server. The Edge Enabler Server includes the session with QoS event notification information of the UE.

#### 8.6.X.3	Information flows

##### 8.6.X.3.1	Session with QoS create request

Table 8.6.X.3.1-1 describes the information elements for a Session with QoS create request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.X.3.1-1: Session with QoS create request

| Information element | Status | Description |
| --- | --- | --- |
| UE IP address | M | The UE IP address. |
| Edge UE ID | O | Identifier uniquely identifying the UE in the EES. |
| IP flow description | M | The IP flow description for the application traffic. |
| Requested QoS reference (NOTE 1) | O | Refers to pre-defined QoS information for the data session between AC and EAS (NOTE 2). |
| Event list | O | A list of associated events to which the EAS subscribes (see 3GPP TS 23.503 [12], clause 6.1.3.18). |
| Notification Target Address | O | Notification Target Address of the EAS where the notification is to be sent by the EES. |
| DNN | O | DNN for the data session between AC and EAS |
| S-NSSAI | O | S-NSSAI for the data session between AC and EAS |
| Requested bandwidth (NOTE 1) | O | Bandwidth requested for the data session between AC and EAS |
| NOTE 1: Only one of requested QoS reference or requested bandwidth shall be provided. NOTE 2: The pre-defined QoS information may be configured in the EES or in the 3GPP Core Network. |  |  |

Editor's note: The support of a list of alternative QoS references via the EDGE-3 reference point is FFS.

Editor's note: It is FFS which events are supported via the EDGE-3 reference point.

Editor's note: For event reporting control, the Event reporting information IE and the Frequency of reporting IE associated with the Event list are FFS.

##### 8.6.X.3.2	Session with QoS create response

Table 8.6.X.3.2-1 describes the information elements for a Session with QoS create response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.X.3.2-1: Session with QoS create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the operation is successful or failure |
| Context ID (NOTE 1) | O | Context identifier corresponding to the information stored for the request in the EES |
| Cause information (NOTE 2) | O | Indicates the cause information for the failure |
| NOTE 1: The Context ID IE is included when the Result indicates success. NOTE 2: The Cause information IE is included when the Result indicates failure. |  |  |

##### 8.6.X.3.3	Session with QoS update request

Table 8.6.X.3.3-1 describes the information elements for a Session with QoS update request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.X.3.3-1: Session with QoS update request

| Information element | Status | Description |
| --- | --- | --- |
| Context ID | M | Context identifier corresponding to the information stored for the request in the EES. |
| Requested QoS reference (NOTE 1) | O | Refers to pre-defined QoS information for the data session between AC and EAS (NOTE 2). |
| Event list | O | A list of associated events to which the EAS subscribes (see 3GPP TS 23.503 [12], clause 6.1.3.18). |
| Requested bandwidth (NOTE 1) | O | Bandwidth requested for the data session between AC and EAS |
| NOTE 1: Only one of requested QoS reference or requested bandwidth shall be provided. NOTE 2: The pre-defined QoS information may be configured in the EES or in the 3GPP Core Network. |  |  |

Editor's note: The support of a list of alternative QoS references via the EDGE-3 reference point is FFS.

Editor's note: It is FFS which events are supported via the EDGE-3 reference point.

Editor's note: For event reporting control, the Event reporting information IE and the Frequency of reporting IE associated with the Event list are FFS.

##### 8.6.X.3.4	Session with QoS update response

Table 8.6.X.3.4-1 describes the information elements for a Session with QoS update response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.X.3.4-1: Session with QoS update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the operation is successful or failure |
| Cause information (NOTE) | O | Indicates the cause information for the failure |
| NOTE: The Cause information IE is included when the Result indicates failure. |  |  |

8.6.X.3.5	Session with QoS revoke request

Table 8.6.X.3.5-1 describes the information elements for a Session with QoS revoke request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.X.3.5-1: Session with QoS event API revoke request

| Information element | Status | Description |
| --- | --- | --- |
| Context ID | M | Context identifier corresponding to the information stored for the request in the EES |

##### 8.6.X.3.6	Session with QoS revoke response

Table 8.6.X.3.6-1 describes the information elements for a Session with QoS revoke response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.X.3.6-1: Session with QoS revoke response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the operation is successful or failure |
| Cause information (NOTE) | O | Indicates the cause information for the failure |
| NOTE: The Cause information IE is included when the Result indicates failure. |  |  |

##### 8.6.X.3.7	Session with QoS event notification

Table 8.6.X.3.7-1 describes the information elements for a Session with QoS event notification from the Edge Enabler Server to the Edge Application Server.

Table 8.6.X.3.7-1: Session with QoS event notification

| Information element | Status | Description |
| --- | --- | --- |
| Context ID | M | Context identifier corresponding to the information stored in the EES during the initial session with QoS create request. |
| Event report | M | Event reporting information as specified in clause 6.1.3.18 of 3GPP TS 23.503 [12]. |

Editor's note: It is FFS which events are supported via the EDGE-3 reference point.

#### 8.6.X.4	APIs

##### 8.6.X.4.1	General

Table 8.6.X.4.1-1 illustrates the API for Session with QoS.

Table 8.6.X.4.1-1: Session with QoS API

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| EDGE3_Session_with_QoS API | Create | Request/Response | Edge Application Server |
|  | Update | Request/Response | Edge Application Server |
|  | Revoke | Request/Response | Edge Application Server |
|  | Notify | Subscribe/Notify (NOTE) | Edge Application Server |
| NOTE: The Subscribe operation is embedded in the Request operation of the Create and Update service operation. |  |  |  |

##### 8.6.X.4.2	EDGE3_Session_with_QoS_create operation

Service operation name: EDGE3_Session_with_QoS_create

Description: The consumer requests a data session with the UE with a specific QoS and may also subscribe to certain event notifications related to the user plane traffic.

Inputs: See clause 8.6.X.3.1.

Outputs: See clause 8.6.X.3.2.

See clause 8.6.X.2 for details of usage of this operation.

##### 8.6.X.4.3	EDGE3_Session_with_QoS_update operation

Service operation name: EDGE3_Session_with_QoS_update

Description: The consumer requests a modification of the QoS of the data session with the UE and may also update event notifications related to the user plane traffic.

Inputs: See clause 8.6.X.3.3.

Outputs: See clause 8.6.X.3.4.

See clause 8.6.X.2 for details of usage of this operation.

##### 8.6.X.4.4	EDGE3_Session_with_QoS_revoke operation

Service operation name: EDGE3_Session_with_QoS_revoke

Description: The consumer requests a revoke of the data session with QoS with the UE and unsubscribes to all related event notifications.

Inputs: See clause 8.6.X.3.5.

Outputs: See clause 8.6.X.3.6.

See clause 8.6.X.2 for details of usage of this operation.

##### 8.6.X.4.5	EDGE3_Session_with_QoS_event_notify operation

Service operation name: EDGE3_Session_with_QoS_event_notify

Description: The consumer is notified by the Edge Enabler Server of an event related to the user plane traffic.

Inputs: See clause 8.6.X.3.7.

Outputs: None.

See clause 8.6.X.2 for details of usage of this operation.

* * * Next Change * * * *

### 8.9.3	Capabilities utilized by Edge Enabler Server

When required, the Edge Enabler Server may utilize:

-	user plane path management events by subscribing with the 3GPP core network for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

-	the location information from the API exposed by 3GPP core network, e.g. SCEF/NEF/SCEF+NEF or LCS (Location Service) as specified in 3GPP TS 23.502 [3], 3GPP TS 23.271 [7], 3GPP TS 36.305 [8], 3GPP TS 23.273 [9] and 3GPP TS 38.305 [10] to obtain the UE's location from the 3GPP Core Network.

-	capabilities exposed by the 3GPP core network, e.g. NEF or PCF, to establish an AF session with QoS, and QoS related event notifications subscribed with the 3GPP core network as specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [12].