3GPP TSG-SA6 Meeting #39-bis-e	S6-201774

- Online, , 12th Oct 2020 - 20th Oct 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0021 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC5 Provisioning in multi-operator V2X scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2020-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces the support capability at the VAE layer for providing common PC5 provisioning policies/parameters in multi-operator V2X scenarios (based on the approved Solution #7 of TR 23.764). |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause (4.x) is added for introducing new requirements for the proposed capability. Furthermore, a new clause (9.x) is added for introducing the new VAE layer capability for PC5 Provisioning in multi-operator V2X scenarios (clause 9.x.2 provides the information flows, and clause 9.x.3 provides the procedure). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the mechanism at the VAE layer for supporting PC5 provisioning in multi-operator V2X scenarios cannot be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

## 4.X	PC5 Provisioning in multi-operator V2X scenarios

### 4.X.1	Description

This subclause specifies the PC5 Provisioning for multi-operator V2X services related requirements.

### 4.X.2	Requirements

[AR-4.X.2-a] The VAE capabilities shall support PC5 policy/parameter provisioning for V2X services provided by a single V2X service provider; and offered by more than one MNO (aka multi-operator V2X services).

[AR-4.X.2-b] The VAE capabilities shall enable provisioning joint PC5 policy/parameters for multi-operator V2X services.

[AR-4.X.2-c] The VAE capabilities shall enable the VAE client to provide to the VAE server information on the status of PC5 policies /parameters, for assisting the provisioning of joint PC5 policy/parameters for multi-operator V2X services.

[AR-4.X.2-d] The VAE capabilities shall enable the VAE server to provide to the V2X-UEs within the muti-operator V2X service, the joint PC5 policy/parameters for multi-operator V2X services.

* * * * * * * SECOND CHANGE * * * * * * *

## 9.x	PC5 Provisioning in multi-operator V2X scenarios

### 9.x.1	General

The VAE server provides PC5 provisioning for multi-operator V2X services, based on the requirement received by the V2X application specific server.

### 9.x.2	Information flows

#### 9.x.2.1	V2X PC5 provisioning requirement

Table 9.x.2.1-1 describes the information flow V2X PC5 provisioning requirement from the V2X application specific server to the VAE server.

Table 9.x.2.1-1: V2X PC5 provisioning requirement

| Information element | Status | Description |
| --- | --- | --- |
| V2X UE ID | M (NOTE) | Identity of the V2X UE for which V2X application requirement is initiated. |
| V2X group ID | M (NOTE) | Identity of the V2X group for which the V2X application requirement is initiated. |
| V2X service ID | M | The V2X service ID for which application requirement corresponds to. |
| Policy/parameters per PLMN | M | The V2X service policy/ radio parameters which are provided by the V2X application specific server (as specified in TS 23.287, clause 5.1.2.1) to the VAE server. |
| Multi-operator V2X service indication | O | An indication that the V2X service comprises V2X-UEs which are connected to more than one PLMNs |
| PLMN ID list | O | The list of the PLMN identities for the PLMNs which offer the V2X service |
| V2X application QoS requirements | O | The application QoS requirements (reliability, delay, jitter) for the V2X service |
| NOTE: Either of the information element should be present. |  |  |

#### 9.x.2.2	PC5 provisioning status request

Table 9.x.2.2-1 describes the information flow PC5 provisioning status request from the VAE server to the VAE client.

Table 9.x.2.2-1: PC5 provisioning status request

| Information element | Status | Description |
| --- | --- | --- |
| VAE Server ID | M | Identity of the VAE server which is requester of the PC5 parameters status. |
| V2X service ID | M | The V2X service ID for which the VAE server’s request corresponds to. |
| PC5 provisiong status report configuration | O | The configuration of the VAE-client reporting related to the PC5 Policy/Parameters status, and optionally PC5 events like PC5 unavailability, PQI load/congestion info |

#### 9.x.2.3	PC5 provisioning status response

Table 9.x.2.3-1 describes the information flow PC5 provisioning status response from the VAE client to the VAE server.

Table 9.x.2.3-1: PC5 provisioning status response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the PC5 provisioning status request. |
| PC5 Policy/Parameters status report | M | The report based on the PC5 Policy/Parameters status request. This includes up-to-date policies/parameters: - selected PQI attributes for the V2X service - PQI load information - communication range for the V2X service - a RAT /interface (NR-PC5, LTE-PC5) preference - an expected RAT availability / unavailability indication - an expected V2X-UE moving out of coverage indication |

#### 9.x.2.4	Multi-operator PC5 provisioning request

Table 9.x.2.4-1 describes the information flow multi-operator PC5 provisioning request from the VAE server to the VAE client.

Table 9.x.2.4-1: Multi-operator PC5 provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| VAE Server ID | M | Identity of the VAE server which is requester of the Multi-operator V2X provisioning request |
| Service ID | M | The V2X service ID for which the VAE server’s request corresponds to. |
| Joint PC5 Provisioning policies/ parameters | M | The common provisioning policies/parameters to be used by the V2X-UEs within the V2X service. These may be: - common policies per RAT (NR PC5, LTE PC5) - a list of common available PQIs for the multi-operator V2X service -an updated V2X application QoS -to- PQI mapping configuration for the multi-operator V2X service -an updated application priority indication |
| Joint radio parameters for out of coverage scenarios | M | The common radio parameters (e.g. resource pool configurations) for joint utilization by the V2X-UEs of the multi-operator V2X service in out of coverage scenarios. |
| Geographical area for the provisioning | O | The geographical area for which the V2X provisioning applies |
| Time validity | O | The time window for which the V2X provisioning applies |

#### 9.x.2.5	Multi-operator PC5 provisioning response

Table 9.x.2.5-1 describes the information flow Multi-operator PC5 provisioning response from the VAE client to the VAE server.

Table 9.x.2.3-1: Multi-operator PC5 provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the multi-operator V2X provisioning request. |

#### 9.x.2.6	V2X PC5 provisioning requirement notification

Table 9.x.2.6-1 describes the information flow V2X PC5 provisioning requirement notification from the VAE server to the V2X application specific server.

Table 9.x.2.6-1: V2X PC5 provisioning requirement notification

| Information element | Status | Description |
| --- | --- | --- |
| Notification | M | The result corresponding to the V2X PC5 provisioning requirement. |

### 9.x.3	PC5 provisioning for multi-operator V2X services

#### 9.x.3.1	General

This subclause describes the procedure for PC5 provisioning for multi-operator V2X services.

#### 9.x.3.2	Procedure

Figure 9.11.3.2-1 illustrates the procedure where the VAE server supports the PC5 provisioning for multi-operator V2X services.

Pre-conditions:

- The V2X UEs have connected to the V2X application server.

- The V2X UEs are authorized by their corresponding PLMNs to use PC5 communication.

Figure 9.x.3.2-1: PC5 provisioning for multi-operator V2X services

1.	The V2X application specific server provides a V2X PC5 provisioning requirement to the VAE server.

2.	The VAE Server sends a PC5 provisioning status request to the VAE Client (within the multi-operator V2X service) to receive up-to-date information on the per PLMN provisioning policies/ parameters.

3.	The VAE Client sends a PC5 provisioning status response to VAE server, which includes a report on the up-to-date policies /parameters per PLMN (e.g. application QoS - to - PQI mapping, range, radio parameters) and optionally PC5 related events (PC5 expected link downgrade).

4.	The VAE Server determines the updated PC5 provisioning policies/parameters to be jointly used across the V2X-UEs within the multi-operator V2X service.

5.	The VAE Server sends a Multi-operator PC5 provisioning request to the VAE client of the V2X UE.

6.  The VAE client sends a Multi-operator PC5 provisioning response to the VAE Server to notify on the result.

7.	VAE Server sends a V2X PC5 provisioning requirement notification to the V2X application specific server to inform on the result of the multi-operator PC5 provisioning.