3GPP TSG-SA WG6 Meeting #39-e	S6-20xxxx

e-meeting, 31st August – 8th September 2020	revision of S6-201380

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on FS 5GMARCH solution on Architecture

Spec:	3GPP TR 23.700-24 v0.7.0

Agenda item:	8.6

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

The existing architecture needs to be improved, especially for interworking with legacy 3GPP message service.

2. Reason for Change

Mainly change:

- MSGin5G Gateway Non-3GPP Message Gateway

- Add a Legacy 3GPP Message Gateway for interworking with Legacy 3GPP Message service.

- For SMS, specify that the Legacy 3GPP Message Gateway is connected directly with SMSC.

- Remove the reference point MSGin5G-X and MSGin5G-Y which is out of scope.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-24 v0.7.0.

* * * First Change * * * *

## 8.1	Application architecture principles

The application architecture for MSGin5G Application Service is designed based on the following architecture principles

The three logical UE types, depending on the type of underlying transport services supported, as follows:

-	5GMSGS UE type - shall support 5GMSGS client and all delivery mechanisms applicable for sending MSGin5G messages request, and is able to deliver and receive a MSGin5G message.

NOTE 1: Support for the MSGin5G message delivery mechanism is not equivalent with support for MSGin5G Application Services. All UE types described in this clause support MSGin5G Application Services

-	Legacy 3GPP UE type - shall not support 5GMSGS client and shall support one of the 3GPP defined message delivery mechanism (e.g. SMS, NIDD, etc).

-	Non-3GPP UE type - shall not support 5GMSGS client and shall support one of non-3GPP message delivery mechanism (e.g. RCS, LwM2M).

NOTE 2:  For UEs which do not host a 5GMSGS Client, the Message Gateway provides interworking to other messaging services (e.g. SMS) to deliver messages to/from these UEs.

NOTE 3: Broadcast is a special message delivery mechanism that are not associated with the type of UE, it is possible for any UE that includes a 3GPP broadcast client to receive the broadcast message when it is in the broadcast areas.

NOTE 4: Every type of UE may be originating and/or terminating UE for MSGin5G messages, depending on implementation or configuration.

For each of these types, support of the corresponding messaging delivery mechanism is exclusive, i.e. 5GMSGS UE type supports only MSGin5G message delivery mechanism and does not support legacy 3GPP message delivery mechanism or non-3GPP message delivery mechanism. Examples of MSGin5G Service deployment scenarios are described in clause 4.

MSGin5G Application Service supports UE:

-	That could be IMS and non-IMS base

-	a non-IMS UE may be 3GPP UE and non-3GPP UE

-	5GMSGS UE comprises both application client (optional) and 5GMSGS client.

NOTE 4:	Application service logic in the application client is out of scope of this specification.

## 8.2	Application architecture

Figure 8.2-1 shows the application architecture of the MSGin5G Application Service.  The MSGin5G Application Service shall fulfil the service requirements which are enumerated in 3GPP TS 22.262 [2].

NOTE 1: "5GMSGS" in this document is referred to the new defined transport for the MSGin5G service. The service name in this document uses the "MSGin5G Service" term in 3GPP TS 22.262 [2].

Figure 8.2-1: Application Architecture of the MSGin5G Service

Editor's note:	Representing usage of SEAL in the above application architecture figure in 8.2-1 is FFS.

Editor's note:	Expanding the above architecture figure for Legacy 3GPP UE (e.g NIDD) is FFS.

“Broadcast is a special message delivery mechanism that are not associated with the type of UE, it is possible for any UE that includes a 3GPP broadcast client to receive the broadcast message when it is in the broadcast areas.”

The 5GMSGS client(s) interacts with SEAL clients over the SEAL-C reference point specified for each SEAL service. The MSGin5G server(s) interacts with SEAL servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL client and the corresponding SEAL server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [9].

Editor's note: The MSGin5G Service enhancements to reference points SEAL-C, SEAL-S, SEAL-UU reference points in 3GPP TS 23.434 [9] is FFS.

## 8.3	Functional elements

### 8.3.1	General

The functional entities of the application architecture for the MSGin5G Service are described in this clause.

### 8.3.2	MSGin5G Server

A MSGin5G server provides server-side functionality to assist 5GMSGS clients with the sending and receiving of messages via the MSGin5G Service to/from Application Servers and/or other MSGin5G service endpoints on other UEs.

Functionalities of MSGin5G Server:

-	To resolve message delivery mechanism for the MSGin5G service end point based on the terminating MSGSin5G service ID to determine if the message is to be delivered to a 5GMSGS UE, an Application Server or a Message Gateway for final delivery;

-	Interworking with non 3GPP messaging service through the Non-3GPP Message Gateway;

-	Interworking with legacy 3GPP messaging service through the Legacy 3GPP Message Gateway;

-	Exchanging MSGin5G messages with application servers, 5GMSGS clients, Legacy 3GPP Message Gateway and Non-3GPP Message Gateway; and

-	Providing information related to the MSGin5G Service, such as 5GMSGS client availability, to 5GMSGS clients, Legacy 3GPP Message Gateway and Non-3GPP Message Gateway.

### 8.3.3	5GMSGS Client

A 5GMSGS client provides client-side functionality for UE Application Clients with the sending and receiving of messages via the MSGin5G Service to/from Application Servers and/or other MSGin5G service endpoints; i.e. UEs.

Functionalities of 5GMSGS client:

-	Exposing MSGin5G APIs to enable application clients to use a MSGin5G Service;

-	Suppporting registration of a 5GMSGS client to a MSGin5G Server to enable the exchange of configuration information required to use a MSGin5G Service; and

-	Exchanging MSGin5G messages via a MSGin5G Server to/from Application Servers and/or other MSGin5G service endpoints; i.e. UEs.

### 8.3.4	Message Gateway

A Message Gateway in MSGin5G application architecture provides functionality to deliver a MSGin5G messages to the non-5GMSGS UE. The application architecture in figure 8.2-1 shows two types of Message Gateways used to deliver MSGin5G messages to different types of non-5GMSGS UEs; i.e. a Non-3GPP Message Gateway is used to deliver MSGin5G message to non-3GPP UEs and the Legacy 3GPP Message Gateway is used to deliver MSGin5G message to Legacy 3GPP UEs.

A Message Gateway performs the role of interconnecting two different messaging delivery mechanisms and assure the message integrity between different message delivery mechanisms. A message delivery mechanism comprises the specific set of protocols, procedures and rules.

Functionalities of a Message Gateway:

- Enable seamless delivery of  a MSGin5G message between different message delivery mechanisms with integrity;

- Communicate with the MSGin5G server using either a 5GMSGS client or similar functions to enable sending and receiving MSGin5G messages;

- Deliver payload of a MSGin5G message to the non-5GMSGS UE using the specific message delivery mechanism available to that non-5GMSGS UE;

- Perform message sender and receiver addresses conversion according to the two connected message delivery mechanisms and maintain the mapping of the address pair used for a response message delivery;

- Perfomer protocols and procedures conversion between the two connected message delivery mechanisms and assure no loss of message payload and essential non message payload information such as a delivery report request;

- Act as a service endpoint to perform message segmentation and reassembly for the non-5GMSGS UEs when needed;

- Perform UE registration, on behave of a non-5GMSGS UE, with the MSGin5G server, if appropriate, and maintain the status of the non-5GMSGS UE registration status.

NOTE 1:	Implementation of the Legacy 3GPP Message Gateway and the MSGin5G server together is deployment option that is out of this specification.

NOTE 2:	For interworking with SMS, the Legacy 3GPP Message Gateway is connected directly to SMSC, the interface between Legacy 3GPP Message Gateway and SMSC is based on 3GPP TS 23.040 [11].

### 8.3.5	Application Client

The application client provides client-side messaging service logic and needed information for the 5GMSGS client to perform MSGin5G service with other endpoints. Using application client to provide this information to the 5GMSGS client is optional as there are other ways to provide this information to the 5GMSGS client such as a native UE implementation with pre-configured information.

### 8.3.6	Application Server

The Application Server supports sending and receiving messages with 5GMSGS service layer protocols and procedures.

### 8.3.7	Legacy 3GPP UE

The Legacy 3GPP UE, including SMS UE with SMS Client, and including other Legacy 3GPP UE (e.g. NIDD) as shown on the Figure 8.2-1, provides client-side functionality for a legacy 3GPP messaging service.

NOTE:	The details of the Legacy 3GPP UE are out of scope of the present document.

### 8.3.8	Non-3GPP Message Client

The Non-3GPP message client provides client-side functionality for a Non-3GPP messaging service.

NOTE:	The details of the Non-3GPP message client are out of scope of the present document.

### 8.3.9	Non-3GPP Message Server

The Non-3GPP message server provides server-side functionality for Non-3GPP messaging service.

NOTE:	The details of the Non-3GPP message server are out of scope of the present document.

### 8.3.10	SEAL client

The following SEAL clients for MSGin5G Service are supported:

-	Group management client as specified in 3GPP TS 23.434 [9];

-	Configuration management client as specified in 3GPP TS 23.434 [9].

### 8.3.11	SEAL server

The following SEAL servers for MSGin5G Service are supported:

-	Group management server as specified in 3GPP TS 23.434 [9];

-	Configuration management server as specified in 3GPP TS 23.434 [9].

Editor's note: Usage of other SEAL services (e.g. location) from 3GPP TS 23.434 [9] for 5GMSGS Service is FFS.

## 8.4	Reference Points

### 8.4.1	General

The reference points of the service architecture for the MSGin5G Service are described in this clause.

### 8.4.2	MSGin5G-1

The interactions related to enabling MSGin5G message exchange between a 5GMSGS client and a MSGin5G Server are supported by the MSGin5G-1 reference point. This reference point supports:

-	Registration of a 5GMSGS client to a MSGin5G Server when not using IMS base solution; and

-	The exchange of MSGin5G messages.

### 8.4.3	MSGin5G-2

The interactions related to enabling MSGin5G message exchange between a MSGin5G Server and the Legacy 3GPP Message Gateway are supported by the MSGin5G-2 reference point. This reference point supports:

-

-	indicating the underlying message delivery mechanism to the gateway;

-	exchange of MSGin5G messages; and

-	registration of Legacy 3GPP Message Gateway to MSGin5G Server.

### 8.4.4	MSGin5G-3

The interactions related to enabling MSGin5G message exchange between an Application Server and a MSGin5G Server are supported by the MSGin5G-3 reference point. This reference point supports:

-	Access to MSGin5G Server and APIs to enable sending and receiving of MSGin5G messages; and

-	Adherence to CAPIF as specified in 3GPP TS 23.222 [13].

### 8.4.5	MSGin5G-4

The interactions related to enabling MSGin5G message exchange between a Non-3GPP Message Gateway and a MSGin5G Server are supported by the MSGin5G-4 reference point. This reference point supports:

-	Registration of Non-3GPP Message Gateway to MSGin5G Server; and

-	The exchange of MSGin5G messages.

### 8.4.6	MSGin5G-5

The interactions related to enabling MSGin5G message related information exchange between an application client and a 5GMSGS client are supported by the MSGin5G-5 reference point. This reference point supports:

-	Providing information from application clients required to enable the 5GMSGS client to construct a MSGin5G message to be delivered to other MSGin5G service endpoints.

-	Configuring application clients with information required to enable the 5GMSGS client and MSGin5G Server to exchange and route MSGin5G messages to other MSGin5G service endpoints.

-	Sending notifications and information in the incoming MSGin5G messages received by the 5GMSGS client to the application clients from other MSGin5G service endpoints.

### 8.4.7	Void

8.4.8	Void

8.4.9	SEAL-C

The following SEAL-C reference points for MSGin5G Service are supported:

-	GM-C reference point for group management as specified in 3GPP TS 23.434 [9];

-	CM-C reference point for configuration management as specified in 3GPP TS 23.434 [9].

### 8.4.10	SEAL-S

The following SEAL-S reference points for MSGin5G Service are supported:

-	GM-S reference point for group management as specified in 3GPP TS 23.434 [9];

-	CM-S reference point for configuration management as specified in 3GPP TS 23.434 [9].

### 8.4.11	SEAL-UU

The following SEAL-UU reference points for MSGin5G Service are supported:

-	GM-UU reference point for group management as specified in 3GPP TS 23.434 [9];

-	CM-UU reference point for configuration management as specified in 3GPP TS 23.434 [9].

* * * Next Change * * * *