3GPP TSG-SA WG6 Meeting #40-e	S6-202168

e-meeting, 16th – 24th November 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	Target EAS discovery

Spec:	3GPP TS 23.558 v1.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

The contribution generalizes the 'Discover target Edge Application Server' procedure for use in multiple scenarios.

2. Reason for Change

Currently the Source EAS decided application context relocation scenario and the Source EES executed application context relocation scenario use the procedure defined in clause 8.8.3.2 for target EAS discovery. However, the description of clause 8.8.3.2 is tailored specifically for the Source EAS decided application context relocation scenario and therefore creates confusion when read in context of Source EES executed application context relocation scenario.

Therefore the contribution generalizes the clause 8.8.3.2 to fit both the scenarios while updating the Source EAS decided application context relocation scenario to take care of the specifics.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.1.0.

* * * First Change * * * *

#### 8.8.3.2	Discover target Edge Application Server

Figure 8.8.3.2-1 illustrates the interactions between the source Edge Enabler Server, Edge Configuration Server and the target Edge Enabler Server(s) to discover target Edge Application Server information. This procedure can be invoked by the source Edge Application Serer or the source Edge Enabler Sever.

Editor's Note:	How Edge Enabler Server is selected from the list of Edge Enabler servers and whether the EEC needs to be notified when this selection happens is for further study.

Pre-conditions:

1.	Source Edge Enabler Server has received a request from an authorized source Edge Application Server to discover target Edge Application Server; or

2.	The Edge Enabler Server has decided that the application context relocation is needed.

Figure 8.8.3.2-1: Discover target Edge Application Server

1.	The source Edge Application Server or the source Edge Enabler Server invokes the Edge Application Server Discovery request on the source Edge Enabler Server. If the request is invoked by the source Edge Application Server, the Edge Application Server discovery request includes the requestor identifier [EAS ID] along with the security credentials and includes EAS discovery filter matching its Edge application Server profile. If target DNAI is available at the source Edge Application Server via UP path management event notification, the source Edge Application Server provides the source Edge Enabler Server with the target DNAI.

NOTE:	The trigger condition to invoke the Discover target Edge Application Server API is up to application service logic, which is out of scope of this specification.

2.	If the received request is from the source Edge Application Server, the source Edge Enabler Server checks if the requesting Edge Application Server is authorized to perform the discovery operation. If it is authorized, the source Edge Enabler Server checks if there exists a target Edge Application Server information (registered or cached) that can satisfy the requesting Edge Application Server information and additional query filters. If the source Edge Enable Server discovers the target Edge Application Server(s), the flow continues with step 5, else the source Edge Enabler Server retrieves the target Edge Enabler Server address(es) from the Edge Configuration Server as specified in clause 8.8.3.3.

3.	The source Edge Enabler Server sends the Edge Application Server Discovery request to the target Edge Enabler Server(s). The source Edge Enabler Server discovery request includes the requestor identifier [EES ID] along with the security credentials and may include EAS discovery filters.

4.	The target Edge Enabler Server responds with the target Edge Application Server information to the source Edge Enabler Server. The source Edge Enabler Server may cache the target Edge Application Server information.

Editor's Note:	How the target Edge Application Server is selected from the list of Edge Application Servers received in step 4 is for further study.

5.	The source Edge Enabler Server responds to the source Edge Application Server with the target Edge Application Server Information.