3GPP TSG-SA WG6 Meeting #39-bis-e	S6-202087 Rev1

e-meeting, 12th – 20th October 2020	(revision of S6-20xxxx)

Source:	China Mobile

Title:	Pseudo-CR on FS_5GMARCH overall Evaluation

Spec:	3GPP TR 23.700-24 v1.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

This document updates the overall Evaluation section based on approved Tdocs in the last meeting (i.e. SA6#39 BIS-e)..

- 2. Reason for Change

This document updates the overall Evaluation section based on approved Tdocs in the last meeting (i.e. SA6#39 BIS-e).

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-24 v1.1.0.

* * * First Change * * * *

# 9	Overall Evaluation

This Technical Report describes the architecture and high-level stage-2 procedure alternatives for MSGin5G service. In this document, 26 Key Issues are derived from stage 1 requirements and technical analysis. 14 of them are addressed by solutions. The related information are shown in Table 9-1.

Editor's note:	The data of KIs and solutions is based on the approved until SA6#39 bis-e. When more KIs and Solutions are finished, this table is needed to be updated accordingly.

Table 9-1: Key Issues and Solutions

| Key Issue | Stage 1 requirement | Solution |
| --- | --- | --- |
| Key Issue 1: support acknowledgement of delivery status | [R-5.1.2-004] | Solution 14: Acknowledgement of Delivery Status (NOTE: Several other solutions related with message delivery contains the solution of Delivery Status report) Solution 26: Aggregated delivery report (Used for Group Message status acknowledgement) |
| Key Issue 2: Cell Broadcast Service in MSGin5GS | [R-5.5.2-002] | Solution 1: Payload size of 2048 octets in Broadcast Solution 7: MSGin5G Server message broadcast |
| Key Issue 3: identifier(s) of the UE | [R-5.2.2-002] [R-5.2.2-003] | Solution 2: Registering 5GMSGS Client to MSGin5G Server Solution 8: Identities Solution 11 (Cover client registration enhancement for Application-to-point messaging in 6.11.1.2 and Application Server registration in 6.11.1.3) |
| Key Issue 4: network triggering the UE | [R-5.1.2-006] | Solution 3: MSGin5G Triggering |
| Key Issue 5: Message transfer from 5GMSGS UE and Non-3GPP UE to MSGin5G Server | [R-5.1.2-007] | Solution 9: Point to point Messaging (covers message delivery from 5GMSGS UE to 3 types of UE). Solution 17: Aggregated messaging for small data messages (covers message delivery from 5GMSGS Client to 5GMSGS Client) Solution 15: Point to point Messaging (UE reply a message) (covers message reply from non-3GPP UE to 5GMSGS UE) Solution 22: MOAT Messaging (message reply from UE to App Server Solution 24: MSGin5G message delivery based on messaging topic (generic solution that can be used for all four messaging scenarios) |
| Key Issue 6: Message delivery from MSGin5G Server to Non-3GPP UE | [R-5.1.2-007] | Solution 5: 5GMSGS Group messaging Solution 9: Point to point Messaging Solution 10: Application to point Message Exchange |
| Key Issue 7: group message | [R-5.4.2-001] [R-8.2-001] [R-9.2-001] [R-5.1.2-007] [R-10.2-001] | Solution 5: 5GMSGS Group messaging Solution 17: Aggregated messaging for small data messages(see 6.17.4) Solution 24: MSGin5G message delivery based on messaging topic (generic solution that can be used for all four messaging scenarios) |
| Key Issue 8: group management | [R-5.4.2-002] | Solution 4: Group management using SEAL (establishing a group, adding UEs to the group or removing UEs from the group, configuration of a maximum number of members in a group, group notification interworking, group list fetch) |
| Key Issue 9: Application-to-point message exchange | [R-5.3.2-001] [R-5.3.2-002] [R-10.2-001] | Solution 10: Application to point Message Exchange (Covers message delivery from Application Server to 3 types of UEs) Solution 11: MSGin5G Application-to-point message exchange (Covers message delivery from Application Server to 5GMSGS UE) Solution 17: Aggregated messaging for small data messages Solution 13: MSGin5G Message Segmentation and Reassembly Solution 22: MOAT Messaging (Covers message delivery from 3 types of UEs to 3 Application Server) Solution 23: APIs provided by MSGin5G server (Addresses interface between MSGin5G Server and Application Server) Solution 24: MSGin5G message delivery based on messaging topic (generic solution that can be used for all four messaging scenarios) |
| Key Issue 10: broadcast area configuration | [R-5.5.2-002] | Solution 16: Use of Broadcast Area configuration in MSGin5G message broadcast |
| Key Issue 11: mobility management of MSGin5G related information | [R-5.1.2-008] |  |
| Key Issue 12: End-to-end latency | [R-5.1.2-001] [R-5.5.2-001] [R-5.3.2-003] |  |
| Key Issue 13: Message store and forward | [R-5.1.2-005] | Solution 21: Store-and-forwarding of MSGin5G messages |
| Key Issue 14: Message Segmentation and Reassembly | [R-5.1.2-002] | Solution 12: Message segmentation and reassembly Solution 13: MSGin5G Message Segmentation and Reassembly |
| Key Issue 15: Message delivery from MSGin5G Server |  |  |
| Key Issue 16: Charging | [R-6.2-001] [R-6.2-002] | Solution 6: Charging NOTE: no content, will be handled by SA5 |
| Key Issue 17: Interconnection | [R-9.2-001] [R-9.2-002] | Solution 18: Interconnection |
| Key Issue 18: Roaming | [R-8.2-001] | Solution 25: Roaming |
| Key Issue 19: Efficient resource usage of the control plane and user plane | generic requirement e.g. section 4.1 | Solution 17: Aggregated messaging for small data messages |
| Key Issue 20: Obtain Contact Information |  | Solution 24: MSGin5G message delivery based on messaging topic (Covers the scenario that the contact information of the other party/parties is/are unknown) (gaps partial addressed) |
| Key Issue 21: Study if IMS is feasible as a solution |  |  |
| Key Issue 22: MSGin5G Service Exposure of multiple delivery mechanisms to the applications |  | Solution 20: Support of multiple delivery mechanisms for legacy 3GPP UEs |
| Key Issue 23: Message delivery from Legacy 3GPP UE | [R-10.2-001] | Solution 15: Point to point Messaging (UE reply a message) (see 6.15.1.2) Solution 22: MOAT Messaging (Legacy-3GPP UE replying a message to Application Server) |
| Key Issue 24: Capabilities of UE without MSGin5G client |  | No Solution is needed. |
| Key Issue 25: deliver message to Application Client | [R-5.1.2-003] | Solution 13: MSGin5G Message Segmentation and Reassembly (Some part of this KI is covered) Solution 17: Aggregated messaging for small data messages (Some part of this KI is covered) |
| Key issue 26 – Capability exposure to Application Server using CAPIF | [R-5.3.2-002] | Solution 19: MSGin5G application enabler layer adaptation to CAPIF |

NOTE: all requirements specified in 3GPP TS 22.262 [2] have been addressed in the present document.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>