3GPP TSG-SA WG6 Meeting #40-e	S6-202207

e-meeting, 16th– 24th November 2020	(revision of S6-20xxxx)

Title:	DNAI clarification and enhancements

Response to:

Release:	Rel.17

Work Item:	EDGEAPP

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG2

Cc:

Contact Person:

#### Name:	Cuili Ge

Tel. Number:

E-mail Address:	gecuili@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

In the context of the EDGEAPP work item, SA6 identifies some issues when using the target DNAI in the UP path management event notification for the application relocation:

Issue#1:

When the AF request is for subscription to notifications about UP path management events, the notification provides the target DNAI. It is possible that the target DNAI provided in notificaiton may not have any associated AF. Currently, in the notification there is no indication whether an AF is available at the target DNAI.

This API behaviour causes the following issue:

- If requesting AF cannot determine whether there is an AF available or can be instantiated in the target DNAI, then a notification with just target DNAI is not very useful for the requesting AF for further processing.

Issue#2:

According to SA2 specifications, different AFs may be assotiated with different set of DNAI(s).

When the AF request is for subscription to notifications about UP path management events, the notification provides a single target DNAI.

This API behaviour causes the following issue:

- When the target DNAI is received by the requesting AF it is possible that the AF associated with the target DNAI is not suitable for serving the UE, due to e.g., capability or load..

- The AFs associated with other DNAIs may be useful to the requesting AF and hence by not receiving all the target DNAIs, the requesting AF cannot select those AFs to serve the UE..

So SA6 would like to ask SA2 the following questions:

Question a): Is SA6’s understanding correct about API behaviour listed in the above issues - Issue#1 and Issue#2?

If answer to question (a) is yes, then

Question b): To resolve Issue#1, will SA2 consider to enhance the UP path management subscription mechanism to allow the AF to provide more details in terms of event filters related the target DNAI (e.g. to notify only when the target DNAI has an associated application deployment)?

Question c): To resolve Issue#2, will SA2 consider to enhance the UP path management event notification mechanism to include multiple target DNAI(s) close to the UE?

2. Actions:

To SA2 group.

ACTION: 	SA6 kindly asks SA2 to provide answers to above questions.

3. Date of Next SA6 Meetings:

SA6#41-e   18th – 22nd January 2021 	e-meeting

SA6#42-e   1st – 5th March 2021 	e-meeting