3GPP TSG-SA WG6 Meeting #40-e	S6-20xxxx

e-meeting, 16th – 24th November 2020	(revision of S6-201987, 2187, 2314)

Source:	Huawei, Hisilicon, CATT, China Mobile, TD Tech

Title:	Automatic ACR operations at edge enabler layer

Spec:	3GPP TS 23.558 v1.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Cuili Ge (gecuili@huawei.com), Niranth (namogh@huawei.com)

1. Introduction

This contribution provides a proposal for automatic ACR operations at edge enabler layer.

2. Reason for Change

Some EAS(s) may not require themselves to involve in finer control of the ACR procedures and will take support of the Edge Enabler Layer to perform ACR operations as an automatic operation.

Such EAS(s) will provide their ACR requirements to the Edge Enabler Server and will rely on Edge Enabler Server to perform all ACR related operations including detection, decision and execution. This enables them to utilize the Edge Enabler Layer functionality of Service Continuity with minimal changes or even no changes (if EAS already relies on such mechanism in cloud deployments).

The automatic ACR service provided by the Edge Enabler Server supports the following operations:

- ACR detection

- ACR decision

- Target EAS discovery

- Application context transfer

- Provide application relocation information to 3GPP CN

- Notifying the UE about completion of ACR and trigger switching of AC traffic to target EAS

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.1.0.

* * * First Change * * * *

#### 8.8.2.x	EES executed automatic application context relocation delegated by EAS

Figure 8.8.2.x-1 illustrates the procedure for the Edge Application Server to delegate to the Edge Enabler Server to execute the application context relocation operations as an automatic operation.

When EAS delegates to EES for the execution of automatic ACR, the source EES performs all the service operations of the application context relocation including detecting the event which may trigger the application context relocation,  making the application context relocation decision, discovering the target EAS, accessing and transferring the application context to the target Edge Enabler Server/ Edge Application Server, notifying the 3GPP network about application context relocation information, notifying the EEC about the target EAS information.

Pre-conditions:

1.	Information related to the source Edge Enabler Server is available with the source Edge Application Server.

2.	The information of the automatic application context relocation delegation service of the source Edge Enabler Server is available to the source EAS, e.g. via service API discovery of CAPIF or pre-configured.

3.	The EES has been provided with the access to the application context.

NOTE 1:	How the EES accesses the application context is up to implementation and outside the scope of the present document.

Figure 8.8.2.x-1: EES executed automatic application context relocation delegated by EAS

Phase I: Detection

1. The source Edge Application Server sends an automatic application context relocation delegation request to request the source Edge Enabler Server to handle all the service operations of the application context relocation for the Application Client. The EAS may send this request e.g. when Application Client is connecting to the source Edge Application Server.

2.	The source Edge Enabler Server checks if the requesting Edge Application Server is authorized to perform the operation. If it is authorized, the source Edge Enabler Server responds with an automatic application context relocation delegation response.

3.	The source Edge Enabler Server monitors the events which may trigger the application context relocation. The source Edge Enabler Server may subscribe to the UP path management event notification, UE location from the 3GPP core network as specified in clause 8.9.3. The Edge Enabler Server may also subscribe to the Edge Application Server load from the source Edge Application Server or from the Edge Application Server management system. The Edge Enabler Server may also receive the detections from the EEC as described in clause 8.8.1.

Editor's note: Whether additional EEC context information is needed by the EES to be able to monitor the events for triggering application context relocation is FFS.

Phase II: Decision

4.	Based on the event monitoring in step 3, the source Edge Enabler Server further decides that the application context relocation is required.

Phase III: Execution

5.	The source Edge Enabler Server discovers the target Edge Application Server(s) as specified in clause 8.8.3.2 and clause 8.8.3.3. The source Edge Enabler Server selects the target Edge Application Server which will serve the Application Client.

6.	The source EES and target EES facilitate application context transfer from source EAS to target EAS (obtained as per step 5) in a secure way.

Editor's Note:	How application context transfer via EEL (e.g. when to start / stop/ pause application context and when to re-direct application traffic without explicit interaction between s-EAS and s-EES) is achieved is FFS.

NOTE 2:	The details of application context and the encryption are outside the scope of the present document. The details of security aspects for application context transfer is specified by SA3.

NOTE 3:	The source Edge Enabler Server and the target Edge Enabler Server are not aware of the content of the application context and the application context is transferred as a container/payload.

Editor's note:	Whether the application context transfer via EEL has negative impact on the application layer is FFS.

Phase IV: Clean up

7.	All required entities perform clean-up including the source Edge Enabler Server notifies the Edge Enabler Client about the target EAS and the application context relocation completion. Also the source Edge Enabler Server may apply the AF traffic influence as specified in 3GPP TS 23.502 [3].

* * * Next Change * * * *

#### 8.8.3.z1	Automatic application context relocation delegation request

Table 8.8.3.z1-1 describes information elements for the automatic application context relocation delegation request from the Edge Application Server to the Edge Enabler Server.

Table 8.8.3.z1-1: Automatic application context relocation delegation request

| Information element | Status | Description |
| --- | --- | --- |
| UE identifier | M | Identifies the UE for which application context relocation is requested |
| EAS discovery filters | O | Set of characteristics to determine required Edge Application Servers, as detailed in Table 8.5.3.1-2. |
| Security credentials | M | Security credentials of the source Edge Application Server |

#### 8.8.3.z2	Automatic application context relocation delegation response

Table 8.8.3.z2-1 describes information elements for the automatic application context relocation delegation response from the Edge Enabler Server to the Edge Application Server.

Table 8.8.3.z2-1: Automatic application context relocation delegation response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of request failure |

#### 8.8.3.z3	Application context relocation event notification (EES – EEC)

Table 8.8.3.z3-1 describes information elements for the application context relocation event notification from the Edge Enabler Server to the Edge Enabler Client.

Table 8.8.3.z3-1: Application context relocation event notification

| Information element | Status | Description |
| --- | --- | --- |
| ACR completion event | M | Indicates that the subscription event which triggers the notification. In this case it is indicates that ACR to a target EAS has been completed. |
| Target EAS endpoint | M | Indicates the endpoint of the target EAS selected during the ACR |

* * * End of Changes * * * *