3GPP TSG-SA WG6 Meeting #41-e	S6-210137

e-meeting, 18th – 26th January 2021

Source:	Huawei, Hisilicon

Title:	Simplification on expression of EAS discovery notification trigger

Spec:	3GPP TS 23.558 v1.2.0

Agenda item:	7.6

Document for:	Approval

Contact:	Cuili Ge (gecuili@huawei.com)

1. Introduction

The contribution introduces clarification on the simplification on expression of EAS discovery notification trigger.

2. Reason for Change

In TS 23.558 clause 8.5.3.3, “EAS availability change” is specified as the only bullet in “EAS discovery notification triggers”, which is mandatory in EAS discovery request. This means “EAS availability change” should be mandatorily included in “EAS discovery notification triggers”, and current expression should be simplified for better understanding.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.2.0.

* * * First Change * * * *

#### 8.3.2.2	Edge Configuration Server discovery

#### 8.5.3.3	EAS discovery subscription request

Table 8.5.3.3-1 describes the information elements for EAS discovery subscription request from the Edge Enabler Client to the Edge Enabler Server.

Table 8.5.3.3-1: EAS discovery subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required Edge Application Servers, as detailed in Table 8.5.3.1-2. |
| EAS discovery notification triggers | M | EEC provided notification triggers for EAS discovery notification, as described in Table 8.5.3.3-2. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

Table 8.5.3.3-2: EAS discovery notification triggers

| Information element | Status | Description |
| --- | --- | --- |
| EAS availability change | O | Change in availability of an EAS for e.g. registration of a new EAS, de-registration of an existing EAS |

* * * End of Change * * * *