3GPP TSG-SA WG6 Meeting #41-e	S6-210167r2

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

Source:	Deutsche Telekom, AT&T

Title:	Pseudo-CR on Service-based interfaces interaction within EDGEAPP

Spec:	3GPP TS 23.558 v 1.2.0

Agenda item:	7.6

Document for:	Approval

Contact:	Vasil dot Aleksiev at magenta dot at

1. Introduction

SA6 is specifying the application layer architecture, procedures and information flows necessary for enabling edge applications (EDGEAPP) over 3GPP networks in 3GPP Rel-17 TS 23.558. Interactions between different entities are presented in reference point representation only. SA6 has already made the decision that some of the reference points shall be exposed as APIs, however currently there is not clear representation of the Service based architecture EDGEAPP spec. EDGEAPP entities can be provided from different vendors and hence better interoperability, scalability would result using open APIs.

2. Reason for Change

Clearly identify SBA is used in EDGEAPP where different functional entities (e.g. EES, ECS, EEC) could easily be deployed from different vendors. Proper scalability and stateless priciples could be supported.

Clarly pointing out the usage of SBA interactions amongst the functional entities in EDGEAPP architecture will help to show that the specified EDGEAPP is an open architecture with posibility for multi-vendor environment enabling better scalability and stateless operations.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 version 17.3.0.

* * * First Change * * * *

## 6.2	Architecture

The Figure 6.2-1 shows the architecture for enabling edge applications. The architecture is defined as service-based and interactions between the identified functional entities are represented in two ways:

-	A service-based representation, where functions (e.g. ECS) enable other authorized functions (e.g. EES) to access their services;

-	A reference point representation, where existing interactions between any two functions (e.g. EES, ECS) is shown by an appropriate point-to-point reference point (e.g. EDGE-6, EDGE-7) – figure 6.2-2.

Entities in the edge architecture shall only use service-based interfaces for their interactions.

Figure 6.2-1: Architecture for enabling edge applications with service-based representation

For the list of APIs exposed by NEF and SCEF see 29.522 and 29.122 respectively.

Table 6.2-1 summarizes the APIs exposed by the ECS.

Table 6.2-1: APIs provided by the ECS

| API Name | References |
| --- | --- |
| Eecs_ServiceProvisioning | 8.3 |
| Eecs_EESRegistration | 8.4.4 |

Table 6.2-2 summarizes the APIs exposed the EES.

Table 6.2-2: APIs provided by the EES

| API Name | References |
| --- | --- |
| Eees_EECRegistration | 8.4.2 |
| Eees_EASRegistration | 8.4.3 |
| Eees_EASDiscovery | 8.5 |
| Eees_UELocation | 8.6.2 |
| Eees_UPPathManagementEvent | 8.6.3 |
| Eees_AppClientInformation | 8.6.4 |
| Eees_UEIdentifier | 8.6.5 |
| Eees_SessionWithQoS | 8.6.6 |
| Eees_TargetEASDiscovery | 8.8.3.2 |

Figure 6.2-2: Architecture for enabling edge applications with reference points

The Edge Data Network is a local Data Network. Edge Application Server(s) and the Edge Enabler Server are contained within the EDN. The Edge Configuration Server provides configurations related to the EES, including details of the Edge Data Network hosting the EES. The UE contains Application Client(s) and the Edge Enabler Client. The Edge Application Server(s), the Edge Enabler Server and the Edge Configuration Server may interact with the 3GPP Core Network.

* * * Next Change * * * *

## 6.6	Capability exposure for enabling edge applications

Capability exposure includes the 3GPP core network (i.e. 5GC, EPC) , Edge Configuration Server and the Edge Enabler Server capability exposure, to fulfil the needs of the edge service operations. The capability exposure functionality is utilized by the functional entities (i.e. Edge Enabler Server, Edge Application Server and Edge Configuration Server) depicted in the architecture for enabling the edge applications.

: APIs provided by the ECS

: APIs provided by the EES