3GPP TSG-SA WG6 Meeting #41-e	S6-210xxx

e-meeting, 18th – 26th January 2021	(revision of S6-210084)

Source:	Ericsson

Title:	ACR API

Spec:	3GPP TS 23.558 v1.2.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution adds the Eees_ACR API.

2. Reason for Change

Over EDGE-1, the EEC can request EES to handle application context relocation request. However, such API is not defined yet. It is proposed to define such API in service continuity clause.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

## 6.6	Capability exposure for enabling edge applications

The Figure 6.6-1 shows the capability exposure for enabling edge applications.

Figure 6.6-1: Capability exposure for enabling edge applications

Capability exposure includes the 3GPP core network (i.e. 5GC, EPC) , Edge Configuration Server and the Edge Enabler Server capability exposure, to fulfil the needs of the edge service operations. The capability exposure functionality is utilized by the functional entities (i.e. Edge Enabler Server, Edge Application Server and Edge Configuration Server) depicted in the architecture for enabling the edge applications.

Table 6.6-1 summarizes the APIs exposed by the ECS.

Table 6.6-1: APIs provided by the ECS

| API Name | References |
| --- | --- |
| Eecs_ServiceProvisioning | 8.3 |
| Eecs_EESRegistration | 8.4.4 |

Table 6.6-2 summarizes the APIs exposed the EES.

Table 6.6-2: APIs provided by the EES

| API Name | References |
| --- | --- |
| Eees_EECRegistration | 8.4.2 |
| Eees_EASRegistration | 8.4.3 |
| Eees_EASDiscovery | 8.5 |
| Eees_UELocation | 8.6.2 |
| Eees_UPPathManagementEvent | 8.6.3 |
| Eees_AppClientInformation | 8.6.4 |
| Eees_UEIdentifier | 8.6.5 |
| Eees_SessionWithQoS | 8.6.6 |
| Eees_TargetEASDiscovery | 8.8.3.2 |
| Eees_AppContextRelocation | 8.8.3.4 |

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the API for fetch target Edge Application Server.

Table 8.8.5.1-1: Eees_TargetEASDiscovery API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS |

Table 8.8.5.1-y illustrates the API for application context relocation.

Table 8.8.5.1-y: Eees_AppContextRelocation API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_AppContextRelocation | Request | Request/Response | EEC |

* * * Next Change * * * *

#### 8.8.5.y	Eees_AppContextRelocation_Request operation

API operation name: Eees_AppContextRelocation_Request

Description: The EEC requests to initiate the application context relocation procedure.

Inputs: See clause 8.8.4.2.

Outputs: See clause 8.8.4.3.

See clause 8.8.3.4 for details of usage of this operation.

* * * End of Changes * * * *