3GPP TSG-SA WG6 Meeting #41-e	S6-210xxx

e-meeting, 18th – 26th January 2021	(revision of S6-210086)

Source:	Ericsson

Title:	EAS discovery and Service provisioning

Spec:	3GPP TS 23.558 v1.2.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution completes the service provisioning (i.e. EES info retrieval) API over EDGE-6 and converges Eees_EASDiscovery and Eees_targetEASDiscovery into one API.

2. Reason for Change

Over EDGE-6, the source EES can ask ECS to provide candidate EES information in order to perform further EAS disocovery. However, such API is not defined yet. It is proposed to define such API in service continuity clause with consumer EES and clarify that EDN info is not applicable for consumer EES.

The EDN (EES or EAS) triggered EAS discovery can be converged with EEC triggered EAS discovery. Even though some input data differences may exist, stage 3 will take care of the API design details including information applicability. It is also proposed to merge Eees_EASDiscovery and Eees_targetEASDiscovery into one API.

Rev1: stage 3 can decide whether the SA6 specified APIs for “EAS discovery” function should be merged into one API or not.

The same applies for the “service provisioning” function.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

#### 8.8.4.x	Service provisioning request

Table 8.8.4.x-1 describes the information elements for service provisioning request from the Edge Enabler Server to the Edge Configuration Server.

Table 8.8.4.x-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | Unique identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS ID | M | The EAS ID. |
| target DNAI | O | The target DNAI to identify the user plane access to the EDN where application servers can be deployed. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE location | O | The location information of the UE. The UE location is described as clause 7.3.2. |

#### 8.8.4.y	Service provisioning response

The information elements specified in the Table 8.3.3.2-1 is used for the Service provisioning response sent from the Edge Configuration Server to the Edge Enabler Server with the following differences:

-	The List of EDN configuration information is not applicable.

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the API for fetch target Edge Application Server.

Table 8.8.5.1-1: Eees_TargetEASDiscovery API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS, EES |

Table 8.8.5.1-x illustrates the API for service provisioning of the Edge Enabler Server information maintained in the Edge Configuration Server.

Table 8.8.5.1-x: Eecs_ServiceProvisioning API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs_ServiceProvisioning | Request | Request/Response | EES |

* * * Next Change * * * *

#### 8.8.5.x	Eecs_ServiceProvisioning_Request operation

API operation name: Eecs_ServiceProvisioning_Request

Description: The consumer requests for one-time service provisioning about the target Edge Enabler Server information.

Inputs: See clause 8.5.3.x.

Outputs: See clause 8.5.3.y.

See clause 8.8.3.3 for details of usage of this operation.

* * * End of Changes * * * *