3GPP TSG-SA WG6 Meeting #42-bis-e	S6-21xxxx

e-meeting, 12th – 20th April 2021	(revision of S6-210923)

Source:	Huawei, Hisilicon

Title:	Support for edge deployments

Spec:	3GPP TS 23.255 v0.3.0

Agenda item:	7.10

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

The contribution provides a proposal for support for edge deployments.

2. Reason for Change

The illustration related to UASAPP supporting edge deployments are required.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.255 v0.3.0.

* * * First Change * * * *

Annex Y (Normative):
Support for edge deployments

The application architecture for supporting edge applications are specified in 3GPP TS 23.558 [x]. The UAS application layer as specified in clause 5 may be deployed in edge computing environment.

Figure Y-1 illustrates a UAS application layer deployment in edge computing environments.

Figure Y-1: UAS application layer deployment in edge computing environment

In the deployment option represented in figure Y-1, within the UE, UAS application specific client(s), UAE client and SEAL clients interact with the Edge Enabler Client (EEC) via EDGE-5 reference point in order to consume edge services. In an Edge Data Network (EDN), the UAS application specific server, UAE server and SEAL servers acting as Edge Application Server (EAS), interacts with the Edge Enabler Server (EES) via EDGE-3 reference point. The service provisioning and EAS discovery enable the UAS application layer entities in the UE to communicate with the application layer entities in the EDN. The interactions between the entities and 5GS are not shown for the sake of simplicity.