3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210766

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	China Mobile

Title:	Delete the EN about ECS configuration information provisioning

Spec:	TS 23.558 v1.4.0

Agenda item:	7.6

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution propose to close the EN about ECS configuration information provisioning.

Editor's Note: [SA2] 5GC provision of ECS configuration information to the UE is in scope of SA2.

2. Reason for Change

S2-2101097 and S2-2101095 updated the“ECS address provisioning” section of the 5GS Edge Computing TS 23.548. S2-2101094 and S2-2101096  updated the related procedure of TS 23.502 to support ECS Address Information  provisioning.

Based on discussion result of above contributions, it has been approved in SA2 to introduce a new feature so that the network is able to send ECS Address Information to the UE, and the details can be found in the latest version of TS 23.548 v0.1.0.

3. Conclusions

The EN about 5GC provision of ECS configuration information to the UE is solved.

4. Proposal

It is proposed to agree the following changes to 3GPP in TS 23.558.

* * * First Change * * * *

### 8.3.2	ECS Discovery

#### 8.3.2.1	General

ECS configuration information consists of one or more URLs and/or IP Address(es) of ECS(s). ECS configuration information can be pre-configured with the EEC, configured by an edge-aware AC, configured by the user, provisioned by MNO through 5GC procedure if the UE has the capability to deliver the ECS configuration information to the EEC on the UE ( See TS 23.548 [xx], clause 6.5.2), or derived from HPLMN identifier for non-roaming scenario or from VPLMN identifier for roaming scenario.

NOTE:	How the ECS configuration information is configured to the EEC by the AC, user, or pre-configuration is out of scope of the present specification.

It may be possible to provide the ECS configuration information to the EEC from the 5GC if the UE has the capability to deliver the ECS configuration information to the EEC on the UE.

If the ECS configuration information is provided by 5GC and available at the EEC, the EEC shall use the information for the initial provisioning request. Otherwise, the EEC shall use pre-configured ECS address for the initial provisioning if ECS configuration information is preconfigured with the EEC.