3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210787_rev1

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	Samsung

Title:	Update on Retrieve EES request and response messages

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

This pCR is proposed to update description on Retrieve EES request and response messages.

2. Reason for Change

For Retrieve EES request message, the description on target DNAI needs to be revised to be aligned with EAS discovery request message in clause 8.8.4.2

Currently, description on IEs in Retrieve EES response message is as follows:

The information elements specified in the Table 8.3.3.9-1 is used as response for T-EES information retrieval with the following differences: - The List of EDN configuration information is not applicable.

The above description needs to be modified to include the T-EES profile and necessary information for UE to establish PDU Session toward the T-EDN as in clause 8.3.3.3.3 (service provisioning response message). Note that the retreived information for the T-EES will be provided to EEC via ACR information notification message.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0

* * * First Change * * * *

#### 8.8.4.6	Retrieve EES request

Table 8.8.4.6-1 describes the information elements to retrieve T-EES information from the ECS.

Table 8.8.4.6-1: Retrieve EES request

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | Unique identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | M | The EASID. |
| target DNAI | O | The target DNAI information which can be associated with potential T-EES(s) and/or T-EAS(s) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE location | O | The location information of the UE. The UE location is described as clause 7.3.2. |

* * * Next Change * * * *

#### 8.8.4.7	Retrieve EES response

The information elements specified in the Table 8.3.3.3.3-1 is used as response for T-EES information retrieval.

* * * End of Change * * * *