3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210799

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	NTT DOCOMO

Title:	Description of service areas

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Yuji Suzuki (yuuji.suzuki.rm@nttdocomo.com)

1. Introduction

This contribution proposes to modify the descriptions of EDN/EES/EAS service areas.

2. Reason for Change

In clauses 7.3.3.4-7.3.3.6, the EDN/EES/EAS service areas are now described in random styles. To improve readability, the descriptions are modified to be consistent.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * First Change * * * *

#### 7.3.3.4	EDN service area

A service area from which the access to the EDN is allowed.

ECSPs can use LADNs, as described in Annex A.2.4 of this document, to deploy EDNs with access restricted from specific areas. When an EDN is deployed using LADN, the EDN service area is same as the LADN service area and rules specified for LADN apply to the UE, as specified in 3GPP TS 23.501 [2].

In a deployment using DNs other than LADNs, the EDN service area is the whole PLMN for non-roaming scenario.

NOTE:	The EDN service area for roaming scenario is out of scope in this release of the specification.

NOTE: 	For the purpose of restricting the access to the EES from specific areas, ECSP can use the EES service area, which is specified in clause 7.3.3.5, even if the EDN service area is the whole PLMN.

The EDN service area may be expressed as a Topological Service Area.

#### 7.3.3.5	EES Service Area

A service area from which the access to the EES is allowed. This service area is equal to or a subset of the service area of the EDN in which the EES resides.

The EES service area may be expressed as a Topological Service Area, a Geographical Service Area or both.

#### 7.3.3.6	EAS service area

A service area from which the access to the EAS is allowed. This service area is equal to or a subset of the service area of the EES which serves the EAS.

The EAS service area may be expressed as a Topological Service Area, a Geographical Service Area or both.