3GPP TSG-SA WG6 Meeting #42-e	S6-210805

e-meeting, 1st – 9th March 2021	(revision of S6-210xxx)

Source:	Ericsson

Title:	E3 exposure context transfer

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution adds a E3 exposure context transfer btw. the EESes (triggered by T-EAS).

2. Reason for Change

As discussed in conf.call on April 1st, the EDGE-3 exposure context (created by consumer EAS interaction with EES and stored in the producer EES) may be transferred from the S-EES to the T-EES so that the T-EAS can utilize the existing exposure subscriptions in the T-EES.

As a valid option, if the T-EAS receives the corresponding correlation id from the S-EAS, it can request the T-EES by including the correlation id and then the T-EES can retrieve the EDGE-3 exposure context from the S-EES; otherwise, the T-EAS can re-subscribe the exposure subscriptions on the T-EES.

This is similar as “Inter NG-RAN node N2 based handover with I-SMF change” (see cl.4.23.7.3 of TS 23.502) where both AMF and I-SMF are changed and SM context is relocated to the target I-SMF based on target AMF provided “SM context ID”.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application Context: A set of data about the Application Client that resides in the Edge Application Server.

Application Context Relocation: Refers to the end-to-end service continuity procedure described in clause 8.8.

Application Context Transfer: Refers to the transfer of the Application Context between the source Edge Application Server and the target Edge Application Server, which is a part of the service continuity procedure described in clause 8.8.

Application Server: Application software resident in the cloud performing the server function.

Edge Computing: A concept, as described in 3GPP TS 23.501 [2], that enables operator and 3rd party services to be hosted close to the UE's access point of attachment, to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network.

Edge Computing Service Provider: A mobile network operator or a 3rd party service provider offering Edge Computing service.

Edge Data Network: A local Data Network that supports the architecture for enabling edge applications.

EEC Context: A set of data about the Edge Enabler Client that resides in the Edge Enabler Server.

Edge Enabler Layer: Refers to the overall functionality provided by the entities such as Edge Enabler Client, Edge Enabler Server and Edge Configuration Server, in support of applications as per the architecture defined in clause 6.

Edge Hosting Environment: An environment providing support required for Edge Application Server's execution.

EDGE-3 exposure context: A set of EDGE-3 exposure subscription data about the EAS that resides in the EES.

* * * Next Change * * * *

### 6.5.10	EDGE-9

EDGE-9 reference point enables interactions between two EESs. EDGE-9 reference point may be provided between EES within different EDN (Figure 6.5.10-1) and within the same EDN (Figure 6.5.10-2).

Figure 6.5.10-1: Inter-EDN EDGE-9

Figure 6.5.10-2: Intra-EDN EDGE-9

EDGE-9 supports:

a)	discovery of T-EAS information to support ACR;

b)	EEC context retrieval and relocation procedures; and

c)	EDGE-3 exposure context retrieval procedure.

* * * Next Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.6 |
| Eees_AutomatedACR | EAS | 8.8.3.7 |
| Eees_ContextRetrieval | EAS | 8.8.3.x |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

* * * Next Change * * * *

#### 8.8.2.4	S-EAS decided ACR scenario

The procedure in this clause illustrates the scenario for S-EAS decided ACR.

In this procedure, the S-EAS may detect the need of ACR locally or is notified by the S-EES. The S-EAS make the decision about whether to perform the ACR, and starts the ACR at a proper time.

Pre-condition:

1.	The S-EAS may depend on the receipt of certain User plane path management events from the S-EES, e.g. "user plane path change" events or "ACR monitoring" events, to detect the need for an ACR. For the following procedure it is assumed that the S-EAS has subscribed to continuously receive the respective events from the S-EES.

Figure 8.8.2.4-1: S-EAS decided ACR scenario

The S-EAS decided ACR scenario is outlined with four main phases: detection, decision, execution and clean up.

Phase I: ACR Detection

1.	The S-EAS either receives notifications from source Edge Enabler Sever indicating that ACR may be required ("ACR monitoring" event), or self detects the need for ACR (e.g. upon receipt of a "user plane path change" event). If the notification indicates "ACR monitoring" event, then the notification will also contain the T-EAS information (see clause 8.6.3.2.3).

NOTE 1:	How the S-EAS self detects the local need for ACR is outside the scope of this specification.

Phase II: ACR Decision

2.	The S-EAS makes the decision to perform the ACR

NOTE 2:	How the S-EAS determines when to start the ACR is outside the scope of this specification.

Phase III:	ACR Execution

3.	If no T-EAS information is available at the S-EAS, the S-EAS discovers the T-EAS as described in clause 8.8.3.2. If this procedure is used for service continuity planning, then UE Location and Target DNAI values in the Retrieve T-EES procedure contain the expected UE Location and expected Target DNAI. After S-EAS determines the T-EAS to use, the S-EAS may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable).

4.	The S-EES sends the target information notification to the EEC as described in clause 8.8.3.6.

5.	The S-EAS transfers the application context to the T-EAS selected in step 3. This process is out of scope of the present specification. After successful ACT, if T-EAS received EDGE-3 exposure context ID(s) from the S-EAS, the T-EAS may initiate the procedure for EDGE-3 exposure context retrieval as described in clause 8.8.3.x.

Editor's Note:	It is FFS whether the EDGE-3 exposure context retrieval procedure is applicable in other scenarios.

NOTE 3:	If used for service continuity planning, post-ACR clean up would only be performed after the UE moves to the expected location.

Phase IV:	Post-ACR clean up

6.	All required entities perform clean-up. The S-EAS informs the S-EES of the complete of ACT and the S-EES informs the EEC of the completion of ACR as described in clause 8.8.1.

* * * Next Change * * * *

#### 8.8.3.x	Retrieve EDGE-3 exposure context procedure

Figure 8.8.3.x-1 illustrates the procedure for EDGE-3 exposure context retrieval. T-EES retrieves the EDGE-3 exposure context data from the S-EES based on request sent by the T-EAS.

NOTE 1:	This procedure is suitable for the single UE exposure subscription over EDGE-3.

Pre-condition:

1.	The application context transfer is finished in the EAS.

Figure 8.8.3.x-1: EDGE-3 exposure context retrieval procedure

1.	The T-EAS sends Retrieve EDGE-3 exposure context request message to the T-EES.

2.	If the request is authorized by the T-EES, the T-EES sends the Retrieve EDGE-3 exposure context request to the S-EES, the request includes a list of EDGE-3 exposure context ID(s) received from step 1.

3.	If the request is authorized by the S-EES, the S-EES sends the Retrieve EDGE-3 exposure context response message to the T-EES. The context data is included in the response message.

4.	The T-EES responds with Retrieve EDGE-3 exposure context response message to the T-EAS. If EDGE-3 exposure context data is received from the S-EES, the T-EES stores the context data and may update the list of EDGE-3 exposure context ID(s). The T-EES also updates the Notifiation Target Address (if received) provided by the T-EAS for each context ID.

NOTE 2:	This context ID can be represented by subscription ID in the EDGE-3 interaction.

#### 8.8.4.x	Retrieve EDGE-3 exposure context request (from T-EAS to T-EES)

Table 8.8.4.x-1 describes the information elements for the Retrieve EDGE-3 exposure context request from T-EAS to T-EES.

Table 8.8.4.x-1: Retrieve EDGE-3 exposure context request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | O | Identifier of the UE |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| List of EDGE-3 exposure context ID(s) | M | A list of the EDGE-3 exposure context identifiers. |
| List of Notification Target Address(es) | O | Notification Target Address of the EAS where the notification is to be sent by the EES, for each context ID. |
| Source EES Endpoint | M | Source EES Endpoint information |

#### 8.8.4.y	Retrieve EDGE-3 exposure context response (from T-EES to T-EAS)

Table 8.8.4.y-1 describes the information elements for the Retrieve EDGE-3 exposure context response from T-EES to T-EAS.

Table 8.8.4.y-1: Retrieve EDGE-3 exposure context response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful |
| > List of updated EDGE-3 exposure context ID(s) | O | If the result is successful, the EES may provide the updated context identifier(s). The absence of an identifier implies no change for the context identifier. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of request failure |

#### 8.8.4.a	Retrieve EDGE-3 exposure context request (from T-EES to S-EES)

Table 8.8.4.a-1 describes the information elements for the Retrieve EDGE-3 exposure context request from T-EES to S-EES.

Table 8.8.4.a-1: Retrieve EDGE-3 exposure context request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | O | Identifier of the UE |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| List of EDGE-3 exposure context ID(s) | M | A list of the EDGE-3 exposure context identifiers. |

#### 8.8.4.b	Retrieve EDGE-3 exposure context response (from S-EES to T-EES)

Table 8.8.4.b-1 describes the information elements for the Retrieve EDGE-3 exposure context response from S-EES to T-EES.

Table 8.8.4.b-1: Retrieve EDGE-3 exposure context response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful |
| > EDGE-3 exposure context information | M | The context information of EDGE-3 exposure data. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of request failure |

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the APIs for ACR.

Table 8.8.5.1-1: ACR APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS, EES |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES |
| Eees_AppContextRelocation | Request | Request/Response | EEC |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_AutomatedACR | Request | Request/Response | EAS |
| Eees_ContextRetrieval | Request | Request/Response | EAS, EES |

* * * Next Change * * * *

#### 8.8.5.x	Eees_ContextRetrieval_Request operation

API operation name: Eees_ContextRetrieval_Request

Description: The consumer requests for the context retrieval.

Inputs: See clause 8.8.4.x, 8.8.4.a.

Outputs: See clause 8.8.4.y, 8.8.4.b.

See clause 8.8.3.x for details of usage of this operation.

* * * End of Changes * * * *