3GPP TSG-SA WG6 Meeting #42-e	S6-210805

e-meeting, 1st – 9th March 2021	(revision of S6-210xxx)

Source:	Ericsson

Title:	E3 exposure context transfer

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution adds a E3 exposure context transfer btw. the EESes (triggered by T-EAS).

2. Reason for Change

As discussed in conf.call on April 1st, the EDGE-3 exposure context (created by consumer EAS interaction with EES and stored in the producer EES) may be transferred from the S-EES to the T-EES so that the T-EAS can utilize the existing exposure subscriptions in the T-EES.

As a valid option, if the T-EAS receives the corresponding correlation id from the S-EAS, it can request the T-EES by including the correlation id and then the T-EES can retrieve the EDGE-3 exposure context from the S-EES; otherwise, the T-EAS can re-subscribe the exposure subscriptions on the T-EES.

This is similar as “Inter NG-RAN node N2 based handover with I-SMF change” (see cl.4.23.7.3 of TS 23.502) where both AMF and I-SMF are changed and SM context is relocated to the target I-SMF based on target AMF provided “SM context ID”.

the benefit is signalling reduction by re-establishing E3 context in the target side in one shot, instead of creating them one by one

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

### 6.5.10	EDGE-9

EDGE-9 reference point enables interactions between two EESs. EDGE-9 reference point may be provided between EES within different EDN (Figure 6.5.10-1) and within the same EDN (Figure 6.5.10-2).

Figure 6.5.10-1: Inter-EDN EDGE-9

Figure 6.5.10-2: Intra-EDN EDGE-9

EDGE-9 supports:

a)	discovery of T-EAS information to support ACR;

b)	EEC context retrieval and relocation procedures; and

c)	EDGE-3 exposure context retrieval procedure.

* * * Next Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.6 |
| Eees_AutomatedACR | EAS | 8.8.3.7 |
| Eees_ContextRetrieval | EAS | 8.8.3.x |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

* * * Next Change * * * *

#### 8.8.2.4	S-EAS decided ACR scenario

The procedure in this clause illustrates the scenario for S-EAS decided ACR.

In this procedure, the S-EAS may detect the need of ACR locally or is notified by the S-EES. The S-EAS make the decision about whether to perform the ACR, and starts the ACR at a proper time.

Pre-condition:

1.	The S-EAS may depend on the receipt of certain User plane path management events from the S-EES, e.g. "user plane path change" events or "ACR monitoring" events, to detect the need for an ACR. For the following procedure it is assumed that the S-EAS has subscribed to continuously receive the respective events from the S-EES.

Figure 8.8.2.4-1: S-EAS decided ACR scenario

The S-EAS decided ACR scenario is outlined with four main phases: detection, decision, execution and clean up.

Phase I: ACR Detection

1.	The S-EAS either receives notifications from source Edge Enabler Sever indicating that ACR may be required ("ACR monitoring" event), or self detects the need for ACR (e.g. upon receipt of a "user plane path change" event). If the notification indicates "ACR monitoring" event, then the notification will also contain the T-EAS information (see clause 8.6.3.2.3).

NOTE 1:	How the S-EAS self detects the local need for ACR is outside the scope of this specification.

Phase II: ACR Decision

2.	The S-EAS makes the decision to perform the ACR

NOTE 2:	How the S-EAS determines when to start the ACR is outside the scope of this specification.

Phase III:	ACR Execution

3.	If no T-EAS information is available at the S-EAS, the S-EAS discovers the T-EAS as described in clause 8.8.3.2. If this procedure is used for service continuity planning, then UE Location and Target DNAI values in the Retrieve T-EES procedure contain the expected UE Location and expected Target DNAI. After S-EAS determines the T-EAS to use, the S-EAS may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable).

4.	The S-EES sends the target information notification to the EEC as described in clause 8.8.3.6.

5.	The S-EAS transfers the application context to the T-EAS selected in step 3. This process is out of scope of the present specification. After successful ACT, if T-EAS received EDGE-3 exposure context ID(s) from the S-EAS, the T-EAS may initiate the procedure for EDGE-3 exposure context retrieval as described in clause 8.8.3.x.

NOTE x:	The EDGE-3 exposure context retrieval is applicable if T-EAS and S-EAS belong to the same ASP.

Editor's Note:	It is FFS whether the EDGE-3 exposure context retrieval procedure is applicable in other scenarios.

NOTE 3:	If used for service continuity planning, post-ACR clean up would only be performed after the UE moves to the expected location.

Phase IV:	Post-ACR clean up

6.	All required entities perform clean-up. The S-EAS informs the S-EES of the complete of ACT and the S-EES informs the EEC of the completion of ACR as described in clause 8.8.1.

* * * Next Change * * * *

#### 8.8.3.x	Retrieve EDGE-3 exposure context procedure

Figure 8.8.3.x-1 illustrates the procedure for EDGE-3 exposure context retrieval. T-EES retrieves the EDGE-3 exposure context data from the S-EES based on request sent by the T-EAS.

NOTE 1:	This procedure is suitable for the single UE exposure subscription over EDGE-3.

Pre-condition:

1.	The application context transfer is finished in the EAS.

Figure 8.8.3.x-1: EDGE-3 exposure context retrieval procedure

1.	The T-EAS sends Retrieve EDGE-3 exposure context request message to the T-EES.

2.	If the request is authorized by the T-EES, the T-EES sends the Retrieve EDGE-3 exposure context request to the S-EES, the request includes a list of EDGE-3 exposure context ID(s) received from step 1.

3.	If the request is authorized by the S-EES, the S-EES sends the Retrieve EDGE-3 exposure context response message to the T-EES. The context data is included in the response message.

4.	The T-EES responds with Retrieve EDGE-3 exposure context response message to the T-EAS. If EDGE-3 exposure context data is received from the S-EES, the T-EES stores the context data and may update the list of EDGE-3 exposure context ID(s). The T-EES also updates the Notifiation Target Address (if received) provided by the T-EAS for each context ID.

NOTE 2:	This context ID can be represented by subscription ID in the EDGE-3 interaction.

Editor's Note:	It is FFS whether a single EAS context ID instead of a list of context ID(s) can be used to optimize the signalling.

#### 8.8.4.x	Retrieve EDGE-3 exposure context request (from T-EAS to T-EES)

Table 8.8.4.x-1 describes the information elements for the Retrieve EDGE-3 exposure context request from T-EAS to T-EES.

Table 8.8.4.x-1: Retrieve EDGE-3 exposure context request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | O | Identifier of the UE |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| List of EDGE-3 exposure context ID(s) | M | A list of the EDGE-3 exposure context identifiers. |
| List of Notification Target Address(es) | O | Notification Target Address of the EAS where the notification is to be sent by the EES, for each context ID. |
| Source EES Endpoint | M | Source EES Endpoint information |

#### 8.8.4.y	Retrieve EDGE-3 exposure context response (from T-EES to T-EAS)

Table 8.8.4.y-1 describes the information elements for the Retrieve EDGE-3 exposure context response from T-EES to T-EAS.

Table 8.8.4.y-1: Retrieve EDGE-3 exposure context response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful |
| > List of updated EDGE-3 exposure context ID(s) | O | If the result is successful, the EES may provide the updated context identifier(s). The absence of an identifier implies no change for the context identifier. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of request failure |

#### 8.8.4.a	Retrieve EDGE-3 exposure context request (from T-EES to S-EES)

Table 8.8.4.a-1 describes the information elements for the Retrieve EDGE-3 exposure context request from T-EES to S-EES.

Table 8.8.4.a-1: Retrieve EDGE-3 exposure context request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | O | Identifier of the UE |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| List of EDGE-3 exposure context ID(s) | M | A list of the EDGE-3 exposure context identifiers. |

#### 8.8.4.b	Retrieve EDGE-3 exposure context response (from S-EES to T-EES)

Table 8.8.4.b-1 describes the information elements for the Retrieve EDGE-3 exposure context response from S-EES to T-EES.

Table 8.8.4.b-1: Retrieve EDGE-3 exposure context response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful |
| > EDGE-3 exposure context information | M | The context information of EDGE-3 exposure data. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of request failure |

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the APIs for ACR.

Table 8.8.5.1-1: ACR APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS, EES |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES |
| Eees_AppContextRelocation | Request | Request/Response | EEC |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_AutomatedACR | Request | Request/Response | EAS |
| Eees_ContextRetrieval | Request | Request/Response | EAS, EES |

* * * Next Change * * * *

#### 8.8.5.x	Eees_ContextRetrieval_Request operation

API operation name: Eees_ContextRetrieval_Request

Description: The consumer requests for the context retrieval.

Inputs: See clause 8.8.4.x, 8.8.4.a.

Outputs: See clause 8.8.4.y, 8.8.4.b.

See clause 8.8.3.x for details of usage of this operation.

* * * End of Changes * * * *