3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210806

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	Ericsson

Title:	EEC registration clarification

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution introduces some clarifications for the EEC registration.

2. Reason for Change

To simplify the procedure (esp. in ACR, to avoid over-engineering for EEC context relocation in different modes), it is proposed that the EEC registration is not a precondition of using EC service for EEC as there is no specific service really depending on registration, but the EEC registration can  offer AC information for EES that can be used in EC service.

By that, the EEC registration becomes a regular edge-1 interaction which can be done by EEC as any other EDGE-1 service (e.g. EEC subscribing to EAS change service).

Like any other EDGE service provided by EES, the service is there and procedure is specified, it is up to the consumer to invoke the EEC registration service or not. Consequently, the EEC context may include EEC registration context piece and/or other piece of information (e.g. EEC subscription for EAS change) due to interaction triggered by the EEC.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

#### 8.4.2.1	General

An EEC performs registration with an EES in order to provide information that can be used by the EES in Edge Computing services. The procedure also enables initialization, update and removal of the EEC context information at the EES. The context may be further used for edge-specific operations or processing, e.g. analytics.

* * * End of Changes * * * *