3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210807

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	Ericsson

Title:	Complete EAS discovery description

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution proposes to complete the EAS discovery description.

2. Reason for Change

The pre-condition should be clear that is is applicable for the S-EAS triggered case.

The current description doesn’t include the case when S-EES only executes step 1b.

In addition, the S-EES receives a list of T-EES from the ECS, therefore S-EES sends the EAS discovery request to each T-EES over EDGE-9.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

#### 8.8.3.2	Discover T-EAS

Figure 8.8.3.2-1 illustrates the procedure for fetching T-EAS information. This procedure may be utilized by a S-EAS, which undertakes the transfer of application context information to a T-EAS directly, or can be invoked by the S-EES itself on deciding to execute ACR.

Pre-conditions:

1.	Information related to the EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.2-1: Discover T-EAS

1.	The S-EAS sends the EAS discovery request to the S-EES or the S-EES decides to execute the ACR. The EAS discovery request from the S-EAS includes the requestor identifier [EASID] along with the security credentials and includes EAS discovery filter matching its EAS profile. If target DNAI is available at the S-EAS via User Plane Path change event, the S-EAS provides the S-EES with the target DNAI.

NOTE:	The trigger condition to invoke the Discover T-EAS API is up to application service logic, which is out of scope of this specification.

2.	If the request is received from the S-EAS, the S-EES checks whether the requesting EAS is authorized to perform the discovery operation. If the S-EES decided to execute the ACR or when the requesting EAS is authorized, the S-EES checks if there exists a T-EAS information (registered or cached) that can satisfy the requesting EAS information, additional query filters and the Expected service KPIs and the Minimum required service KPIs if received from the EEC during the EAS discovery or from the S-EAS in step 1. If the S-EES finds the T-EAS(s) in the cached or registered information, the flow either continues with step 5 for the S-EAS triggered discovery or stops for the S-EES decided ACR execution, else the S-EES retrieves the T-EES address from the ECS as specified in clause 8.8.3.3 and continues with step 3.

3.	The S-EES invokes the EAS discovery request on the T-EES retrieved from the ECS. The S-EES discovery request includes the requestor identifier [EESID] along with the security credentials and includes EAS discovery filter. In the EAS discovery filter, the S-EES may include the Expected service KPIs and the Minimum required service KPIs if received from the EEC during the EAS discovery or from the S-EAS in step 1.

Upon receiving the request, the T-EES may trigger the EAS management system to instantiate the T-EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.11.

4.	The T-EES discovers the T-EAS(s) and responds with the discovered T-EAS information to the S-EES. To filter T-EAS(s), the T-EES utilizes the discovery filters (e.g. Expected service KPIs and the Minimum required service KPIs). The S-EES may cache the T-EAS information.

5.	If the request was received from the S-EAS, the S-EES responds to the S-EAS with the T-EAS Information.

* * * End of Changes * * * *