3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210809

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	Ericsson

Title:	Monitored EES in ECS

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution proposes provisioning of the EES to be monitored by the ECS.

2. Reason for Change

The EDN service should be the EDN service area in cl.7.3.3.5.

cl.8.3.1 mentions the ECS triggered service provisioning as follows: ECS receives the EDN/DNAI change notification of the UE from 5GC when the ECS subscribes to the user plane path management events as specified in clause 8.9.2.

It only specified the condition to trigger such service provisioning based on DNAI change which is associated with user plane path change, the corresponding monitored EES in the service provisioning subscription is still missing.

It is proposed to add EES endpoint in the service provisioning subscription request; otherwise the ECS doesn’t know which specific traffic related to EEC-EES communication it should monitor.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

##### 8.3.3.3.4	Service provisioning subscription request

Table 8.3.3.3.4-1 describes the information elements for service provisioning subscription request from the EEC to the ECS.

Table 8.3.3.3.4-1: Service provisioning subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the EEC should be sent to. |
| AC profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| monitored EES information | O | List of EES information to be monitored, it may be present when the UE Identifier is provided. |
| > EES Endpoint | M | Endpoint information of the EES that EEC use to communicate with. The ECS uses this IE and UE Identifier to utilize the user plane path management capability of the 3GPP core network as specified in clause 8.9.2, and further notifies the EEC with service provisioning update. |

* * * Next Change * * * *

##### 8.3.3.3.7	Service provisioning subscription update request

Table 8.3.3.3.7-1 describes the information elements for service provisioning subscription update request from the EEC to the ECS.

Table 8.3.3.3.7-1: Service provisioning subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Connectivity information | O | Details about the connectivity of the UE with the PLMN, such as type of the 3GPP network. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| AC profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| monitored EES information | O | List of EES information to be monitored, it may be present when the UE Identifier is provided. |
| > EES Endpoint | M | Endpoint information of the EES that EEC use to communicate with. The ECS uses this IE and UE Identifier to utilize the user plane path management capability of the 3GPP core network as specified in clause 8.9.2, and further notifies the EEC with service provisioning update. |

* * * End of Change * * * *